VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

Velsanet Hardware Design Series

Consolidated Volume: WP#29 – WP#34

Network Equipment · Matrix RTL / Physical Design · CPO Optical Package

This volume reproduces the six network equipment design documents WP#29 through WP#34 in full and unmodified, in sequence. Per the Velsanet addition-only revision principle, no content of any constituent document has been altered, abridged, or re-ordered. Each document retains its own title block, classification, revision log, and open items register, and remains individually authoritative.

Republic of Korea  |  Velsanet Architecture Group

Contents

WP#29  Network Equipment System Architecture Design (SAD)

WP#30  Network Equipment RTL & Physical Design

WP#31  Schematic & BOM

WP#32  PCB Layout Design

WP#33  Integration Test Plan

WP#34  CPO Package & Optical Fusion Specification

VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

WP#29 — Network Equipment System Architecture Design (SAD)

O8 Connectivity & Placement Spec · Multi-Layer Multi-Plane Matrix Applied to Node Equipment

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Public — Open Specification | v1.10 │ 2026 | O8/D12/I20 node block architecture, geographic placement, Core Stack negotiation protocol, O8 operational architecture (Pass/self-healing/Master-Standby), NPU-Matrix integration and physical-constraint feedback loop, hierarchical core scaling (O8/D12/I20 face/layer/bundle differentiation) | Absorbs WP#25 R1-4/R1-5; Velsanet_Pending_Updates_Log Table 1; Compendium §8; WP#20 §5.3/§6.2; WP#24 §8.2/§8.3; R5 slot-level operational spec per 창호; R6 NPU-Matrix execution model per 창호; R7 document-sequence scope clarification per 창호; R8 hierarchical core scaling per 창호 — supersedes R5-era hardware-uniformity principle |

Republic of Korea | Velsanet Architecture Group

Governance note

This specification is published in full under the Velsanet open-specification policy. Certification against the node block architecture and negotiation protocol in this document — not confidentiality — is the mechanism by which conformance and interoperability are controlled.

# Revision Log — R2 vs v1.0 (retained)

| # | Change | Rationale |
| --- | --- | --- |
| R2-1 (superseded by R5-4) | Fixed O8 MOCT ribbon physical structure: 12 cores/layer × 4 layers = 48 cores per bundle; 4 bundles = 192 cores total input. Reorganized as 8 physical connector slots × 24 cores/slot for the front interface panel. | Per 창호 (original). SUPERSEDED — see R5-4: slot figure corrected to 192 cores/slot, total 1,536 cores. |
| R2-2 | Fixed O8 chassis height at approximately 2U given the 8-slot layout. | Per 창호 — resolves OI-29-07 for O8; D12/I20 chassis sizing remains open. |
| R2-3 | Added Section 10.6, Rack Mounting Position: rack-rail mounting point sits further back than a standard-depth unit for front-facing-connector clearance. | Per 창호 — physical constraint; exact offset not yet fixed (OI-29-09). |
| R3-1 | Added Section 4.1, building O8 sizing bottom-up from PAI/EAI per-instance equipment configurations (WP#26-28) instead of top-down from H6-level figures. Flagged EAI's 48-core Dual E2E total as numerically coincidental with, but distinct from, WP#23's H6 mesh-link 48 and the averaging-rule 120/144. | Per 창호's redirection during that session. Reframes OI-29-10 as a two-part question. |
| R4-1 | Superseded Pending Updates Log §1.4's Rx/Tx face-separation framing. New Section 6.1 splits the Connectivity Plane's 24 cores by function: Face 1 (12) = hypercube peer connectivity; Face 2 (12) = PAI/EAI request cooperation. Each core is a single bidirectional path. | Per 창호: verified "1 core = 1 E2E (bidirectional)" as the dominant Velsanet pattern; Face1/Face2 maps onto WP#20 §6's two node-intelligence functions. |

Table R-1. Revision Log — R2/R3/R4 (unchanged from v1.3)

# Revision Log — R5 (this revision)

| # | Change | Rationale |
| --- | --- | --- |
| R5-1 | Added Sections 4.3–4.5: Hierarchical Schlegel Diagram Projection (O8→D12→I20) and National Network AI (Velsa) / Global Network State Awareness, as part of the static block architecture. | Per 창호. Topology-monitoring and orchestration structure belongs alongside Section 4's block architecture; content also referenced from §10.7.6/10.7.7 rather than duplicated. |
| R5-2 | Added Section 10.7, O8 Node Operational Architecture: Pass function (O/E-EO active forwarding), external-loopback self-healing, slot-level allocation, dynamic Mode A/Mode B slot reallocation based on PAI/EAI presence, hierarchical restriction of PAI/EAI attachment to O8 only, applicability to D12/I20, and Master-Standby active failover (10.7.10). | Per 창호. Defines the node's runtime/operational behavior, which Sections 3–8 (logical/electrical architecture) did not cover. |
| R5-3 | Corrected §10.7.3 Slot 1 allocation from an initial three-way split (12 hypercube + 12 external-loopback-diagnostic + 168 external traffic) to a two-way split: 12 cores (hypercube/NPU) + 180 cores (direct-connect Pass path to Slot 2). | Per 창호's clarification this session: "external loopback" is not a separate diagnostic channel but the direct physical connection between Slot 1 (hypercube-facing) and Slot 2 (PAI/EAI-facing) — the same two-way pattern already used for Slots 2 and 5. |
| R5-4 | Superseded R2-1's core-count figures. O8's front-panel structure is now 8 slots × 192 cores/slot = 1,536 cores total (not 8 × 24 = 192 total as previously fixed). Table 6-1 and Table 10-1 updated accordingly. | Per 창호's explicit confirmation this session, based on the slot-level breakdown in §10.7.3. NOTE — this creates a numeric inconsistency with WP#20, WP#24, WP#25 §B.13.1, and the Velsanet Architecture Compendium, which all state "O8 = 192 optical cores." Per 창호's explicit instruction, this revision applies to WP#29 only; the cross-document inconsistency is not resolved here but is logged as a pending cross-document patch item (see note at end of Section 11). |
| R5-5 | Table 6-1's "Instance Plane — 168 cores" row is retired (marked historical/superseded), not remapped onto the new 8-slot structure. | Per 창호's confirmation: the Connectivity Plane / Instance Plane two-way split is fully superseded by §10.7.3's eight-slot structure; there is no forward mapping for the old 168-core figure. |
| R5-6 | Added Open Items OI-29-11 through OI-29-18 (Section 11): O/E-EO conversion characterization, internal direct-connect diagnostic cycle, Mode B timeout/transition latency, and Master-Standby failover parameters (neighbor-selection rule, trigger threshold, reversibility). | New gaps surfaced by §10.7's operational-layer detail, following the same practice of registering genuine gaps rather than inventing numbers. |
| R5-7 | §10.8 (proposed as a separate Mode A/B detection-detail section) was not added separately; its content is folded into §10.7.4.3, which already defines the same transition/detection logic. | Per Claude's recommendation, accepted by 창호, to avoid a duplicate section. |
| R5-8 | Document version updated to v1.4 (R5). | Reflects the scope of this revision. |

Table R-2. Revision Log — R5

# Revision Log — R6 (this revision)

| # | Change | Rationale |
| --- | --- | --- |
| R6-1 | Added Section 6.2, Connectivity Plane — NPU-Executed Task Model: clarifies that the Connectivity Plane (Slot 1/2 control cores) is a request/notification channel triggering deterministic NPU execution, not a statistical or predictive control plane. | Per 창호. Prevents the NPU-Matrix integration work (R6-2) from being read as introducing learned/predicted switching decisions, which would conflict with the A=f(T) principle that Velsanet's structural guarantees are physical, not statistical. |
| R6-2 | Added Section 10.7.11, NPU–Matrix Integration Architecture: defines the Node Intelligence NPU (§10.7.3, Slot 1) as a dedicated execution engine distinct from the Cognition Cube's interchangeable GPU/NPU/TPU cubification substrate (WP#24 Table 9-2, out of standardization scope). Two execution paths: (a) executing accepted PAI/EAI cooperation requests as Matrix configuration commands; (b) executing hypercube failover/switchover on Matrix-reported link loss (§10.7.2, §10.7.10). | Per 창호's clarification this session: the NPU 'receives AI cooperation requests and notifications, then performs the network-configuration work' and 'handles hypercube switchover on inter-equipment link loss' — an execution role, not a learning/prediction role. Confirmed as a separate, fixed hardware NPU, not the Cognition Cube's swappable substrate. |
| R6-3 | Added Section 10.7.12, Physical-Constraint Feedback Loop: states the loop by which Matrix-reported physical metrics (link status/LOS/BER; latency where instrumented) trigger NPU execution of pre-defined actions (alternate-path activation, Master-Standby handover, negotiated configuration) — no step is learned or statistically predicted. | Per 창호, in response to Claude's request for the feedback-loop structure; explicitly scoped to keep every step traceable to an already-defined mechanism (§7, §10.7.2, §10.7.10) rather than introducing new decision logic. |
| R6-4 | Added Open Items OI-29-20 (NPU–Matrix command interface not specified) and OI-29-21 (no node-level temperature sensor/feedback interface defined anywhere in the series — distinct from OI-29-06's thermal budget/capacity gap). | New gaps surfaced by the NPU-Matrix execution model. |
| R6-5 | Document version updated to v1.5 (R6). | Reflects the scope of this revision. |

Table R-3. Revision Log — R6

# Revision Log — R7 (this revision)

| # | Change | Rationale |
| --- | --- | --- |
| R7-1 | Updated Table 2-1 and Table 12-1: WP#30's title and scope expanded from "Network Equipment RTL (Switching/Routing)" to "Network Equipment RTL & Physical Design (Switching/Routing)", explicitly adding Multi-Layer Multi-Plane Matrix physical implementation — semiconductor circuit design, FPGA→ASIC process migration, die-level layout — as in-scope for WP#30. | Per 창호. Consolidates the Matrix's physical-implementation work (previously undifferentiated within the WP#30 proposal) into an explicit scope statement, following the FPGA→ASIC migration pattern already used in WP#26. |
| R7-2 | Explicitly distinguished WP#30's die-level (semiconductor) layout from WP#32's board-level (PCB) layout in both tables, to prevent the shared term "layout" from being read as scope overlap. | Per Claude's note, confirmed by 창호: WP#30 = chip/die-level physical design; WP#32 = PCB-level placement/routing. Different engineering domains, same word. |
| R7-3 | WP#29, WP#31, WP#32, WP#33 scope statements unchanged — confirmed by 창호 as "변경 없음" (no change), WP#29 remaining focused on system-level specification only. | Per 창호's explicit confirmation. |
| R7-4 | Document version updated to v1.6 (R7). | Reflects the scope of this revision. |

Table R-4. Revision Log — R7

# Revision Log — R8 (this revision)

| # | Change | Rationale |
| --- | --- | --- |
| R8-1 | SUPERSEDES §10.7.8's "hardware uniformity" principle. D12 and I20 no longer share O8's identical 8-slot/192-core-per-slot chassis. Each tier now has its own face (slot) count and its own per-face core structure: O8 = 8 faces × (12 cores × 4 layers × 4 bundles = 192 cores/face) = 1,536 total; D12 = 12 faces × (12 × 8 × 8 = 768 cores/face) = 9,216 total; I20 = 20 faces × (12 × 16 × 16 = 3,072 cores/face) = 61,440 total. | Per 창호's explicit confirmation this session. Face counts (8/12/20) follow the Platonic solid face-count of each node's namesake (octahedron/dodecahedron/icosahedron), consistent with WP#20's polyhedral naming; layer and bundle counts each double per tier (4→8→16), consistent with the ×4-per-tier pattern 창호 described (2× layers × 2× bundles = 4× cores/face per tier). |
| R8-2 | Added Section 4.6, Hierarchical Core Scaling: T4 → H6 → O8 → D12 → I20, stating the confirmed O8/D12/I20 figures and the face/layer/bundle scaling rule. | Per 창호. T4/H6 figures are explicitly NOT fixed by this revision (OI-29-22) — only O8/D12/I20 were confirmed this session. |
| R8-3 | Updated Table 10-1 (§10.4): D12/I20 chassis are no longer "reference basis only, height not yet fixed" — concrete face/core figures are now given, though chassis height itself remains open (OI-29-07 revised scope, see Section 11). | Reflects R8-1. |
| R8-4 | Added Open Items OI-29-22 (T4/H6 core-scaling figures not confirmed — pattern may or may not extend downward) and OI-29-23 (§10.7.3's slot-internal allocation — 12 control + 180 direct-connect — not yet scaled for D12/I20's larger per-face core counts; explicitly deferred to a future session per 창호). | New gaps surfaced by R8-1/R8-2; explicitly not resolved in this revision. |
| R8-5 | Document version updated to v1.7 (R8). | Reflects the scope of this revision. |

Table R-5. Revision Log — R8

# Revision Log — R9 (this revision)

| # | Change | Rationale |
| --- | --- | --- |
| R9-1 | Added Section 7.0a and Table 7-1a: three new Core Stack negotiation message types — Release (PAI/EAI → O8, intentional disconnection), Release_Ack (O8 → PAI/EAI, confirms release), and Force_Release (O8 → PAI/EAI, node-initiated forced disconnection). Table 7-1's message-types row updated to reference them. | Per 창호. Surfaced while drafting WP#30 §5's wire-level encoding of WP#29's four original message types — an explicit-release message was missing at the architecture level, not just the encoding level, so it is fixed here first rather than added silently in WP#30. |
| R9-2 | Documented Force_Release's trigger discipline: it is issued by the NPU's rule-based execution of the §10.7.2 self-healing sequence, not a discretionary judgment call. | Confirms consistency with §6.2's principle that the NPU executes pre-defined deterministic responses rather than deciding outcomes itself. |
| R9-3 | Document version updated to v1.8. | Reflects the scope of this revision. |

Table R-6. Revision Log — R9

# Revision Log — R10 (this revision)

| # | Change | Rationale |
| --- | --- | --- |
| R10-1 | Added a Mode B requirement for Master/Standby roles at §10.7.10: a node serving as Master or Standby (any tier) must be in Mode B (no PAI/EAI attached), not Mode A. A Mode A node remains valid for ordinary same-tier connectivity but is disqualified from the Master/Standby role specifically. | Per 창호: a Mode A node's face capacity is shared with AI processing, leaving insufficient capacity to reliably fulfill the upward hypercube relay and failover-coordination role. Reuses the existing Mode A/B mechanism (§10.7.4) rather than introducing a new node-hardware category, per 창호's confirmed direction. |
| R10-2 | Added Open Item OI-29-24: the mechanism for checking Mode A/B status before Master/Standby selection (static deployment-time constraint vs. dynamic check at failover time) is not specified. | New gap surfaced by R10-1. |
| R10-3 | Document version updated to v1.9. | Reflects the scope of this revision. |

Table R-7. Revision Log — R10

| # | Change | Rationale |
| --- | --- | --- |
| R11-1 | Terminology correction: “connector”/“Connector Type” wording in §10.4 Table 10-1, §10.5 Table 10-2 header, and OI-29-09 replaced with “ribbon slot”/“Interface Type”/“MOCT ribbon” — MOCT is factory-fused with no field connectors (WP#03 §4.1–4.2, WP#24 §1.4), so labeling the fixed ribbon slots as “connectors” contradicted that invariant. No physical or numeric spec changed. | Per 창호 review — terminology audit. |
| R11-2 | Document version updated to v1.10. | Reflects the scope of this revision. |

Table R-8. Revision Log — R11

# 1. Purpose and Scope

WP#29 is the System Architecture Design (SAD) document for Velsanet network equipment at the O8/D12/I20 node level. It fulfills two deferrals recorded in WP#25's own revision log — R1-4 ("Geographic/topological placement of O8/D12/I20... will be addressed in a separate forthcoming document, not in WP#25") and R1-5 ("Core Stack Activation Condition is determined by negotiation... The negotiation protocol itself... [deferred]") — and absorbs the corresponding backlog already confirmed in Velsanet_Pending_Updates_Log §1 (Table 1).

This document also applies the Multi-Layer Multi-Plane Matrix principle — defined at the physical/photonic layer in the Velsanet Architecture Compendium §8 and fixed in patent claims 3–4 of the prior-art search report (1020220070628) — to the O8/D12/I20 node equipment layer specifically.

Scope includes: the Multi-Layer Multi-Plane Matrix principle restated for node equipment (Section 3); O8/D12/I20 block architecture including hierarchical topology projection and national orchestration (Section 4); density-driven geographic placement (Section 5); O8 core allocation (Section 6); the Core Stack negotiation protocol (Section 7); the node equipment interface to the Cognition Cube documents (Section 8); node physical architecture (Section 10); and, new in R5, O8 node operational architecture — Pass function, self-healing, dynamic mode allocation, and Master-Standby failover (Section 10.7).

Out of scope: the Cognition Cube RTL/register-level design itself (WP#24–26, unchanged here); node-to-node switching/routing RTL (proposed WP#30); schematic, BOM, PCB layout, and integration test planning (proposed WP#31–33).

# 2. Positioning in the Document Sequence

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 | SEU1/SEU2 Synchronization Specification | SEU1/SEU2 record format, PTP profiles, completeness algorithms | Complete |
| WP #26 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, FPGA→ASIC migration | Complete — 12 open items |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete |
| WP #29 | Network Equipment SAD (this document) | O8/D12/I20 block architecture, placement, Core Stack negotiation, O8 operational architecture, NPU-Matrix execution model | Complete — v1.10 |
| WP #30 (proposed) | Network Equipment RTL & Physical Design (Switching/Routing) | Node-to-node switching/routing logic (RTL) + Matrix physical implementation: semiconductor circuit design, FPGA→ASIC process migration, die-level layout — distinct from WP#32's PCB-level layout | Proposed — next |
| WP #31 (proposed) | Schematic & BOM | 800G transceiver, FPGA/ASIC, power circuit connections | Proposed |
| WP #32 (proposed) | PCB Layout Design | 800G PAM4 board-level placement/routing, thermal design | Proposed |
| WP #33 (proposed) | Integration Test Plan | Node-to-node connectivity, fault recovery, performance testing | Proposed |

Table 2-1. Document Sequence — Updated at WP#29 v1.10

# 3. Multi-Layer Multi-Plane Matrix — Principle Recap

The Velsanet Architecture Compendium §8 ("Layered Matrix–Transceiver Physical Structure") defines the Multi-Layer Multi-Plane Matrix as the structure formed jointly by the MOCT transceiver, the polymer-form optical core layer, and the matrix itself — coupled through the Multi-Layer Ribbon Fiber Coupling System (MLRFCS).

The same concept is fixed in patent claim language (prior-art search report 1020220070628): claim 3 defines the matrix as the mechanism by which a hardware connection request is routed through multiple channels to a transmission port; claim 4 extends this to multi-directional transmission toward neighboring channel exchangers, including channel-number translation.

| Layer | What the Matrix Does at That Layer |
| --- | --- |
| Photonic (Compendium §8) | Physical channel/core coupling between MOCT transceiver and ribbon fiber assembly — deterministic, passive, no round-trip |
| Equipment/node (this document) | Channel switching and multi-directional transmission between O8/D12/I20 nodes — same claim-3/4 mechanism at the node-to-node level |
| Intra-node (WP#26) | SEU1/SEU2 RTL engines — internal to a single Dedicated Equipment instance, not the inter-node matrix |

Table 3-1. Multi-Layer Multi-Plane Matrix — Scope by Layer

This document is specifically about the middle row: how the matrix principle governs O8/D12/I20 node equipment.

# 4. O8/D12/I20 Node Block Architecture

Node roles and Q-layer assignments are fixed at WP#20 §5.3 and restated unchanged here as the block-architecture baseline for this document.

| Node | Q Layer | AI Layer | Structural Role |
| --- | --- | --- | --- |
| T4 | None (Autonomous) | None | Personal / minimal access — rhizome origin, free connection, no positional constraint |
| H6 | None (Autonomous) | None | Physical / RAN mapping — anchors to physical infrastructure; core count fixed and bounded |
| O8 | Q6 or above | PAI | Access / City Layer — first intelligence tier; PAI natively embedded for city-scale coordination |
| D12 | Q4–Q6 | AAI | Regional Coordination — multi-domain agent AI, coordinates across multiple O8 clusters within a region |
| I20 | Q3–Q4 | AsAI | Global / Sovereign Judgment — highest intelligence tier, structural authority at national/continental scale |

Table 4-1. Node Intelligence Hierarchy (restated from WP#20 Table, §5.3)

O8's cumulative cubification compression, fixed at WP#24 §8.2–§8.3, is restated here as the traffic-volume baseline against which this document's core-allocation decisions (Section 6) are made.

| Layer | Input Channels | Output Channels (Cumulative Reduction) |
| --- | --- | --- |
| O8 (boundary) | 8 channels | 1 channel (1/8) |
| D12 (regional aggregation) | N × O8 count | 1 channel (1/(8×N)) |
| Core (central) | M × D12 count | 1 channel (1/(8×N×M)) |

Table 4-2. Hierarchical Cubification Compression (restated from WP#24 Table 8-3)

## 4.1 O8 Sizing, Built Bottom-Up from PAI and EAI Equipment Configuration

Rather than infer O8's internal structure top-down from H6-level figures (WP#23), this section builds it bottom-up from the two equipment configurations already fully specified in WP#26–28: what a single PAI instance requires, and what a single EAI instance requires.

Figure 4-1. Velsanet Equipment ↔ PAI Configuration

Figure 4-2. Velsanet Equipment ↔ EAI Configuration

A disambiguation worth stating explicitly: EAI's per-instance total is 48 cores (24+24, Dual E2E, WP#24 Table 5-2) — a different "48" from WP#23's H6 Face 3/4/6 mesh-link figure and from the (H6_H+O8_H)/2 averaging figure. All three are numerically 48 by coincidence, not the same quantity.

| Instance Type | Per-Instance Core Requirement |
| --- | --- |
| PAI | 1 of 384 O8 input lanes (shared collection structure) + 24 dedicated uplink cores to D12 (Figure 4-1) |
| EAI | 48 cores total (24+24, Dual E2E, physically separate per WP#24 Table 5-2), 1:1 deployment, attach level to O8 hierarchy unresolved (Figure 4-2) |

Table 4-3. Per-Instance Core Requirements — PAI vs EAI

Open — OI-29-10: H6↔O8 link core count is unresolved: WP#23 fixes it at 144 cores (12×12 stack); a (H6_H+O8_H)/2 averaging rule gives 120 cores. Separately, it is not yet fixed whether EAI instances attach directly to O8's lane structure or via H6 first — this must be resolved before the 144-vs-120 question can be framed correctly.

## 4.2 (Reserved)

Reserved for future EAI attach-topology resolution once OI-29-10 is closed. No content added in R5.

## 4.3 Hypercube Information Exchange via Schlegel Diagram Projection (O8 → D12)

Each O8 projects its local Q7 hypercube topology to its regional D12, using a Schlegel diagram projection — a standard technique for representing an n-dimensional polytope's structure as a lower-dimensional graph while preserving adjacency relationships. D12 aggregates the projections received from all O8 nodes in its region into a unified regional Schlegel diagram, giving the D12 tier a single structural view of every O8 under its coordination without requiring D12 to process raw per-instance PAI/EAI traffic (Section 10.7.5).

## 4.4 Hierarchical Schlegel Projection: O8 → D12 → I20

The Schlegel diagram projection defined in Section 4.3 is applied hierarchically across the Velsanet node layers. Each D12 projects its aggregated regional Schlegel diagram upward to the national I20; I20 aggregates projections from all D12 nodes under its authority to form a national/continental structural view of the entire hypercube network.

This hierarchical projection enables: scalable topology management, since each node manages only its own local scope and is not burdened with global state; efficient fault detection, since link failures propagate upward automatically as structural changes in the projected diagram rather than requiring separate fault-reporting messages; and coordinated self-healing, since I20 — holding the national view — can issue cross-regional rerouting commands that no single D12 or O8 could determine on its own.

The projection itself is one-way, flowing upward for topology monitoring only. Control and configuration commands flow downward through the same O8→D12→I20 hierarchy, in the reverse direction (Section 4.5).

## 4.5 National Network AI (Velsa) and Global Network State Awareness

At the I20 layer, the Velsanet Network AI (Velsa) operates as the national/continental orchestrator. Velsa receives the hierarchical Schlegel projections aggregated from all D12 nodes (Section 4.4), forming a unified real-time structural map of every O8, D12, and I20 node in its scope.

Based on this aggregated view, Velsa can: detect link failures and traffic bottlenecks across the entire network from a single vantage point; issue optimal path reconfiguration commands that span multiple regions, something no individual D12 has visibility to determine on its own; and coordinate self-healing responses and traffic load balancing at national scale, sequencing multiple regional recoveries rather than leaving them to resolve independently and possibly in conflict.

Reconfiguration decisions propagate downward through the hierarchy (I20 → D12 → O8) for execution at the node level. This top-down control path, combined with the bottom-up Schlegel projection of Section 4.4, is what allows the Velsanet network to operate as a single coherent cognitive entity rather than a collection of independently-acting nodes.

## 4.6 Hierarchical Core Scaling: O8 → D12 → I20 (New in R8; supersedes §10.7.8 hardware uniformity)

R8, per 창호. This section supersedes §10.7.8's earlier "hardware uniformity" principle. Each node tier has its own face (slot) count and its own per-face optical-core structure — they are not physically identical chassis at different logical roles, as R5–R7 assumed, but distinct physical designs following a consistent scaling rule.

Each face's core count is built from the same three-factor structure already fixed for O8 at R5-4/§10.7.3 — 12 cores × layer count × bundle count — but layer count and bundle count each double at every tier moving up the hierarchy, while the face count itself follows the Platonic-solid face count of each node's namesake (octahedron = 8, dodecahedron = 12, icosahedron = 20, consistent with WP#20's polyhedral node naming).

| Node | Faces (= slots) | Layers | Bundles | Cores/Face (12×layers×bundles) | Total Cores (faces × cores/face) |
| --- | --- | --- | --- | --- | --- |
| O8 | 8 | 4 | 4 | 192 | 1,536 |
| D12 | 12 | 8 | 8 | 768 | 9,216 |
| I20 | 20 | 16 | 16 | 3,072 | 61,440 |

Table 4-4. Hierarchical Core Scaling — O8/D12/I20 (confirmed per 창호, this session)

This scaling is structural, not arbitrary: each tier preserves the same 12-core building block and the same Multi-Layer Multi-Plane Matrix principle (Section 3), while layer and bundle counts double per tier (4→8→16) to accommodate the increased connectivity and processing demands of higher-tier nodes, and face count follows each node's polyhedral identity.

Open — OI-29-22: T4 and H6 core-scaling figures are not confirmed by this revision. Whether the same face/layer/bundle pattern extends downward to T4 and H6 (which are Autonomous nodes with no AI layer, per Table 4-1) — and if so, what their figures are — is undecided and is not to be inferred from the O8/D12/I20 pattern without explicit confirmation.

Open — OI-29-23: §10.7.3's O8 slot-internal allocation (12 control cores + 180 direct-connect Pass-path cores, out of 192) is not yet scaled for D12's 768-cores/face or I20's 3,072-cores/face. Whether the control-core count stays fixed at 12 (with all additional capacity going to the Pass path) or scales proportionally is explicitly deferred to a future session, per 창호.

# 5. Geographic/Topological Placement & Density-Driven D12 Sizing

This section resolves WP#25 R1-4 using the concepts already confirmed in Pending Updates Log §1.1–§1.2.

| Rule | Definition |
| --- | --- |
| D12 = f(density) | D12 boundary is a function of O8-cluster density, not administrative district or compass quadrant. High-density areas let one district stand alone as one D12; low-density areas merge multiple districts into one D12. Consistent with WP#24 §8.3's "D12 = N × O8 count" leaving N unspecified, and with A=f(T) generally. |
| I20 remains national/continental only | I20 must remain the national/continental sovereign tier (WP#20 §5.3 Q3–Q4). A city, however large, is an aggregation of multiple D12s reporting to one national I20 — not an I20 itself. |

Table 5-1. Density-Driven Placement Rules (Pending Updates Log §1.1–§1.2)

Open — OI-29-04: Numeric density threshold for "high-density" vs "low-density" D12 aggregation is not fixed — rule is qualitative (D12 = f(density)) without a specific density function.

# 6. O8 Core Allocation — Connectivity Plane vs. Instance Plane (Historical, Superseded in R5)

SUPERSEDED IN R5 — see Section 10.7.3 for the current 8-slot / 1,536-core allocation. The table below is retained for historical reference and to preserve the derivation of Face 1 / Face 2, which carries forward unchanged into the new slot structure (Slot 1 = Face 1, Slot 2 = Face 2, Section 10.7.3).

This section originally resolved the reconciliation flagged in Pending Updates Log §1.3–§1.5: how O8's 192-core allocation (Table 4-2 baseline, as understood prior to R5) divided between equipment-level negotiation traffic and real-time SEU traffic.

| Plane | Core Allocation (pre-R5) | Traffic Type | R5 Status |
| --- | --- | --- | --- |
| Connectivity Plane | 24 cores — see Table 6-2 | Non-real-time — attach/detach, capability/health handshake, provisioning | Retained conceptually — now realized as Slot 1 (12) + Slot 2 (12), §10.7.3 |
| Instance Plane | 168 cores, allocated per Core Stack (Section 7) | Real-time — SEU1/SEU2 traffic per WP#25 B.13 | RETIRED — not remapped. Superseded in full by the 8-slot structure; no forward-mapped quantity (R5-5). |

Table 6-1. O8 192-Core Split (Pending Updates Log §1.3) — historical, superseded by §10.7.3 in R5

Splitting out a non-real-time Connectivity Plane did not violate the Interface Tax / Deterministic Parallelism guarantee that real-time SEU traffic requires (WP#20 §6, non-collision constraint), because Connectivity Plane traffic was explicitly non-real-time and did not share cores with instance-plane traffic. This non-collision principle is preserved in the R5 slot structure: Slot 1/2's control cores remain physically separate from the Pass-path cores.

## 6.1 Connectivity Plane — Functional Split (Resolved, R4; Face Mapping Preserved in R5)

Superseding Pending Updates Log §1.4's original Rx/Tx face-separation framing: the Connectivity Plane's cores split by function, not by transmit direction, mapping directly onto the two node-intelligence functions already defined at WP#20 §6.1–§6.3.

| Face | Cores | Function | R5 Slot Mapping |
| --- | --- | --- | --- |
| Face 1 | 12 | Hypercube-conformant peer connectivity (WP#20 §6.2) — connections to external, same-tier nodes, prioritized to preserve Q7 hypercube structural properties | Slot 1 control cores (§10.7.3) |
| Face 2 | 12 | PAI/EAI request cooperation (WP#20 §6.3) — parallel core support issued in response to PAI/AAI/AsAI requests for higher bandwidth, redundancy, or multi-path transmission | Slot 2 control cores (§10.7.3) |

Table 6-2. Connectivity Plane — Functional Split by WP#20 §6 Node-Intelligence Function (Face definitions carried forward into R5 slot structure)

Within either face, each core is a single bidirectional path, consistent with the dominant Velsanet pattern (WP#04 §8.1; WP#25 R1-3) — not a fixed receive-only/transmit-only assignment. Pending Updates Log §1.5's face-separation principle (SEU1/SEU2-A/SEU2-B each on their own physical face within a Core Stack) is unaffected by this revision or by R5.

## 6.2 Connectivity Plane — NPU-Executed Task Model (New in R6)

The Connectivity Plane (Slot 1/2 control cores, §10.7.3) is not a statistical or predictive control plane. It is the request/notification channel through which the Node Intelligence NPU (§10.7.11) receives two categories of trigger: accepted AI cooperation requests (from Section 7's negotiation protocol) and Matrix-reported link-status events (§10.7.2).

In both cases, the NPU executes a pre-defined, deterministic response — configuration per the negotiated request, or failover per §10.7.2/§10.7.10 — rather than learning or predicting network behavior. This preserves the deterministic-latency principle stated for the Pass function (§10.7.9) and is consistent with the A=f(T) principle that Velsanet's structural guarantees are physical, not statistical: the NPU executes outcomes that Section 7's negotiation protocol and the hypercube topology already determine; it does not itself decide them.

# 7. Core Stack Activation & Negotiation Protocol

This section resolves WP#25 R1-5, using Pending Updates Log §1.6.

| Protocol Element | Definition |
| --- | --- |
| Parties | AI system (PAI/EAI) and Velsanet equipment (Velsa/Network AI), negotiating over the Connectivity Plane (Section 6 / Slot 1–2, §10.7.3) |
| Message types | Attach; capability/health handshake; layer activation/deactivation request; equipment-initiated renegotiation (e.g. on power shortage); Release; Release_Ack; Force_Release (R9 — see Table 7-1a) |
| Scope fixed by WP#25 | Only the existence and timing of negotiation was fixed in WP#25; the protocol format itself was deferred to this document |

Table 7-1. Core Stack Negotiation Protocol — Confirmed Elements (Pending Updates Log §1.6)

## 7.0a Release, Release_Ack, and Force_Release (New in R9)

R9, per 창호. Surfaced while drafting WP#30 §5's wire-level encoding: the original four message types (Table 7-1) had no explicit connection-release message — an architecture-level gap, not just an encoding detail, so it is fixed here rather than added silently in WP#30.

| Message | Direction | Purpose |
| --- | --- | --- |
| Release | PAI/EAI → O8 | Explicit, intentional disconnection request (normal termination) |
| Release_Ack | O8 → PAI/EAI | Confirms the release has completed |
| Force_Release | O8 → PAI/EAI | Node-initiated forced disconnection notice |

Table 7-1a. New Message Types (R9)

Force_Release's trigger discipline: per 창호's confirmation, Force_Release is issued by the NPU's rule-based execution of the self-healing sequence already defined at §10.7.2 (LOS/BER detection → alternate path discovery → Pass activation) — it is not a discretionary NPU judgment call ("this looks like it might be failing"). This keeps Force_Release consistent with §6.2's principle that the NPU executes pre-defined deterministic responses rather than deciding outcomes itself.

## 7.1 Degradation-Priority Scenario — Proposed, Not Yet Confirmed

Unconfirmed — PU-1.7: Pending Updates Log §1.7 records a draft degradation-priority scenario for equipment-initiated renegotiation, explicitly marked as a recommendation, not yet confirmed by 창호. Layer 0 (SEU1) and Layer 1 (SEU2-A) would never be reclaimed. If Layer 2 (SEU1 redundancy) and Layer 3 (SEU2-B, cognition reporting) cannot both be sustained, Layer 2 would be released before Layer 3, consistent with WP#25 B.13.2's "SEU2-A failure must not block SEU2-B" invariant. This remains open (OI-29-02) until 창호 confirms or revises it.

# 8. Interface Definition — Node-to-Node and Node-to-Cognition-Cube

O8/D12/I20 node equipment is the deployment context in which WP#24–26's Cognition Cube hardware operates: the SEU1/SEU2 RTL engines (WP#26) sit inside the Dedicated Equipment attached to a node's instance-facing slots, while the node itself handles Connectivity Plane negotiation (Section 7) and inter-node channel switching (Section 3) that gets traffic to and from that equipment.

Open — OI-29-01: The exact register/signal-level interface between the node equipment defined in this document and the Dedicated Equipment RTL defined in WP#26 is not specified. Candidate for WP#30.

# 10. Node Equipment Physical Architecture

Sections 3–8 fixed the logical/electrical architecture of node equipment. This section adds the physical-implementation elements a System Architecture Design document requires.

## 10.1 Block Diagram

Figure 10-1. O8 Node Equipment — Internal Block Diagram

## 10.2 Power & Clock Distribution

A node chassis draws from facility power into a node-level PSU, which distributes to: (a) the always-on control/negotiation blocks (Slot 1/2 control cores), and (b) per-instance rails feeding each Dedicated Equipment instance's own SEU1/SEU2 power domains. Clock distribution follows the same split: the node forwards a PTP master reference to instance-facing traffic, while control/negotiation traffic is non-real-time and does not require PTP discipline.

Open — OI-29-05: The exact node-level PSU topology (single supply with N-way distribution vs. redundant dual-feed, specific voltage rails) is not specified.

## 10.3 Thermal / Cooling Design

The Compendium §6.1 establishes passive/heat-pipe spreading for general heat, with liquid cooling reserved for high-performance optical core transceivers and processors. This document adopts the same split at node level.

Open — OI-29-06: No node-level thermal budget (watts, junction temperature, airflow CFM) is specified.

## 10.4 Physical Form Factor

| Parameter | Specification (R5) |
| --- | --- |
| MOCT ribbon structure (O8) | Superseded in R5 — see §10.7.3 for the current 8-slot/1,536-core breakdown |
| Front-panel slot layout (O8) | 8 fixed MOCT ribbon slots × 192 cores/slot = 1,536 cores total (R5-4; supersedes v1.3's 24 cores/slot, 192 total) |
| O8 chassis height | Approximately 2U, given the 8-slot front-panel layout (unchanged from v1.3) |
| Mounting standard | 19-inch rack, EIA-310-D (WP#27 §5.1) |
| D12/I20 chassis | SUPERSEDED IN R8 — no longer O8-uniform. D12 = 12 faces × 768 cores/face = 9,216 total; I20 = 20 faces × 3,072 cores/face = 61,440 total (Table 4-4, §4.6). Chassis height for both remains open (OI-29-07). |

Table 10-1. Physical Form Factor — Updated for R5 1,536-core structure

Open — OI-29-07: D12/I20 chassis height is not yet fixed — depends on how many O8 uplinks each aggregates, which depends on the density rule in Section 5 (OI-29-04).

## 10.5 Interface Panel Configuration

| Panel Group | Interface Type | Maps To |
| --- | --- | --- |
| Optical in/out (O8) | 8× MOCT fixed-ribbon slots, 192 cores/slot (R5), factory-fused, no field connectors | Slot 1–8 functions (§10.7.3); uplink Bundle to D12 |
| Power in | Facility PDU or PoE++/DC per deployment context | Node-level PSU (Section 10.2) |
| Management port (OOB) | Out-of-band, execution-layer/vendor choice — not yet specified | Node Intelligence block status/diagnostics — no register-level definition exists yet |

Table 10-2. Interface Panel Groups

Open — OI-29-08: The management port's physical connector type and protocol are not specified — no Velsanet document to date defines a node-level management/diagnostic interface at all.

## 10.6 Rack Mounting Position

Per 창호: because the 8 MOCT slots face forward on the front panel, the chassis's rack-rail mounting point must sit further back in the rack than a standard-depth 2U unit would otherwise require.

Figure 10-2. O8 Rack Mounting — Side View, Rail Set-Back vs. Standard Rear-Cabled Reference

Open — OI-29-09: The exact rail-position offset is not yet dimensioned — depends on the specific MOCT ribbon's physical length and minimum bend radius.

## 10.7 O8 Node Operational Architecture: Pass Function, Self-Healing, and Hierarchical Topology Management

This section defines the complete operational behavior of the O8 node: its role as an active optical exchange, its self-healing mechanisms via the Pass function, and its hierarchical integration with D12 and I20 nodes. New in R5.

### 10.7.1 Pass Function Overview

The O8 node performs optical-to-electrical-to-optical (O/E-EO) conversion on incoming optical signals. This conversion enables the node to inspect, switch, and redirect traffic without requiring packet-level processing, while maintaining deterministic latency and physical isolation.

The Pass function allows an O8 node to receive an optical signal from an external source (another O8, D12, or I20), convert it to the electrical domain for switching decisions, and retransmit it as an optical signal to the next node. This is not a passive optical bypass — it is an active, low-latency forwarding operation enabling traffic demultiplexing, path selection based on real-time link status, and fault circumvention around failed links within the hypercube topology.

The Pass function is implemented per core or per bundle, and is managed by the Node Intelligence block in coordination with the Connectivity Plane (Section 6).

### 10.7.2 External Loopback and Self-Healing

Clarification (per 창호, this session): the "external loopback" described in this section is realized physically as the direct-connect path between Slot 1 (hypercube/NPU-facing) and Slot 2 (PAI/EAI control-facing) within a single O8 node — see §10.7.3. It is not a separate diagnostic-only channel, and by itself it is not a signal exchanged with an adjacent physical node. Traffic entering the node from the hypercube side (Slot 1) and destined for a PAI/EAI instance (Slot 2), or vice versa, passes through this direct connection with O/E-EO conversion at each end; this is the physical realization of the Pass function (§10.7.1) within one node. Where self-healing additionally requires rerouting around a failed link to another O8/D12/I20 node, that inter-node rerouting is carried over the general hypercube connectivity of Slot 1 (Face 1, §6.1) and the intermediate nodes' own Slot 1↔Slot 2 direct-connect paths in sequence — it is not a separate loopback-specific inter-node channel.

The self-healing sequence is as follows: (1) Link failure detection — loss of signal (LOS) or bit-error-rate (BER) degradation is detected on a specific slot or core group; (2) Alternate path discovery — the Node Intelligence block consults the hypercube topology map and identifies available alternate routes; (3) Pass activation — traffic is passed through intermediate O8 nodes using O/E-EO conversion and forwarded to the alternate path; (4) E2E reconnection — the original E2E path is restored via the alternate route, with no disruption to the end devices (PAI/EAI).

This mechanism is intended to allow the Velsanet network to self-heal from link failures within microsecond-scale recovery time, without human intervention or control-plane round-trips.

### 10.7.3 Slot-Level Pass and Loopback Allocation

R5-3/R5-4 correction: Slot 1 is corrected from an originally-drafted three-way split to a two-way split, matching the Slot 2/Slot 5 pattern. Each of the 8 slots carries 192 cores (not 24 as in v1.3); total O8 core count is 1,536 (8 × 192), superseding R2-1 — see R5-4 and the cross-document note at the end of Section 11.

| Slot | Cores | Allocation |
| --- | --- | --- |
| Slot 1 | 192 | 12 cores: hypercube topology control (NPU-driven Q7 maintenance, = Face 1, §6.1) + 180 cores: direct-connect Pass path to Slot 2 (external loopback realization, §10.7.2) |
| Slot 2 | 192 | 12 cores: inbound control (PAI/EAI control and negotiation signaling, = Face 2, §6.1) + 180 cores: direct-connect Pass path to Slot 1 |
| Slot 3 | 192 | Inbound data reserve (secondary inbound path for PAI/EAI traffic) |
| Slot 4 | 192 | Inbound data reserve (tertiary inbound path for PAI/EAI traffic) |
| Slot 5 | 192 | 12 cores: outbound control (PAI/EAI control and negotiation signaling) + 180 cores: Pass path (O/E-EO conversion, forwarding to the external network) |
| Slot 6 | 192 | External expansion (D12, I20, H6, T4 connectivity) |
| Slot 7 | 192 | External expansion (D12, I20, H6, T4 connectivity) |
| Slot 8 | 192 | External expansion (D12, I20, H6, T4 connectivity) |

Table 10-3. O8 Slot-Level Core Allocation — 8 slots × 192 cores = 1,536 cores total (R5)

### 10.7.4 Dynamic Slot Allocation Based on PAI/EAI Presence

The O8 node operates in two distinct modes depending on whether PAI or EAI systems are attached, allowing the same hardware to serve as either a PAI/EAI-aware cognitive edge node or a pure hypercube network switch.

### 10.7.4.1 Mode A: PAI/EAI Attached (Full Cognitive Mode)

When PAI or EAI instances are present, Slot 2 and Slot 5 are used for inbound and outbound control and data traffic. Slot 3 and Slot 4 serve as inbound reserves. Slots 6–8 are used for external expansion.

### 10.7.4.2 Mode B: No PAI/EAI Attached (Pure Hypercube Mode)

When no PAI/EAI systems are connected, the O8 operates as a dedicated hypercube network switch. Slots 2–5 are repurposed for hypercube expansion and alternate path provisioning. Slot 1 remains dedicated to hypercube topology and external traffic; Slots 6–8 remain for external expansion. D12 and I20 nodes operate permanently in Mode B (§10.7.8), since PAI/EAI never attaches at those tiers (§10.7.5).

### 10.7.4.3 Mode Transition and Detection (incorporates former §10.8 content, R5-7)

The transition between Mode A and Mode B is determined by the Connectivity Plane during the initial attach handshake: if PAI or EAI instances are detected on Slot 2 or Slot 3, the O8 operates in Mode A; if no PAI/EAI attachment is detected within a configurable timeout, the O8 defaults to Mode B. Reconfiguration is dynamic — if a PAI/EAI instance attaches after the O8 has booted in Mode B, a renegotiation triggers the transition to Mode A, and Slots 2–5 are repartitioned accordingly. The configurable timeout and renegotiation trigger threshold are deployment-specific and shall be defined in the site-specific configuration profile.

Open — OI-29-13: The Mode B detection timeout value and renegotiation trigger threshold are not numerically specified — left as deployment-specific in §10.7.4.3, but no default or reference value exists anywhere in the document series.

Open — OI-29-14: Mode B → Mode A transition latency (time from PAI/EAI attach detection to completed Slot 2–5 repartition) is not characterized.

### 10.7.5 Hierarchical Restriction: PAI/EAI Attachment Only at O8

PAI and EAI systems shall attach only to O8 nodes within the Velsanet hierarchy. No direct PAI/EAI attachment is permitted at D12, I20, H6, or T4 nodes. This ensures raw multimodal data is processed and cubed at the first intelligence tier (O8); higher-tier nodes (D12, I20) operate exclusively on synthesized Cognition Cubes, not on raw PAI/EAI signals; and the governance/coordination functions of AAI and AsAI are decoupled from per-instance signal processing. Any deployment attempting to attach PAI or EAI directly to D12, I20, H6, or T4 is non-conformant and shall be rejected by the Connectivity Plane negotiation protocol (Section 7).

### 10.7.6 Hierarchical Schlegel Diagram Projection

See Section 4.4 for the full definition of hierarchical Schlegel diagram projection (O8 → D12 → I20). Relocated to Section 4 in R5 to keep the static block-architecture and topology-monitoring definitions together; this reference is retained here for readers navigating from the operational-architecture section.

### 10.7.7 National Network AI (Velsa) and Global Network State Awareness

See Section 4.5 for the full definition of Velsa's role as national/continental orchestrator. Relocated to Section 4 in R5; retained here as a cross-reference.

### 10.7.8 Applicability to D12 and I20 Nodes (SUPERSEDED IN R8 — see §4.6)

SUPERSEDED IN R8 — retained for historical reference only. This section originally stated that D12 and I20 share O8's identical 8-slot/192-core-per-slot chassis ("hardware uniformity"). Per 창호's explicit confirmation this session, that is no longer the architecture: D12 and I20 each have their own face count and per-face core structure — see Table 4-4 (§4.6) for the confirmed figures (D12 = 12 faces × 768 cores/face = 9,216 total; I20 = 20 faces × 3,072 cores/face = 61,440 total).

What is retained from this section's original intent: D12 and I20 still do not attach PAI or EAI systems, so their slot allocation still follows Mode B (§10.7.4.2) permanently. The Node Intelligence block at D12 still executes AAI governance functions; at I20 it still executes AsAI execution and sovereign judgment functions. What no longer holds is the claim that this is achieved on identical physical hardware — it is now achieved on tier-specific hardware running the same Mode B logic.

### 10.7.9 Design Principles Summary

The O8 operational architecture is governed by: active forwarding (Pass is an active O/E-EO operation, not a passive optical bypass); deterministic latency (switching and conversion latency is bounded and predictable); hardware uniformity (the same O8 chassis supports both Mode A and Mode B with identical physical slot/core configurations); software-defined allocation (slot roles are determined by the Node Intelligence block based on attachment state, not fixed hardware strapping); scalable healing (self-healing operates at the optical core level, independent of higher-layer protocols); hierarchical awareness (topology information flows upward via Schlegel projection, control flows downward); and resilience (the hypercube topology provides multiple alternate paths, ensuring no single point of failure).

### 10.7.10 Master-Slave Hierarchy with Active-Standby Failover

New in R5, per 창호. Master-Standby redundancy applies starting at the O8 tier and propagates upward through D12 and I20, following the hardware-uniform pattern established in §10.7.8.

Because D12 and I20 (and, per §10.7.5, H6 and T4) do not have PAI or EAI directly attached, standby designation at these tiers cannot rely on instance-level failover the way an O8's PAI/EAI-facing slots might. Instead, redundancy is achieved structurally, through the hypercube topology itself, rather than through a dedicated redundant physical unit co-located with the master.

For any given master node (O8, D12, or I20), an adjacent node within the hypercube (Q7) topology graph — reached via the same hypercube-conformant peer connectivity already defined for the Pass function (§10.7.1) and Connectivity Plane Face 1 (§6.1) — serves as that master's standby. This reuses existing hypercube adjacency rather than requiring additional dedicated standby hardware or a separate physical redundancy link.

Master failure is treated as a specific case of the link/node failure detection sequence already defined in §10.7.2 (LOS/BER-based detection → alternate path discovery via the hypercube topology map → Pass activation). Upon detecting a master's failure, its designated hypercube-neighbor standby assumes the master's coordination role for that scope (regional for D12, national/continental for I20, or local for O8), using the same Pass-based rerouting mechanism rather than a separate failover protocol.

Open — OI-29-16: The rule for selecting which hypercube neighbor serves as standby, when a node has multiple hypercube-adjacent neighbors, is not specified (fixed assignment at deployment time vs. dynamic election at failover time).

Open — OI-29-17: Failover trigger threshold (heartbeat/LOS timeout value) for master-node failure specifically is not specified — §10.7.2 defines the general detection sequence but not a master-failure-specific timing value.

Open — OI-29-18: Failover reversibility is not specified — whether the standby's assumption of master role is temporary (reverts automatically when the original master recovers) or requires an explicit reversion/renegotiation step.

New this session, per 창호 — Mode B requirement for Master/Standby roles: a node serving as Master or Standby (any tier — O8, D12, or I20) must operate in Mode B (§10.7.4.2, no PAI/EAI attached), not Mode A. Rationale: a Mode A node's face capacity is shared with AI processing (PAI/EAI cognitive traffic, §10.7.4.1), leaving insufficient capacity to reliably fulfill the upward hypercube relay and failover-coordination role Master/Standby requires. This constraint applies specifically to the Master/Standby role — a Mode A node remains fully valid for ordinary same-tier connectivity (O8↔O8, D12↔D12, I20↔I20 general traffic); it is only disqualified from being selected as the Master or Standby for hypercube coordination purposes.

Open — OI-29-24: The mechanism by which a node's Mode A/B status is checked before Master/Standby selection or election (§10.7.10, OI-29-16) is not specified — whether this is a static deployment-time constraint (only Mode B nodes are ever candidates) or a dynamic check performed at failover time is undetermined.

### 10.7.11 NPU–Matrix Integration Architecture (New in R6)

The Node Intelligence NPU referenced in §10.7.3 (Slot 1, Q7 hypercube maintenance) is a dedicated node-level execution engine, physically and functionally distinct from the interchangeable GPU/NPU/TPU cubification substrate defined in the Cognition Cube hardware series (WP#24, Table 9-2) — that substrate is an out-of-scope, swappable implementation choice for SEU cubification; this NPU is fixed Node Intelligence hardware and is in-scope for this document.

The NPU performs two execution functions, both triggered by upstream events rather than by learned or predicted state:

Request execution path — when a PAI/EAI cooperation request is received on Slot 2 (Face 2, §6.1) and accepted under the Core Stack negotiation protocol (Section 7), the NPU issues the corresponding configuration commands to the Multi-Layer Multi-Plane Matrix (Section 3), which executes the requested switching/path activation. The NPU does not decide whether to accept the request — that determination is made by the negotiation protocol (Section 7); the NPU executes the accepted outcome.

Failover execution path — when the Matrix detects a link-loss condition between Velsanet equipment (LOS/BER degradation, §10.7.2), it reports this to the NPU, which executes the hypercube switchover: activating the alternate path (§10.7.2) or, for a master-node failure, initiating the hypercube-neighbor standby handover (§10.7.10). The NPU executes these already-defined mechanisms; it does not introduce a new decision layer.

Open — OI-29-20: The NPU–Matrix command interface (register/signal-level protocol by which the NPU issues configuration commands to the Matrix, and by which the Matrix reports link-status events to the NPU) is not specified.

### 10.7.12 Physical-Constraint Feedback Loop (New in R6)

The NPU's execution decisions are triggered by physical metrics reported from the Matrix: link status (LOS/BER, §10.7.2) and, where instrumented, latency. The loop is: Matrix detects a physical-layer condition → Matrix reports the condition to the NPU (§10.7.11) → NPU selects and issues the corresponding pre-defined action (alternate-path activation, Master-Standby handover, or negotiated configuration) → Matrix executes the action. Every step in this loop resolves to a fixed, previously-defined mechanism (§7, §10.7.2, §10.7.10); no step involves a learned or statistically-predicted response.

Open — OI-29-21: Temperature is not currently a feedback input to the NPU — no node-level temperature sensor or interface has been defined anywhere in the Velsanet document series (distinct from OI-29-06's thermal budget/capacity gap).

# 11. Open Items Register

| ID | Section | Open Item | Status |
| --- | --- | --- | --- |
| OI-29-01 | 8 | Node-to-Dedicated-Equipment register/signal interface not yet mapped to WP#26 Table 4-2 | Open — candidate for WP#30 |
| OI-29-02 | 7.1 | Degradation-priority scenario — drafted but explicitly unconfirmed by 창호 | Unconfirmed — needs sign-off |
| OI-29-03 | 6 | Exact negotiation message encoding (bit-level format) not specified — only message types fixed | Open — candidate for WP#30 |
| OI-29-04 | 5 | Numeric density threshold for D12 aggregation not fixed — rule is qualitative | Open — decision owner 창호 |
| OI-29-05 | 10.2 | Node-level PSU topology not specified | Open — candidate for WP#31 |
| OI-29-06 | 10.3 | Node-level thermal budget not specified | Open — depends on other open items |
| OI-29-07 | 10.4 | D12/I20 chassis height not fixed | Open — depends on OI-29-04 |
| OI-29-08 | 10.5 | Management/diagnostic port connector and protocol not defined anywhere in the series | Open — genuine gap |
| OI-29-09 | 10.6 | Exact rack-rail mounting offset not dimensioned | Open — new in R2 |
| OI-29-10 | 4.1 | H6↔O8 link core count unresolved (144 vs 120); EAI attach path via O8 vs H6 unresolved | Open — two-part question |
| OI-29-11 | 10.7.1 | O/E-EO conversion latency and power consumption not characterized | Open — new in R5 |
| OI-29-12 | 10.7.2 | Slot1↔Slot2 direct-connect path diagnostic test pattern and cycle not specified | Open — new in R5 |
| OI-29-13 | 10.7.4.3 | Mode B detection timeout and renegotiation trigger threshold not numerically specified | Open — new in R5 |
| OI-29-14 | 10.7.4.3 | Mode B → Mode A transition latency not characterized | Open — new in R5 |
| OI-29-16 | 10.7.10 | Standby-neighbor selection rule (fixed vs dynamic election) not specified | Open — new in R5 |
| OI-29-17 | 10.7.10 | Master-failure-specific failover trigger threshold not specified | Open — new in R5 |
| OI-29-18 | 10.7.10 | Failover reversibility (auto-revert vs explicit reversion) not specified | Open — new in R5 |
| OI-29-20 | 10.7.11 | NPU–Matrix command interface (register/signal-level protocol) not specified | Open — new in R6 |
| OI-29-21 | 10.7.12 | No node-level temperature sensor/feedback interface defined anywhere in the series (distinct from OI-29-06) | Open — new in R6 |
| OI-29-22 | 4.6 | T4/H6 core-scaling figures not confirmed — pattern extension downward is undecided | Open — new in R8 |
| OI-29-23 | 4.6 / 10.7.3 | D12/I20 slot-internal allocation (control vs Pass-path split) not scaled from O8's 12+180 pattern | Open — new in R8, deferred to future session |
| OI-29-24 | 10.7.10 | Mechanism for checking Mode A/B status before Master/Standby selection (static vs. dynamic) not specified | Open — new in R10 |

Table 11-1. WP#29 Open Items (22 open items; OI-29-15 note below)

OI-29-08 (management/diagnostic port) is reinforced rather than duplicated by the R5 operational-layer material — no new management-port item was created; the existing OI-29-08 stands as the single register entry for this gap across both the physical (Section 10.5) and operational (Section 10.7) layers.

Cross-document note (not an open item within this document — logged for future patch): R5-4 sets O8's total core count at 1,536 (8 slots × 192 cores), which is inconsistent with the "O8 = 192 optical cores" figure stated in WP#20 §5.3, WP#24 §5.2/§7.2, WP#25 §B.13.1, and the Velsanet Architecture Compendium. Per 창호's explicit instruction, this revision was made in WP#29 only; the other documents are not modified here. This inconsistency should be recorded in Velsanet_Pending_Updates_Log as a new entry pending 창호's decision on how (or whether) to propagate the 1,536-core figure to the rest of the document series.

# 12. Conclusion and Future Document Proposals

WP#29 v1.10 completes the two deferrals recorded in WP#25's revision log (R1-4, R1-5), restates the Multi-Layer Multi-Plane Matrix principle at the node-equipment layer, fixes O8/D12/I20 block architecture including hierarchical Schlegel projection and national Velsa orchestration (Section 4.3–4.5), and defines the O8 node's full operational architecture: the Pass function, self-healing, dynamic Mode A/B slot allocation, the O8-only PAI/EAI attachment restriction, and Master-Standby active failover reused from hypercube adjacency (Section 10.7). It also corrects the O8 slot-level core allocation to 1,536 total cores (8 × 192), explicitly flagged against the rest of the document series rather than silently propagated. New in R6, it defines the Node Intelligence NPU as a dedicated, deterministic execution engine — distinct from the Cognition Cube's swappable cubification substrate — that executes accepted cooperation requests and hypercube failover as Matrix commands, and states the physical-constraint feedback loop by which Matrix-reported physical metrics trigger that execution, explicitly preserving the A=f(T) principle that Velsanet's guarantees are physical/structural rather than statistical (Sections 6.2, 10.7.11, 10.7.12). New in R7, WP#30's proposed scope (Table 2-1, Table 12-1) is clarified to explicitly include the Matrix's physical implementation (semiconductor circuit design, FPGA→ASIC process migration, die-level layout), distinguished from WP#32's board-level PCB layout. New in R8, hierarchical core scaling (Section 4.6) supersedes §10.7.8's hardware-uniformity principle: O8, D12, and I20 are now confirmed as distinct physical designs — 8/12/20 faces respectively, with per-face core counts of 192/768/3,072 (1,536/9,216/61,440 total) — rather than one uniform chassis serving all three tiers. New in R9, three Core Stack negotiation message types were added (Section 7.0a) — Release, Release_Ack, and Force_Release — closing a connection-teardown gap surfaced while drafting WP#30's wire-level encoding; Force_Release is confirmed as NPU rule-based execution, consistent with §6.2. New in R10, §10.7.10 now requires Master/Standby nodes to be in Mode B (no PAI/EAI attached) — reusing the existing Mode A/B mechanism, per 창호, rather than introducing a new node-hardware category — since a Mode A node's shared face capacity would compromise its hypercube relay and failover-coordination reliability.

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #26 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, FPGA→ASIC migration | Complete — 12 open items |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete |
| WP #29 | Network Equipment SAD (this document) | O8/D12/I20 block architecture, placement, negotiation protocol, operational architecture, NPU-Matrix execution model | Complete — v1.10, 22 open items |
| WP #30 (proposed) | Network Equipment RTL & Physical Design (Switching/Routing) | Node-to-node switching/routing logic (RTL) + Matrix physical implementation: semiconductor circuit design, FPGA→ASIC process migration, die-level layout — distinct from WP#32's PCB-level layout | Proposed — next |
| WP #31 (proposed) | Schematic & BOM | 800G transceiver, FPGA/ASIC, power circuit connections | Proposed |
| WP #32 (proposed) | PCB Layout Design | 800G PAM4 board-level placement/routing, thermal design | Proposed |
| WP #33 (proposed) | Integration Test Plan | Node-to-node connectivity, fault recovery, performance testing | Proposed |

Table 12-1. Future Document Proposals — Updated at WP#29 v1.10

Notice

This document is an internal technical design document of the Velsanet Architecture Group. All design specifications are based on software MVP validation results and will be finalized through physical hardware validation.

Velsanet Architecture Group  —  Internal Technical Document

VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

WP#30 — Network Equipment RTL & Physical Design (Switching/Routing)

Matrix Switching/Routing RTL · Register Interfaces · Semiconductor Physical Implementation

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Internal Draft — all sections drafted, 18 open items remain before Complete status; OI-30-17 reframed as vendor-agnostic qualification (§7.2.1), candidate screening reference added (§7.2.2) | v1.5 │ 2026 | Node-to-node switching/routing RTL for the Multi-Layer Multi-Plane Matrix; register/signal interfaces to WP#26 and to the Node Intelligence NPU; Matrix physical implementation (semiconductor circuit design, FPGA→ASIC process migration, die-level layout) | Resolves WP#29 OI-29-01, OI-29-03, OI-29-20; extends WP#29 §3, §10.7.11; follows WP#26's RTL/FPGA-ASIC document pattern; scope fixed at WP#29 R7 (Table 2-1/12-1) |

Republic of Korea | Velsanet Architecture Group

Status note

This document was started as a skeleton (v0.1) and is being filled in section by section, per 창호's stated order: interface definition first (Sections 3.1, 4, 6), then message encoding (5), RTL module breakdown (3.2+), timing (8), physical design (7), validation (9). Structure and naming are fixed before numbers — register bit-fields, message encoding, and physical-design numbers are added only once the logic they encode is settled, per 창호's explicit principle. No Velsanet-specific number appears in this draft unless it was already fixed in a prior document or explicitly confirmed by 창호 this session.

# 1. Purpose and Scope

WP#30 is the RTL and physical-design counterpart to WP#26 for node-level (inter-node) switching and routing, as distinct from WP#26's intra-node SEU1/SEU2 RTL. It was first proposed at WP#29 §12 (Table 12-1) and its scope was fixed at WP#29 R7: node-to-node switching/routing RTL for the Multi-Layer Multi-Plane Matrix (WP#29 §3), plus the Matrix's physical implementation — semiconductor circuit design, FPGA→ASIC process migration, and die-level layout — explicitly distinguished from WP#32's board-level (PCB) layout.

This document exists specifically to resolve three open items already registered against it in WP#29's Open Items Register (Section 11): OI-29-01 (node-to-Dedicated-Equipment register/signal interface), OI-29-03 (negotiation message bit-level encoding), and OI-29-20 (NPU–Matrix command interface).

Scope includes: Matrix switching/routing RTL architecture (Section 3); the node-to-Dedicated-Equipment register interface (Section 4); Core Stack negotiation message encoding (Section 5); the NPU–Matrix command interface (Section 6); Matrix physical implementation — circuit design, FPGA→ASIC migration, die-level layout (Section 7); timing analysis (Section 8); and hardware validation strategy (Section 9).

Out of scope: the SEU1/SEU2 RTL engines themselves (WP#26, unchanged here); node equipment's logical/electrical architecture and physical form factor at the board/chassis level (WP#29, unchanged here); schematic, BOM, and PCB-level layout (WP#31–32); integration test planning (WP#33).

# 2. Positioning in the Document Sequence

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 | SEU1/SEU2 Synchronization Specification | SEU1/SEU2 record format, PTP profiles, completeness algorithms | Complete |
| WP #26 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, FPGA→ASIC migration | Complete — 12 open items |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete |
| WP #29 | Network Equipment SAD | O8/D12/I20 block architecture, placement, Core Stack negotiation, O8 operational architecture, NPU-Matrix execution model | Complete — v1.6 (R7) |
| WP #30 (this document) | Network Equipment RTL & Physical Design (Switching/Routing) | Matrix switching/routing RTL, register interfaces, semiconductor physical implementation | All sections drafted — v1.5, OI-30-17 resolved (16nm-class) |
| WP #31 (proposed) | Schematic & BOM | 800G transceiver, FPGA/ASIC, power circuit connections | Proposed |
| WP #32 (proposed) | PCB Layout Design | 800G PAM4 board-level placement/routing, thermal design | Proposed |
| WP #33 (proposed) | Integration Test Plan | Node-to-node connectivity, fault recovery, performance testing | Proposed |

Table 2-1. Document Sequence — Updated at WP#30 (skeleton)

# 3. Matrix Switching/Routing RTL Architecture

Scope: RTL-level realization of the Multi-Layer Multi-Plane Matrix's node-equipment behavior, as defined at the architecture level in WP#29 §3 (channel switching and multi-directional transmission between O8/D12/I20 nodes) and §10.7.1 (Pass function — O/E-EO conversion, active forwarding, deterministic latency).

## 3.1 Monitor Channel Definition

Per 창호. This section resolves a naming collision surfaced during drafting: WP#20 §4.2 already fixes "Channel 1" as the Self-Awareness Bootstrap channel — a one-time, passive, deterministic negotiation that completes before any data transmission begins (device authentication, physical alignment verification, structural identity exchange). The always-on link-monitoring/switch-trigger channel needed for Matrix RTL is a distinct mechanism and is given a distinct name below to avoid conflating the two.

| ID | Defined In | Role |
| --- | --- | --- |
| BOOTSTRAP_CH | WP#20 §4.2 (unchanged) | Equipment registration and initialization negotiation. Active once, at node initialization; completes before any data transmission begins. |
| MONITOR_CH | WP#30 (new, this section) | Real-time optical link monitoring and switch-trigger channel for Matrix switching control. Active continuously, independent of BOOTSTRAP_CH. |

Table 3-1. BOOTSTRAP_CH vs. MONITOR_CH — disambiguation

### 3.1.1 Naming and ID

MONITOR_CH is the RTL-level name for the mechanism WP#29 described conceptually at §10.7.2 ("Matrix detects a physical-layer condition → reports to the NPU") and §10.7.11 (NPU–Matrix command interface, OI-29-20). This section gives that mechanism a fixed name and channel role so later sections (register map, RTL data path) have a stable reference point; it does not change WP#29's architecture, only implements it.

### 3.1.2 Functional Role

MONITOR_CH receives incoming signals from the external optical core and triggers internal Matrix switching decisions. It operates continuously after BOOTSTRAP_CH completes, independent of the bootstrap negotiation channel — the two never share state or timing.

### 3.1.3 RTL Interface

The received signal is delivered to the Matrix switch control logic via a dedicated RTL interface. The signal is interpreted as a fixed-format control word containing: link status (LOS/BER, per WP#29 §10.7.2); incoming face/slot identifier (per WP#29 §10.7.3's slot structure); and requested switching action, where applicable.

MONITOR_CH exposes three 32-bit registers, per 창호's confirmation this session that register width follows the existing WP#24 Table 1-1 convention (WORM_CTRL, WORM_STATUS/ECC, WORM_AUDIT_PTR are all 32-bit).

| Register | Width | Field Content |
| --- | --- | --- |
| MONITOR_EVENT_REG | 32-bit | Fully defined — see Table 3-2 below. |
| MONITOR_STATUS_REG | 32-bit | Fully defined — see Table 3-2c below. |
| MONITOR_CONFIG_REG | 32-bit | Fully defined — see Table 3-2d below. |

Table 3-2a. MONITOR_CH Register Set — all three registers fully defined (OI-30-09 resolved)

MONITOR_EVENT_REG control-word format, confirmed per 창호 this session:

| Bits | Field | Width | Description |
| --- | --- | --- | --- |
| 31:27 | RESERVED | 5 | Reserved for future use |
| 26:22 | FACE_ID | 5 | Target face/slot ID, 0–31 — covers I20's 20 faces (§4.6) with headroom to 32 |
| 21:8 | RESERVED | 14 | Reserved for future use |
| 7:6 | ACTION | 2 | Requested switching action — see Table 3-3 |
| 5:4 | RESERVED | 2 | Reserved for future use |
| 3 | BER_THRESHOLD | 1 | BER threshold exceeded (1 = exceeded) — event-level flag, not a continuous BER measurement, consistent with WP#29 §10.7.2's condition-detection framing |
| 2 | LOS | 1 | Loss of signal (1 = LOS detected) |
| 1:0 | RESERVED | 2 | Reserved for future use |

Table 3-2b. MONITOR_EVENT_REG Control-Word Format (32-bit) — confirmed per 창호

| ACTION | Value | Meaning | WP#29 Basis |
| --- | --- | --- | --- |
| No action | 0x0 | Default / no switching action requested | — |
| Alternate-path activation | 0x1 | Reroute around a failed link | §10.7.2 self-healing |
| Master-Standby handover | 0x2 | Hypercube-neighbor standby assumes master role | §10.7.10 |
| Negotiated configuration | 0x3 | Apply accepted Core Stack negotiation outcome | §7; §10.7.11 request execution path |

Table 3-3. ACTION Field Encoding (2-bit) — confirmed per 창호

MONITOR_STATUS_REG field layout, confirmed per 창호 this session. Scope note: MONITOR_CH is dedicated to hypercube link monitoring (LOS/BER, §3.1.2) only — PAI/EAI cooperation-request status is out of scope for this register and belongs to Slot 2/Face 2 (WP#29 §6.1) instead, handled via §6 (NPU–Matrix Command Interface) or §4 (Node-to-Dedicated-Equipment Interface).

| Bits | Field | Width | Description |
| --- | --- | --- | --- |
| 31:16 | EVENT_COUNT | 16 | Cumulative count of monitored events |
| 15:12 | RESERVED | 4 | Reserved for future use |
| 11:8 | LAST_EVENT_TYPE | 4 | 0 = none, 1 = LOS, 2 = BER, 3–15 = RESERVED (deliberately excludes cooperation-request events — out of MONITOR_CH scope, see note above) |
| 7:2 | RESERVED | 6 | Reserved for future use |
| 1 | LINK_UP | 1 | Current link status (1 = up, 0 = LOS) |
| 0 | RESERVED | 1 | Reserved for future use |

Table 3-2c. MONITOR_STATUS_REG Field Layout (32-bit) — confirmed per 창호

MONITOR_CONFIG_REG field layout, confirmed per 창호 this session:

| Bits | Field | Width | Description |
| --- | --- | --- | --- |
| 31:24 | SCALE_DOWN_THRESHOLD | 8 | Resource scale-down threshold — illustrative value only (e.g. 20%), see OI-30-11 |
| 23:16 | SCALE_UP_THRESHOLD | 8 | Resource scale-up threshold — illustrative value only (e.g. 80%), see OI-30-11 |
| 15:12 | BER_THRESHOLD_LEVEL | 4 | BER threshold setting (0–15); specific meaning per level is implementation-defined |
| 11:8 | RESERVED | 4 | Reserved for future use |
| 7:2 | RESERVED | 6 | Reserved for future use |
| 1 | EVENT_REPORT_EN | 1 | Enable NPU interrupt on event occurrence |
| 0 | MONITOR_EN | 1 | Enable the monitor channel |

Table 3-2d. MONITOR_CONFIG_REG Field Layout (32-bit) — confirmed per 창호

Open — OI-30-11: SCALE_UP_THRESHOLD/SCALE_DOWN_THRESHOLD values are not fixed — 80%/20% are illustrative only, subject to deployment-specific tuning. Field width (8 bits each) is confirmed; the actual threshold values are a deployment decision, not an architecture decision.

Note on D12/I20 applicability (per OI-29-23): The MONITOR_CH control-word format (Table 3-2b) uses a 5-bit FACE_ID field, which numerically covers O8 (8 faces), D12 (12 faces), and I20 (20 faces). However, the slot-internal allocation logic that MONITOR_CH implements — specifically the 12-core control + 180-core direct-connect Pass path split defined in WP#29 §10.7.3 for O8 — has not yet been scaled or confirmed for D12 (768 cores/face) or I20 (3,072 cores/face). Whether the control-core count stays fixed at 12 (with all additional capacity routed to the Pass path) or scales proportionally is deferred to OI-29-23. The FACE_ID width itself is sufficient; the applicability of the MONITOR_CH mechanism to D12/I20 is not yet determined.

### 3.1.4 Always-On Operation

Unlike BOOTSTRAP_CH, which is active only during initial device registration (WP#20 §4.2), MONITOR_CH remains active throughout the entire operational lifecycle of the node — consistent with WP#29 §10.7.2's self-healing sequence requiring continuous LOS/BER detection, not a one-time check.

## 3.2 Event-Triggered Resource Management

Per 창호. Traffic-driven activation/deactivation of optical cores/paths, governed by fixed, pre-defined rules evaluated against MONITOR_CONFIG_REG's threshold fields — not by NPU judgment or learned/predicted traffic patterns. This keeps resource scaling consistent with §6.2's principle that the NPU executes pre-defined deterministic responses rather than deciding outcomes itself.

| Rule | Trigger | Action |
| --- | --- | --- |
| Scale-up | Traffic exceeds SCALE_UP_THRESHOLD | Activate additional dormant optical core(s)/path(s) |
| Scale-down | Traffic falls below SCALE_DOWN_THRESHOLD | Deactivate previously-activated optical core(s)/path(s) |
| Self-healing (existing, restated) | LOS/BER detected (MONITOR_EVENT_REG) | Alternate-path activation, ACTION = 0x1 (Table 3-3) |

Table 3-4. Event-Triggered Resource Management Rules — confirmed per 창호

This is deliberately scoped as rule-based, event-triggered reconfiguration — the same category of mechanism as WP#29 §10.7.4's Mode A/B transition (fixed rule, evaluated against a threshold, executed deterministically), not continuous statistical traffic prediction. Where the specific threshold values come from (SCALE_UP/DOWN_THRESHOLD) is separately flagged as OI-30-11; the rule structure itself is fixed.

## 3.3 RTL Data Path (resolves OI-30-10)

Per 창호. The RTL circuit path connecting MONITOR_CH to the Matrix switch control logic:

Module path: MONITOR_CH receiver → control-word decoder → rule evaluator (§3.2's Scale-up/down and self-healing rules) → Matrix switch control logic.

| Signal | Direction | Description |
| --- | --- | --- |
| monitor_event_valid | MONITOR_CH → decoder | Asserted when a new MONITOR_EVENT_REG value is ready to be read |
| monitor_event_ready | decoder → MONITOR_CH | Asserted when the decoder is ready to accept the next event (back-pressure) |
| monitor_control_word[31:0] | MONITOR_CH → decoder | The MONITOR_EVENT_REG control word itself (Table 3-2b) |

Table 3-5. MONITOR_CH → Matrix Switch RTL Interface Signals — confirmed per 창호

Open — OI-30-12: Clock domain relationship between MONITOR_CH and the Matrix switch control logic is not fixed — whether they share one clock domain or MONITOR_CH runs on its own clock with a CDC (clock-domain-crossing) boundary at the decoder is undecided.

[상세 미정 — 차후 세션에서 확정] The remaining Matrix RTL module breakdown (switch fabric, path-selection logic proper, fault-circumvention state machine internals) beyond the Monitor Channel and its immediate RTL interface is not yet drafted — see OI-30-04.

# 4. Node-to-Dedicated-Equipment Register Interface (mostly resolves OI-29-01/OI-30-01)

Correction, per 창호 this session: the mapping target originally referenced ("WP#26 Table 4-2") does not exist in any project document. The actual register set is WP#24 §3's Transform Engine registers (DEMUX Engine, SEU Assembly Engine, MUX Engine) — WP#26 is the RTL implementation of those registers, not a separate register source. This section maps against WP#24 §3 directly.

Scope: the register/signal-level interface between the node equipment (WP#29 Slot structure, §10.7.3) and the Dedicated Equipment RTL, implemented via the Transform Engine registers defined in WP#24 §3.

## 4.1 Slot 3/4 (Inbound Data Path) — SEU Assembly Registers

Per 창호's correction: these registers concern actual SEU assembly data, which per WP#29 §10.7.3 flows through Slot 3/4 ("inbound data reserve"), not Slot 2 (control-only, §6.1 Face 2).

| WP#24 Register | Width | Direction | Description |
| --- | --- | --- | --- |
| DEMUX_LANE_CFG | 16-bit | Node → DE | Set PAI (0x0) / EAI (0x1) mode |
| SEU_WINDOW_CTRL | 16-bit | Node → DE | Set SEU window (800–1500 ms) |
| SEU_CHANNEL_COMPLETE | 16-bit | DE → Node | Report per-channel completeness bitmap |
| SEU_MISSING_FLAG | 16-bit | DE → Node | Report missing-channel flags |
| SEU_ASSEMBLY_STATUS | 8-bit | DE → Node | Report SEU assembly state (0–3, WP#24 §3.2) |

Table 4-1. Slot 3/4 ↔ WP#24 SEU Assembly Engine Register Mapping — confirmed per 창호

## 4.2 Slot 2 (Control/Cooperation Path) — Deferred to Section 5

Slot 2's 12 control cores (§10.7.3, Face 2 §6.1) carry PAI/EAI cooperation-request signaling — this is the Core Stack negotiation protocol's wire format, not a Transform Engine register. It is deliberately not mapped in this section; it will be defined when Section 5 (Core Stack Negotiation Message Encoding, resolves OI-29-03/OI-30-02) is drafted.

## 4.3 Slot 1↔2 Pass Path — No Register (Direct Optical Connection)

The 180 direct-connect cores between Slot 1 and Slot 2 (§10.7.3, realizing the "external loopback" per §10.7.2) are a direct optical connection with O/E-EO conversion at each end — there is no WP#24 register involved, consistent with §10.7.1's description of the Pass function as physical forwarding rather than register-mediated switching.

## 4.4 Bundle Uplink (Per-Instance) — MUX Engine Registers

Per 창호: the MUX Engine (WP#24 §3.3) operates at a different layer than the O8 Slot structure — it maps an individual PAI/EAI instance's cognition-bus lanes to that instance's own 24-core Bundle uplink, beneath the 192-core Slot layer rather than assigned to any one Slot.

| WP#24 Register | Width | Direction | Description |
| --- | --- | --- | --- |
| MUX_CH_MAP | 128-bit | Node → DE | Cognition-bus lane → 24-core Bundle mapping table |
| MUX_BUNDLE_STATUS | 8-bit | DE → Node | Bundle status (BUNDLE_READY / PARTIAL_BUNDLE / OVERFLOW) |

Table 4-2. Bundle Uplink (Per-Instance) ↔ WP#24 MUX Engine Register Mapping — confirmed per 창호

## 4.5 Common Monitoring (All Slots) — DEMUX Engine Registers

| WP#24 Register | Width | Direction | Description |
| --- | --- | --- | --- |
| DEMUX_FAULT_EXCLUDE | 384-bit | DE → Node | Bitmap of faulty cores auto-excluded |
| DEMUX_LATENCY_MON | 16-bit | DE → Node | Measured processing latency (target <2ns, WP#24 §3.1) |
| DEMUX_CLK_PLL_STATUS | 16-bit | DE → Node | Per-channel PLL lock bitmap |
| DEMUX_NONCOLLISION_CHK | 1-bit | DE → Node | Non-collision verification complete (POST-time check, Core(ri)∩Core(rj)=∅) |

Table 4-3. Common Monitoring ↔ WP#24 DEMUX Engine Register Mapping — confirmed per 창호

## 4.6 EAI-Specific Control Plane

| WP#24 Register | Width | Direction | Description |
| --- | --- | --- | --- |
| SEU_CTRL_PLANE_WINDOW | 16-bit | Node → DE | EAI-only control-plane window (CH9–CH16, independent of the cognition-plane window) |

Table 4-4. EAI Control Plane ↔ WP#24 Register Mapping — confirmed per 창호

Open — OI-30-01: (Mostly resolved) Register-level mapping is confirmed for Slot 3/4, Bundle Uplink, common monitoring, and EAI control plane (Tables 4-1 through 4-4). Two parts remain open: (a) Slot 2's cooperation-request wire format, deferred to Section 5; and (b) handshake/timing between node and Dedicated Equipment (Valid/Ready vs. DMA-style) — not addressed this session.

# 5. Core Stack Negotiation Message Encoding (resolves OI-29-03/OI-30-02)

Scope: bit-level encoding of the seven negotiation message types now fixed at the architecture level in WP#29 Table 7-1/7-1a (Attach; capability/health handshake; layer activation/deactivation request; equipment-initiated renegotiation; Release; Release_Ack; Force_Release, the last three added at WP#29 R9) — WP#29 fixes only the existence and message types, explicitly deferring the wire format to this document. These messages are carried over Slot 2's 12 control cores (§4.2).

## 5.1 Frame Format

128-bit fixed-length frame, confirmed per 창호 this session, following the precedent already set by WP#24 §2.4's ACL violation log format (EPOCH_ID/TIMESTAMP_NS/REQUESTOR_ID/... = 128-bit unified audit format) — a negotiation message records who requested what and when, structurally similar to an audit entry.

## 5.2 Common Header

| Bits | Field | Width | Description |
| --- | --- | --- | --- |
| 127:124 | MSG_TYPE | 4 | See Table 5-1 |
| 123:116 | SEQ_NUM | 8 | Message sequence number |
| 115:68 | TIMESTAMP | 48 | Reuses WP#24 §2.4's TIMESTAMP_NS width |
| 67:52 | ENTITY_ID | 16 | PAI/EAI instance identifier — confirmed per 창호 |
| 51:0 | PAYLOAD | 52 | Message-type-specific, see §5.3 |

Table 5-0. Common Header (76 bits) + Payload (52 bits) = 128-bit frame — confirmed per 창호

## 5.3 Message Types and Payloads

| MSG_TYPE | Name | Direction | Purpose |
| --- | --- | --- | --- |
| 0x1 | Attach | PAI/EAI → O8 | Equipment registration request |
| 0x2 | Handshake | Bidirectional | Capability/health check |
| 0x3 | Activation | Bidirectional | Layer activation/deactivation |
| 0x4 | Renegotiation | O8 → PAI/EAI | Renegotiation request (e.g. on fault/power shortage) |
| 0x5 | Release | PAI/EAI → O8 | Explicit, intentional disconnection request (WP#29 R9) |
| 0x6 | Release_Ack | O8 → PAI/EAI | Confirms release completed (WP#29 R9) |
| 0x7 | Force_Release | O8 → PAI/EAI | Node-initiated forced disconnection; NPU rule-based execution of §10.7.2, per WP#29 R9 |

Table 5-1. MSG_TYPE Encoding — 7 types, confirmed per 창호 / WP#29 R9

Payload (52-bit) structure per message type:

| MSG_TYPE | Payload Field(s) | Notes |
| --- | --- | --- |
| 0x1 Attach | CAPABILITY_FLAGS (bitmap of supported layers) | PAI/EAI mode bit deliberately excluded — already carried by DEMUX_LANE_CFG (§4.1); avoids two sources of truth for the same fact, per 창호 |
| 0x2 Handshake | HEALTH_STATUS (2-bit) + RESERVED (50-bit) | 0x0 OK, 0x1 DEGRADED, 0x2 FAULT, 0x3 UNKNOWN |
| 0x3 Activation | LAYER_ID (Core Stack layer 0–3, WP#25) + ACTION (1-bit: activate/deactivate) + RESERVED |
| 0x4 Renegotiation | REASON_CODE (e.g. power shortage, WP#29 §7) + RESERVED |
| 0x5 Release | RELEASE_REASON (8-bit: 0x0 normal termination, 0x1 user request, 0x2 session expiry, 0x3 resource reclaim) + RESERVED |
| 0x6 Release_Ack | Echoes the RELEASE_REASON being acknowledged + RESERVED |
| 0x7 Force_Release | FORCE_REASON (8-bit: 0x1 link loss, 0x2 power shortage, 0x3 hardware fault, 0x4 timeout) + RESERVED |

Table 5-2. Per-Message-Type Payload Structure (within the 52-bit PAYLOAD field) — confirmed per 창호

Open — OI-30-14: Exact bit-level sub-field widths within each message type's 52-bit payload (e.g. how many bits for CAPABILITY_FLAGS vs. its RESERVED remainder) are fixed only where explicitly confirmed above (HEALTH_STATUS = 2-bit, RELEASE_REASON/FORCE_REASON = 8-bit); other fields' exact widths (CAPABILITY_FLAGS, LAYER_ID, REASON_CODE) are named but not bit-width-finalized.

# 6. NPU–Matrix Command Interface (resolves OI-29-20/OI-30-03)

Per 창호: this section defines no new registers. §3.1.3's MONITOR_EVENT_REG and MONITOR_CONFIG_REG already implement WP#29 §10.7.11's NPU–Matrix command interface at the RTL level (as stated at §3.1.1). §6 re-presents them from the NPU–Matrix relationship's point of view, rather than defining a separate NPU_CMD_REG/MATRIX_EVENT_REG pair — which would have duplicated the same information in two places.

## 6.1 NPU → Matrix Command Interface

Realized by MONITOR_CONFIG_REG (§3.1.3, Table 3-2d). The NPU configures Matrix monitoring/scaling behavior — MONITOR_EN, EVENT_REPORT_EN, BER_THRESHOLD_LEVEL, SCALE_UP/DOWN_THRESHOLD — through this register; no separate command register exists.

## 6.2 Matrix → NPU Event Interface

Realized by MONITOR_EVENT_REG (§3.1.3, Table 3-2b) and MONITOR_STATUS_REG (Table 3-2c). The Matrix reports link-status events (LOS, BER, FACE_ID, ACTION) to the NPU through these registers; Force_Release (WP#29 §7.0a/R9) is executed via this same path, per §3.3's rule evaluator.

## 6.3 Interrupt / Polling

Controlled by MONITOR_CONFIG_REG.EVENT_REPORT_EN (§3.2d): when set, the Matrix's event reporting to the NPU is interrupt-driven; when clear, the NPU must poll MONITOR_EVENT_REG/MONITOR_STATUS_REG directly. No new field is needed — this bit already exists.

## 6.4 Open Items

Open — OI-30-15: LAYER_MASK — a bitmask over Core Stack Layers 0–3 (WP#25 §B.13.1) — is referenced as a plausible Activation-message field (§5.3) but WP#25 defines the layers themselves, not a bitmask encoding of them. Bit assignment not finalized.

Open — OI-30-16: Command latency budget for NPU→Matrix execution (event detection to action execution) is not specified. WP#25 Table A-5's <100ns figure is a PTP synchronization accuracy budget, not a command-execution latency budget — the two are different quantities and the latter has no fixed value yet.

# 7. Matrix Physical Implementation

Scope: physical realization of the Matrix switching/routing RTL (Section 3) as semiconductor hardware — circuit design, FPGA→ASIC process migration following the pattern already established in WP#26, and die-level layout. Explicitly distinguished from WP#32's board-level (PCB) layout, per WP#29 R7.

## 7.1 FPGA Prototype Target

Per WP#24 Table 5-1 / WP#26 §4.4 precedent: Xilinx UltraScale+ recommended for prototyping and initial low-volume production, inherited here for the Matrix switch fabric's FPGA stage.

Resource floor and transceiver requirement, per 창호 this session: estimated resource requirement is ≥VU9P equivalent (WP#26 OI-02's candidate device, used here as a floor given the Matrix fabric's larger port count than a single SEU1 engine instance). The 800G PAM4 SerDes requirement points toward GTM-class transceivers (58Gbps, already named as a WP#26 §7.2 PHY option for 800G) rather than the older GTY class — which in practice points toward VU19P or a newer UltraScale+/Versal family, since GTM support is not present on earlier-generation parts like VU9P/VU13P.

Open — OI-30-05: (Narrowed) A specific device is not yet selected, but the requirement is narrowed: ≥VU9P-equivalent resource floor, GTM-class transceiver support (→ VU19P or newer family, not VU9P/VU13P themselves, which lack GTM). Precise LUT/BRAM/DSP/SerDes occupancy still requires RTL synthesis against a chosen device, not yet performed — same open status as WP#26 OI-02 for SEU1, now with a narrower device-family constraint.

## 7.2 ASIC Process Node (Resolved this session)

Decision, per 창호: 16nm-class mature node, phased. WP#26 §4.5's SEU1 rationale (mature node sufficient, control-dominated logic) was explicitly NOT assumed to transfer automatically to the Matrix fabric — see the density/latency concern raised earlier this session. Having weighed that concern against Velsanet's current program stage (architecture record / RTL / PCB design — not pre-tapeout), 창호 selected a 16nm-class mature node for the initial ASIC target, with a 5nm-class migration explicitly kept open as a later roadmap step rather than a current commitment.

Rationale (창호's evaluation): at Velsanet's current stage, a 16nm node offers a mature EDA toolchain and rich reference designs (predictable schedule), lower mask cost and higher yield (lower up-front risk), and — per the power estimate below — sufficient PMIC headroom without requiring the density gains a leading-edge node would provide. 800G-class switching has industry precedent at 16nm. The roadmap explicitly keeps a future 16nm→5nm migration open once market response and power/area competitiveness at scale justify the substantially higher NRE.

| Phase | Target | Trigger / Rationale |
| --- | --- | --- |
| Phase 1 (current) | 16nm-class mature node | RTL/physical design proceeds on 16nm reference; PMIC headroom confirmed (170W available vs. ~110–155W estimated ASIC draw, see below); low mask cost keeps focus on design verification |
| Phase 2 (future, not committed) | 5nm-class node, contingent | Revisit once 16nm product market response is known, and power/area optimization is confirmed as a competitive requirement; would require accumulated 5nm design experience via external IP/foundry partnership |

Table 7-1a. ASIC Process Node Roadmap — decided per 창호 this session

Power estimate supporting Phase 1 (order-of-magnitude, not a synthesis-derived figure): WP#31 §5 Table 5-1's VU19P FPGA rail set totals approximately 225W at maximum current draw (170W VCCINT alone, MPM3695-100 ×2 at up to 200A). Applying the general FPGA→ASIC power-reduction precedent already used for SEU1 (WP#26 §4.5/§8.1, though not automatically assumed to transfer at Matrix scale — §7.2 above), an ASIC implementation might draw on the order of 110–155W, leaving headroom under the existing 170W VCCINT-equivalent PMIC capacity. This is an estimate for process-node decision purposes, not a die power budget — see OI-30-06 (§7.4), which remains open for the actual synthesis-derived figure.

Open — OI-30-17: (Resolved — 16nm-class, Phase 1; reframed per 창호) Rather than narrowing to one named foundry, this item is now a vendor-agnostic qualification: any foundry satisfying §7.2.1's requirements qualifies, and the specific vendor remains genuinely open. Three independent candidate-screening reports (ARM+Samsung, Intel+Broadcom, NVIDIA+TSMC — each self-labeled "Independent Technical Analysis," not vendor-committed data) are logged as screening references, not as a shortlist to choose among. The 5nm migration trigger conditions (Table 7-1a, Phase 2) remain directional, not committed criteria with numeric thresholds.

## 7.2.1 Foundry Qualification Requirements (Vendor-Agnostic)

Per 창호's explicit direction this session: the foundry decision for OI-30-17 is not about choosing among named vendors — it is about defining the requirements any foundry must satisfy. Whichever vendor meets every row below qualifies; none is pre-selected here.

| Requirement | Specification | Source |
| --- | --- | --- |
| Process node | 16nm-class mature node (Phase 1); 5nm-class as future roadmap, trigger conditions not yet numeric | WP#26 §8.2 precedent; WP#30 Table 7-1a |
| SerDes | GTM-class, 800G PAM4 per-core support | WP#26 §4.4/§7 |
| Power envelope | Must accommodate ASIC-stage estimate of 110–155W (order-of-magnitude, not synthesis-derived) | WP#31 §5.1a |
| Silicon photonics | Foundry must offer a silicon photonics PDK compatible with MOCT's polymer waveguide / semiconductor matrix layer | WP#03 §8.3 |
| Advanced packaging | CPO-class packaging (die-to-photonics co-packaging), not standard flip-chip/wire-bond only | WP#34 §3 |
| MEMS alignment | Lithographic-precision static MEMS alignment structures manufacturable at the same process node | WP#03 §8.3; WP#34 §4 |
| ASIC pin compatibility | Pinout must be a superset or functional subset of the VU19P reference pinout (Option A shared footprint) | WP#32 §3.1.1 |

Table 7-1b. Foundry Qualification Requirements — vendor-agnostic, confirmed per 창호

Three independent candidate-screening reports exist (Velsanet_ARM_Samsung_Platform_Mapping_Report, Velsanet_Intel_Broadcom_Platform_Mapping_Report, Velsanet_NVIDIA_TSMC_Platform_Mapping_Report, all v1.0, March 2026) — each self-labeled "Independent Technical Analysis," meaning they are Velsanet-authored public-information mappings, not vendor-committed PDK or pinout data. They are logged here as screening references against the table above, not as a shortlist narrowing OI-30-17 to three options. Any of the three, a combination, or an unlisted fourth vendor qualifies equally if it satisfies Table 7-1b — the requirement, not the vendor name, is what this document fixes.

## 7.2.2 Candidate Screening Reference (Non-Binding)

The three independent screening reports do include vendor-specific detail relevant to Table 7-1b, not just the one-line summary already logged in §7.2.1. Restated here for reference — this table describes what each report claims about its own vendor pairing; it is not independently verified against actual vendor engagement, and listing it here does not narrow OI-30-17 to these three.

| Dimension (Table 7-1b) | ARM + Samsung | Intel + Broadcom | NVIDIA + TSMC |
| --- | --- | --- | --- |
| Process node cited | Samsung SF3/SF4 (3nm/4nm GAA/FinFET) | Intel 18A (1.8nm-class RibbonFET GAA) | TSMC N3P (3nm) |
| Silicon photonics PDK | Samsung SiPho PDK — integrated waveguide structures | Intel SiPho PDK — waveguides, ring modulators, Ge photodetectors, grating couplers, co-fabricated with CMOS | TSMC SiPho PDK — waveguides, modulators, photodetectors, grating couplers on CMOS substrate |
| MEMS-relevant process control | SF4 GAA dimensional control cited for MEMS alignment structures; report claims dedicated MEMS foundry capability | 18A's dimensional control cited generally; report does not detail a separate MEMS-specific process line | Report does not detail MEMS-specific process control beyond general N3P precision |
| Advanced/CPO-class packaging | H-Cube / X-Cube heterogeneous integration — multi-die stacking of photonic + electronic layers | Intel CPO program (cited production experience) + EMIB 2.5D die-to-die interconnect | CoWoS-L (2.5D, compute+HBM co-integration) + InFO (thin fan-out multi-layer) |
| Production maturity claim | Not separately claimed beyond process node availability | Photonics heritage since mid-2000s; report claims currently-shipping 400G/800G CPO modules | Report claims SiPho "production-proven at scale," cites hyperscaler PIC use |

Table 7-1c. Candidate Screening Reference — restated from the three v1.0 (March 2026) "Independent Technical Analysis" reports, self-reported by each report, not vendor-verified

This table is descriptive, not evaluative: it restates what each report says about itself, not a scored comparison. None of the three fields (process node, PDK, packaging) is independently confirmed against the actual vendor's public or NDA documentation by this document series — that confirmation is exactly what OI-30-17 still requires before any vendor is engaged.

## 7.3 Die-Level Floorplan and Implementation Style

Per WP#26 §8.2 precedent (Implementation Style Decision — full-custom / standard-cell / structured ASIC, mapped by block based on requirements):

| Block | Candidate Style | Rationale (draft, follows WP#26 §8.2 pattern) |
| --- | --- | --- |
| Switch fabric core (crossbar) | Full-custom or highly-structured — TBD pending OI-30-17 | High port-count switching favors density/timing-optimized custom layout, similar to WP#26's treatment of safety/WORM-adjacent logic, but for different reasons (throughput, not isolation) |
| Control-word decoder / rule evaluator (§3.1, §3.2) | Standard-cell (synthesized) | State-machine/control-dominated logic — matches WP#26's SEU1 Assembly Engine treatment |
| MONITOR_CH registers (§3.1.3) | Standard-cell (synthesized) | Register-file logic, same category as WP#26's synthesized blocks |

Table 7-1. Candidate Implementation Style by Block — draft, follows WP#26 §8.2 pattern

Open — OI-30-06: (Retained) Die-level floorplan (physical block placement, not just implementation style) is not yet drafted — depends on OI-30-17's process-node decision and the RTL module breakdown still open under OI-30-04.

## 7.4 Power / Thermal Budget

Per WP#26 §4.5/§5.5 precedent, SEU1/SEU2's die-level power is stated as bounded primarily by SerDes/PHY interface count rather than control logic — but this rationale does not transfer directly to the Matrix switch fabric, whose SerDes count (1,536+ cores) is far larger than a single SEU1/SEU2 engine instance's, and whose switching logic itself (not just PHY) may be a significant power contributor at this port count.

Open — OI-30-06: (Restated, power aspect) Die-level power/thermal budget for the Matrix switch fabric is not estimated — WP#26's "PHY-dominated" conclusion for SEU1 is not assumed to transfer given the much larger port count; a separate estimate is needed once OI-30-17 (process node) and the RTL module breakdown (OI-30-04) are further along.

## 7.5 FPGA→ASIC Migration Path

Per WP#26 §8.3 precedent, the same staged, tool-agnostic flow is inherited: RTL functional simulation → FPGA bring-up and in-system validation → timing closure against budgets (Section 8 of this document) → physical verification and isolation checks specific to the Matrix's own boundaries (e.g. MONITOR_CH's clock-domain question, OI-30-12) → tape-out sign-off. As in WP#26, specific EDA tooling is left to the implementing team as an execution-time decision outside the architecture record.

# 8. Timing Analysis

Scope: timing closure for the Matrix RTL (Section 3) and its physical implementation (Section 7), following the timing-analysis pattern WP#26 already established for the SEU1/SEU2 RTL engines.

## 8.1 Target Clock Frequency

Open — OI-30-07: (Retained) Target clock frequency and timing-closure budgets for the Matrix switch fabric are not yet fixed — depends on the RTL module breakdown (OI-30-04) and process-node decision (OI-30-17).

## 8.2 Input Stage Latency

Confirmed, reusing WP#24 §3.1: DEMUX_LATENCY_MON's target is <2ns (WP#24 Table 3-1 / this document's Table 4-3, §4.5). This applies to the DEMUX Engine's own processing latency at the node's input stage, ahead of the Matrix switch fabric proper.

## 8.3 Matrix Switching Latency

Open — OI-30-16: (Retained) Matrix switching latency — from MONITOR_CH event detection to executed switching action — is not specified. Distinct from §8.2's input-stage latency and from §8.4's PTP accuracy budget.

## 8.4 PTP Synchronization vs. Matrix Switching Latency

The PTP synchronization accuracy budget is specified in WP#25 Table A-5 as <100ns. This budget governs time synchronization between SEU1/SEU2 domains — it does not govern the Matrix switch fabric's internal switching latency.

The Matrix switching latency (§8.3, OI-30-16) is a separate budget and is not to be conflated with the PTP accuracy budget. This section explicitly maintains the distinction established at §6.4: WP#25 Table A-5's <100ns figure is a PTP synchronization accuracy budget, not a command-execution or switching-latency budget.

# 9. Hardware Validation Strategy

Scope: validation layers and pass criteria for the Matrix RTL and physical implementation, following the pattern of WP#26 Table 9-1 (staged verification/validation plan) and WP#28's field validation procedure.

| Stage | Scope | Basis |
| --- | --- | --- |
| 9.1 RTL Simulation | Functional verification of all Matrix RTL modules (switch fabric, MONITOR_CH, control-word decoder, rule evaluator) | Follows WP#26 Table 9-1's RTL-simulation stage |
| 9.2 FPGA Bring-up | In-system validation of 800G interface + Matrix switching on the Xilinx UltraScale+ target (§7.1) | Follows WP#26 Table 9-1's FPGA bring-up stage and this document's §7.1 |
| 9.3 ASIC Post-Silicon | Timing, power, and signal-integrity validation on fabricated silicon | Follows WP#26 Table 9-1's ASIC sign-off stage |
| 9.4 Field Validation | Self-healing/alternate-path activation validated under real link-loss conditions in the field | Follows WP#28 Table 6-1's field validation procedure pattern |

Table 9-1. Staged Hardware Validation Plan — follows WP#26/WP#28 precedent

Open — OI-30-08: (Retained) Specific test items and pass/fail criteria for each of the four stages above are not yet drafted — the stages themselves are fixed, but exit criteria (analogous to WP#26 Table 9-1's per-row "Exit Criterion" column) are not.

Open — OI-30-20: Pass criteria specifically for self-healing/alternate-path field validation (§9.4) — e.g. maximum acceptable recovery time, required fault-injection coverage — are not defined.

# 10. Open Items Register

| ID | Section | Open Item | Status |
| --- | --- | --- | --- |
| OI-30-01 | 4 | Register mapping confirmed for Slot 3/4, Bundle Uplink, monitoring, EAI control plane (Tables 4-1–4-4); Slot 2 wire format and handshake/timing remain open | Partially resolved — narrowed |
| OI-30-02 | 5 | 128-bit frame format, common header, and per-message-type payload structure confirmed for all 7 message types (Tables 5-0/5-1/5-2) | RESOLVED (mostly) — sub-field widths narrowed to OI-30-14 |
| OI-30-14 | 5.3 | Exact bit widths for CAPABILITY_FLAGS, LAYER_ID, REASON_CODE payload sub-fields not finalized (HEALTH_STATUS/RELEASE_REASON/FORCE_REASON are fixed) | Open — new, narrowed from OI-30-02 |
| OI-30-03 | 6 | NPU–Matrix interface confirmed as a reuse of MONITOR_CONFIG_REG/MONITOR_EVENT_REG (§3.1.3) — no separate register needed | RESOLVED — via §3.1.3 reuse, no new registers |
| OI-30-15 | 6.4 | LAYER_MASK bit assignment over Core Stack Layers 0–3 not finalized | Open — new |
| OI-30-16 | 6.4 | NPU→Matrix command execution latency budget not specified (distinct from WP#25's PTP accuracy budget) | Open — new |
| OI-30-04 | 3 | Matrix switching/routing RTL module breakdown and state-machine design not yet drafted | Open — new |
| OI-30-05 | 7.1 | Device family narrowed to VU19P/newer (GTM-class 800G PAM4 SerDes requirement excludes VU9P/VU13P); specific part and occupancy still not decided | Open — narrowed further |
| OI-30-06 | 7.3/7.4 | Die-level floorplan and power/thermal budget not yet drafted — depends on OI-30-17 | Open — unchanged, now split floorplan/power |
| OI-30-17 | 7.2 | RESOLVED: 16nm-class mature node, Phase 1 (Table 7-1a); reframed as vendor-agnostic qualification (Table 7-1b, §7.2.1) — any foundry meeting those requirements qualifies; specific vendor remains open, and the three candidate-screening reports (ARM+Samsung, Intel+Broadcom, NVIDIA+TSMC) are references, not a shortlist | Resolved — reframed as vendor-agnostic qualification |
| OI-30-07 | 8 | Target clock frequency and timing-closure budgets not yet fixed | Open — new |
| OI-30-08 | 9 | 4 validation stages fixed (Table 9-1); per-stage exit criteria not yet drafted | Open — narrowed |
| OI-30-20 | 9.4 | Pass criteria for self-healing/alternate-path field validation (recovery time, fault-injection coverage) not defined | Open — new |
| OI-30-09 | 3.1.3 | MONITOR_STATUS_REG/CONFIG_REG field layout | RESOLVED — Table 3-2c/3-2d |
| OI-30-10 | 3.3 | RTL data path from MONITOR_CH into Matrix switch fabric | RESOLVED — Table 3-5 (module path + signals); clocking split out as OI-30-12 |
| OI-30-11 | 3.1.3 / 3.2 | SCALE_UP/DOWN_THRESHOLD values not fixed — 80%/20% illustrative only | Open — new, deployment decision |
| OI-30-12 | 3.3 | Clock domain relationship between MONITOR_CH and Matrix switch control logic (shared vs. CDC) not fixed | Open — new |
| (cross-ref) OI-29-23 | 3.1.3 | MONITOR_CH's O8-based slot allocation logic (12 control + 180 Pass) not yet scaled/confirmed for D12/I20 — FACE_ID width itself is sufficient (5-bit covers 20 faces) | Open — tracked in WP#29, noted here per 창호 |
| OI-30-13 | 4 | Handshake/timing between node and Dedicated Equipment (Valid/Ready vs. DMA-style) not addressed | Open — new |

Table 10-1. WP#30 Open Items — v1.5 (18 own items + 1 cross-referenced; OI-30-17 reframed as vendor-agnostic qualification)

# 11. Conclusion and Next Steps

OI-30-17 (ASIC process node) is resolved this session: 창호 selected a 16nm-class mature node for Phase 1, explicitly not inheriting SEU1's precedent automatically but arriving at the same class of node through independent reasoning suited to Velsanet's current program stage (architecture record / RTL / PCB design, not pre-tapeout) — mature toolchain, lower NRE risk, and confirmed PMIC headroom (≈225W VU19P FPGA envelope vs. an estimated 110–155W ASIC draw). A 5nm-class migration is kept as an explicit, non-committed Phase 2 roadmap item (Table 7-1a). This resolution substantially unblocks WP#31's remaining ASIC-stage BOM item and WP#32's §3 (component placement) and §6 (thermal design), both previously blocked on this item. Specific foundry selection remains open, and per 창호's explicit direction this session, this document no longer frames that as narrowing toward one named vendor — §7.2.1's Table 7-1b fixes vendor-agnostic qualification requirements instead, so any foundry meeting them qualifies. Three independent candidate-screening reports (ARM+Samsung, Intel+Broadcom, NVIDIA+TSMC) are logged as references against that table, not as a shortlist. This is the last major cross-document blocker in the WP#30/31/32 chain — downstream propagation into WP#31 §8 and WP#32 §3/§6 is the natural next step.

Velsanet Architecture Group  —  Internal Technical Document (Skeleton Draft)

VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

WP#31 — Schematic & BOM

System Block Diagram · Component Selection · Power/Clock Distribution · 800G Optical Interface · Bill of Materials

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Internal Draft — all sections (§3–8) drafted, updated for WP#30 OI-30-17, WP#29 R10 (Master/Standby Mode B), and QSFP-DD/citation corrections (OI-31-11, OI-31-12) | v1.2 │ 2026 | Node equipment schematic-level design (System Block Diagram, Component Selection, Power/Clock Distribution, 800G Optical Interface) and Bill of Materials — die/board-level component connections, distinct from WP#32's PCB placement/routing | Extends WP#24 §4 (electrical interfaces), §7/Figure 7-1 (PCB component block list); WP#23 §6 (BOM pattern); WP#26 §4.4/§7 (FPGA target, 800G interface); WP#30 (Matrix RTL/physical implementation this schematic realizes); scope fixed at WP#29 R7 (Table 2-1/12-1) |

Republic of Korea | Velsanet Architecture Group

Status note

This document is a skeleton, per 창호's stated method (following the WP#30 pattern): section scopes and open items are fixed now; component selection, specific part numbers, and BOM values are filled in section by section in later sessions, grounded only in what prior documents (WP#23 §6, WP#24 §4/§7, WP#26 §4.4/§7) actually specify. No part number, voltage, or dimension is invented in this draft.

# 1. Purpose and Scope

WP#31 is the schematic-level design and Bill of Materials document for Velsanet network equipment (O8/D12/I20 nodes), realizing WP#30's Matrix switching/routing RTL and physical implementation as an actual component-level circuit. It was proposed at WP#29 §12 (Table 12-1) and its scope was fixed at WP#29 R7: component selection, power/clock distribution, and the 800G optical interface at the schematic (component-connection) level — explicitly distinct from WP#32's PCB-level (board placement/routing) layout.

ASIC-stage assumptions in this document (§5.1a, §8 item 12) are based on WP#30's resolved process-node decision (16nm-class, Phase 1, WP#30 Table 7-1a) — foundry selection itself remains open there and is not decided in this document.

New, per 창호 this session — Master/Standby BOM variant: per WP#29 R10 (§10.7.10), any node designated Master or Standby for hypercube coordination must be Mode B (no PAI/EAI attached). Since the Dedicated Equipment card (WP#24 Figure 7-1 — citation unverified, see OI-31-12) carries PAI/EAI's SEU1/SEU2 RTL and is treated in this document as a fixed sub-assembly (§1, out of scope), a Master/Standby-designated node's BOM (§8) omits that sub-assembly entirely, rather than populating it and leaving it unused. This is stated here as a BOM-variant principle; the specific mechanism (separate BOM line-item set vs. a populate/no-populate option on one BOM) is not yet decided — see OI-31-10.

Scope includes: system block diagram extending WP#24 Figure 7-1's component list to the node-equipment level (Section 3); component selection for FPGA/ASIC, optical transceivers, memory, and power ICs (Section 4); power distribution network (Section 5); clock distribution (Section 6); 800G optical interface schematic (Section 7); and the Bill of Materials (Section 8).

Out of scope: the Matrix RTL itself (WP#30, unchanged here); PCB-level component placement and trace routing (WP#32); integration test planning (WP#33); the Dedicated Equipment's own internal schematic (WP#24 Figure 7-1's DEMUX/SEU/MUX ICs, which this document treats as a fixed input block rather than redesigning).

# 2. Positioning in the Document Sequence

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces, PCB layout (Dedicated Equipment card) | Complete |
| WP #25 | SEU1/SEU2 Synchronization Specification | SEU1/SEU2 record format, PTP profiles, completeness algorithms | Complete |
| WP #26 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, FPGA→ASIC migration | Complete — 12 open items |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete |
| WP #29 | Network Equipment SAD | O8/D12/I20 block architecture, placement, Core Stack negotiation, O8 operational architecture, NPU-Matrix execution model | Complete — v1.8 |
| WP #30 | Network Equipment RTL & Physical Design (Switching/Routing) | Matrix switching/routing RTL, register interfaces, semiconductor physical implementation | All sections drafted — v1.1 |
| WP #31 (this document) | Schematic & BOM | System block diagram, component selection, power/clock distribution, 800G interface, BOM | All sections drafted — v1.0 |
| WP #32 (proposed) | PCB Layout Design | 800G PAM4 board-level placement/routing, thermal design | Proposed |
| WP #33 (proposed) | Integration Test Plan | Node-to-node connectivity, fault recovery, performance testing | Proposed |

Table 2-1. Document Sequence — Updated at WP#31 (skeleton)

# 3. System Block Diagram

The node equipment schematic extends WP#24 Figure 7-1's Dedicated Equipment component block list by adding the node-level blocks defined in WP#29 and WP#30.

## 3.1 Existing Dedicated Equipment Blocks (from WP#24 Figure 7-1)

| Block | Source |
| --- | --- |
| DEMUX IC (Block A) | WP#24 Figure 7-1 |
| SEU Assembly Engine | WP#24 Figure 7-1 |
| WORM CTRL / HBM Gate ASIC | WP#24 Figure 7-1 |
| MUX IC (Block B) | WP#24 Figure 7-1 |
| Network Adapter / L4-L5 PHY | WP#24 Figure 7-1 |
| HBF ACL / 5-Axis Matrix | WP#24 Figure 7-1 |
| HBF Cube Write-Back (Block C) | WP#24 Figure 7-1 |
| M-AXIS Lock register | WP#24 Figure 7-1 |
| D12 Uplink / 800G Fabric PHY | WP#24 Figure 7-1 |
| Edge Header, CH1–CH16 pin block | WP#24 Figure 7-1 |
| CH11 Relay Island (isolated, no shared trace) | WP#24 Figure 7-1 |

Table 3-1. Existing Dedicated Equipment Blocks — reused from WP#24 Figure 7-1 (citation unverified — see OI-31-12)

## 3.2 Node-Level Additions (from WP#29/WP#30)

| Block | Function | Source |
| --- | --- | --- |
| Matrix Switch Fabric | Receives SerDes lanes from the 800G optical interface (§7) and performs switching/routing | WP#30 §3 |
| MONITOR_CH | Monitors link status (LOS/BER) and reports to the NPU | WP#30 §3.1 |
| NPU (Node Intelligence) | Executes rule-based decisions (self-healing, resource scaling) and configures the Matrix via MONITOR_CONFIG_REG | WP#29 §10.7.11 / WP#30 §6 |

Table 3-2. Node-Level Additions — confirmed per 창호, sourced from WP#29/WP#30

[상세 미정 — 차후 세션에서 확정] Signal flow between blocks (e.g. DEMUX → Matrix → MUX, MONITOR_CH → NPU → Matrix) and the exact connection topology are not yet drawn — depends on WP#30 OI-30-04 (RTL module breakdown).

## 3.3 Inter-Block Connectivity Rules

To preserve signal integrity and maintain architectural separation between real-time data switching and node management/control functions, the node equipment schematic distinguishes two physical connection classes:

| Class | Description |
| --- | --- |
| Data Path | High-speed differential pairs (SerDes lanes) originating from the 800G optical interface (§7) are mapped directly to the Matrix Switch Fabric inputs. These paths operate at 800G PAM4 rates and are routed according to high-speed PCB rules (controlled impedance, differential pairing, length matching). Carries user-facing PAI/EAI traffic and inter-node data. |
| Control Path | Monitoring and management interfaces — specifically MONITOR_CH (link status, LOS/BER, §3.1; realized as monitor_control_word[31:0] per WP#30 §3.3) and the NPU configuration interface (§6) — use a secondary lower-speed bus, physically and electrically separate from the high-speed data fabric. This prevents control-plane traffic from coupling onto the high-speed data paths. Typical control-path protocols (SPI, I2C, UART, or parallel register buses) run at orders of magnitude lower speed than the 800G data lanes and are routed on different PCB layers or separate connector pins where applicable. |

Table 3-3. Data Path vs. Control Path — confirmed per 창호

[상세 미정 — 차후 세션에서 확정] Exact control-bus topology (star vs. daisy-chain, pin assignments, voltage levels, specific interface protocol) depends on component selection (§4) and the NPU–Matrix command interface latency details (§6, WP#30 OI-30-16), both still open.

# 4. Component Selection

Selecting actual parts (manufacturer, part number, key specification) for the FPGA/ASIC implementing the Matrix switch fabric, optical transceivers for the 800G interface, memory, and power-supply ICs — following WP#23 §6's BOM pattern of naming a real component/specification/key-parameter/quantity per line, not placeholder categories.

Narrowed by WP#30 OI-30-05 (FPGA target narrowed to VU19P or newer, GTM-class transceiver requirement). Candidate parts below were verified against real third-party VU19P designs via web search this session, with confidence level noted per item.

| Subsystem | Candidate | Key Parameter | Confidence |
| --- | --- | --- | --- |
| Power (PMIC) | MPM3695-100 / MPM3695-25 / MPM3695-10 / MPM3632C / MPM3650 (MPS) | 16V input, scalable DC/DC modules; exact per-rail assignment now confirmed — see §5 Table 5-1 | Confirmed — MPS's official VU19P reference document fetched and verified this session (§5) |
| Memory | DDR4 SO-DIMM (multi-slot, per-FPGA) | Confirmed pattern: Aldec HES-VU19PD-ZU7EV (real VU19P board, 4× VU19P) uses "5x SODIMM external memory... 4x connected to VU19P devices" | Confirmed — real VU19P board precedent found via web search |
| 800G Interface — Bring-up/Verification Stage Only (QSFP-DD; not the production MOCT-native interface, see OI-31-11) | QSFP-DD | Confirmed pattern: same Aldec HES-VU19PD-ZU7EV board description explicitly lists "QSFP-DD, FMC, DDR4, PCIe, NVMe & Eth" | Confirmed — same real VU19P board precedent |
| Host/Control Interface | PCIe (Gen3/Gen4, native to UltraScale+ hard blocks) | Same Aldec board; also consistent with WP#26 §7.2's FPGA hard-IP transceiver pattern | Confirmed — same real VU19P board precedent |

Table 4-1. Component Selection Candidates — verified against real VU19P designs (web search, this session)

Open — OI-31-02: (Narrowed) Candidate parts confirmed for power, memory, and interface subsystems (Table 4-1); PMIC rail assignment now fully confirmed at §5 Table 5-1 against MPS's official VU19P document. Remaining: exact part quantities for memory (capacity/ECC configuration) and ASIC-stage parts (depends on WP#30 OI-30-17) — this is a verified candidate shortlist, not a final BOM.

# 5. Power Distribution Network

Voltage rails, current capacity, and distribution topology, following WP#29 §10.2's split between always-on control/negotiation power and per-instance SEU1/SEU2 power domains (WP#26 Table 5-2's independent-power-domain pattern).

## 5.1 Voltage Rails (VU19P, module-based solution)

Confirmed against MPS's official VU19P/VU27P/VU29P/VU45P/VU47P power reference document (module-based "smallest size" solution), fetched and verified this session — not the earlier draft's rail assignment, which mismapped three of the eight rails.

| Rail | Voltage / Current | MPS Part (verified) |
| --- | --- | --- |
| VCCINT | 0.85V, 100–200A (scalable) | MPM3695-100 (2×) |
| VCCBRAM, VCCINT_IO | 0.85V, 12A | MPM3695-25 |
| VCCAUX, VCCAUX_IO, VCCADC | 1.8V, 7A | MPM3695-10 |
| MGTYAVCC (transceiver core) | 0.9V, 7A | MPM3695-10 |
| MGTYAVAUX (transceiver aux) | 1.8V, 0.6A | MPM3632C |
| MGTYAVTT (transceiver termination) | 1.2V, 16A | MPM3695-25 |
| VCCO [1..] HDIO, HPIO (I/O) | 1.1–3.3V, 3A | MPM3650 |
| DDR_VTT (termination) | VDD/2, 3A | MP20075 |

Table 5-1. VU19P Power Rails — verified against MPS official reference document (web-fetched, this session), module-based solution

## 5.1a PMIC Headroom vs. ASIC Power Estimate (New — reflects WP#30 §7.2)

Per 창호, following WP#30's resolution of OI-30-17 (16nm-class ASIC, Phase 1, WP#30 Table 7-1a): the VCCINT PMIC configuration above (MPM3695-100 ×2, up to 200A / 170W at 0.85V) was sized against the VU19P FPGA's full rail set (≈225W total, WP#30 §7.2). WP#30's order-of-magnitude ASIC power estimate (110–155W, not a synthesis-derived figure) falls within this existing PMIC's capacity — no PMIC upsizing is indicated by the current estimate.

Open — OI-31-09: The 110–155W ASIC estimate is order-of-magnitude only (WP#30 §7.2), not derived from RTL synthesis against a selected foundry/node. PMIC sizing here should be re-confirmed once WP#30's foundry selection (OI-30-17, narrowed but not fully closed) and actual synthesis power are available — this section states headroom against an estimate, not a final power budget.

## 5.2 PMBus Network — Data Path / Control Path Domain Split

Consistent with WP#31 §3.3's Data Path / Control Path separation, the PMBus-capable rails (those marked PMBus/I2C = yes in the MPS document: VCCINT, VCCBRAM/VCCINT_IO, VCCAUX family, MGTYAVCC, MGTYAVTT) split naturally along the same line as §3.3's signal classes, since the transceiver-facing and core-logic rails are exactly the ones feeding the Matrix switch fabric's data path:

| Domain | Rails | Rationale |
| --- | --- | --- |
| Domain A — Data Path Power | VCCINT, MGTYAVCC, MGTYAVTT (transceiver core + termination, directly feeding 800G SerDes and Matrix switch fabric core logic) | Highest-current rails (up to 200A), PMBus-monitorable; a fault here is a Data Path event by WP#31 §3.3's own definition |
| Domain B — Control Path Power | VCCBRAM/VCCINT_IO, VCCAUX family, VCCO (I/O), DDR_VTT | Lower-current rails serving control/management logic, MONITOR_CH, and I/O — remains accessible even if Domain A is faulted or isolated |

Table 5-2. PMBus Domain Split — Data Path vs. Control Path power, per 창호's §3.3 extension

This split ensures a PMBus fault or high-current transient on the Data Path (Domain A) does not compromise the NPU's ability to monitor Control Path (Domain B) power integrity — the same fault-isolation logic already established for signal routing in §3.3, extended to the power network.

Open — OI-31-08: Exact PMBus addressing, bus speed, and master controller assignment (FPGA-internal PMBus controller vs. external) are not yet fixed. Note also that MGTYAVAUX (MPM3632C) and VCCO/DDR_VTT (MPM3650/MP20075) are not PMBus-capable per the MPS document — these rails require a separate monitoring path if their status is needed by the NPU, which this document does not yet define.

# 6. Clock Distribution

The node equipment clocking architecture distinguishes between the external optical clock source received from the network (OIL clock tree) and the internal electrical distribution to node blocks (PTP clock distribution).

## 6.1 OIL Clock Tree (Node Input)

The OIL clock tree, sourced from the I20-rooted passive optical clock tree (WP#23), provides the master reference clock to the node equipment over the MOCT ribbon. This clock is a fixed input to the node — not designed or modified at the node level, but received and conditioned for internal use. The node's PLL/clock conditioner accepts this input and generates the local clocks for the PTP distribution network.

[상세 미정 — 차후 세션에서 확정] OIL-to-PTP interface parameters (frequency, amplitude, termination) depend on the specific optical transceiver and PLL selection (OI-31-02/OI-31-04).

## 6.2 PTP Clock Distribution (Node Internal)

The node forwards a PTP master reference to instance-facing traffic, consistent with WP#29 §10.2 and WP#25 Table A-4/B-4's per-SEU1/SEU2 PTP domains (Domain 4 for SEU1, Domain 5 for SEU2). This distribution is electrical and occurs within the node equipment: the PLL/clock buffer outputs drive the instance-facing slots (Slot 3/4 per WP#29 §10.7.3) and the Matrix switch fabric (§3.2).

[상세 미정 — 차후 세션에서 확정] Clock buffer selection, fan-out topology, and skew budgets depend on component selection (§4) and OI-31-04.

## 6.3 MONITOR_CH Clock Domain

The clock-domain relationship between MONITOR_CH and the Matrix switch control logic is not yet fixed — see WP#30 OI-30-12 (shared clock vs. CDC). This item is explicitly deferred to WP#30's resolution; WP#31 will reflect the outcome once OI-30-12 is closed.

[상세 미정 — 차후 세션에서 확정] PLL/oscillator selection, clock-tree topology, and specific buffer/divider part numbers for the PTP distribution network depend on OI-31-04 (still open).

# 7. 800G Optical Interface Schematic

The MOCT fixed ribbon (WP#03) connects to the node equipment through optical transceivers, which present electrical SerDes lanes to the FPGA/ASIC implementing the Matrix switch fabric (WP#30 §3).

## 7.1 Lane Grouping (from WP#26 §7.3)

Reused directly from WP#26 §7.3, which treats this grouping as an immutable input fixed at WP#24 §5.2 — not an RTL or schematic design choice at any layer:

| Configuration | Lane Grouping |
| --- | --- |
| PAI/SEU1 | 48 lanes/channel × 8 groups |
| EAI | 24 lanes/channel × 16 groups (spanning SEU1/E2E #1 and SEU2/E2E #2) |

Table 7-1. Lane Grouping — reused from WP#26 §7.3 / WP#24 §5.2, confirmed per 창호

## 7.2 Transceiver-to-FPGA Connection

The optical transceiver converts the MOCT ribbon's optical signals to electrical SerDes lanes. The FPGA/ASIC's SerDes blocks receive these lanes and feed them to the Matrix switch fabric (WP#30 §3).

[상세 미정 — 차후 세션에서 확정] Transceiver part selection and PCS/FEC placement (FPGA-side vs. ASIC-side) depend on WP#26 OI-07 (line-coding/FEC/skew-alignment strategy), still open — this document does not resolve OI-07, only notes the dependency.

# 8. Bill of Materials

Node-equipment-level parts list, following WP#23 Table 6-1's pattern (#, Assembly/Block, Component/Specification, Key Parameter, Qty) — adapted from WP#23's RU context to O8/D12/I20 node context. Populated from §4/§5's confirmed candidates; passive components (resistors, capacitors, crystals) are not itemized, since no schematic netlist exists yet to derive real values or counts from — listing them would mean inventing figures, which this document series does not do.

| # | Assembly/Block | Component/Specification | Key Parameter | Qty |
| --- | --- | --- | --- | --- |
| 1 | FPGA (Matrix switch fabric) | Xilinx/AMD UltraScale+ VU19P or newer GTM-class family (WP#30 OI-30-05) | GTM-class 58Gbps SerDes required for 800G PAM4 | 1 |
| 2 | PMIC — VCCINT | MPM3695-100 (MPS) | 0.85V, 100–200A scalable, PMBus | 2 (paralleled) |
| 3 | PMIC — VCCBRAM/VCCINT_IO | MPM3695-25 (MPS) | 0.85V, 12A, PMBus | 1 |
| 4 | PMIC — VCCAUX family / MGTYAVCC | MPM3695-10 (MPS) | 1.8V/0.9V, 7A, PMBus | 2 |
| 5 | PMIC — MGTYAVAUX | MPM3632C (MPS) | 1.8V, 0.6A, non-PMBus | 1 |
| 6 | PMIC — MGTYAVTT | MPM3695-25 (MPS) | 1.2V, 16A, PMBus | 1 |
| 7 | PMIC — VCCO (I/O) | MPM3650 (MPS) | 1.1–3.3V, 3A, non-PMBus | 1 |
| 8 | DDR termination regulator | MP20075 (MPS) | VDD/2, 3A, non-PMBus | 1 |
| 9 | Memory | DDR4 SO-DIMM | Capacity/ECC configuration not yet fixed — see Open Items | TBD |
| 10 | 800G optical interface — bring-up/verification stage only (QSFP-DD; production interface is MOCT-native, see OI-31-11) | QSFP-DD | Confirmed against real VU19P board precedent (§4) | Per §7 lane grouping — count TBD pending §3 block diagram |
| 11 | Host/control interface | PCIe (Gen3/Gen4, UltraScale+ hard IP) | Native to FPGA — no separate part | — |
| 12 | Matrix switch fabric ASIC (production stage) | 16nm-class node, Phase 1 (WP#30 Table 7-1a); specific foundry/part not yet selected | Node class decided; foundry selection still open (OI-30-17 narrowed) | TBD |

Table 8-1. Node Equipment Internal BOM — populated from §4/§5 confirmed candidates; items 9, 10 (count), and 12 remain open

Open — OI-31-06: (Narrowed) BOM structure and 11 of 12 line items are populated from confirmed candidates. Remaining gaps: memory capacity/ECC configuration (item 9); exact QSFP-DD lane count for the bring-up interface (item 10, see OI-31-11), which depends on WP#31 §3's block diagram signal-flow (OI-31-01, itself depending on WP#30 OI-30-04); and the ASIC-stage Matrix fabric part (item 12, depends on WP#30 OI-30-17). Passive components are explicitly out of scope until a schematic netlist exists. Rack/site-level assembly BOM (WP#23 Table 6-2's second sub-level) is not attempted here — node equipment does not yet have a confirmed rack/chassis design to enumerate (WP#29 OI-29-05, OI-29-07 still open).

# 9. Open Items Register

| ID | Section | Open Item | Status |
| --- | --- | --- | --- |
| OI-31-01 | 3 | Block list confirmed (Tables 3-1/3-2: existing + 3 node-level additions); signal-flow/connection topology not yet drawn — depends on WP#30 OI-30-04 | Partially resolved — narrowed |
| OI-31-02 | 4 | Candidate shortlist confirmed (Table 4-1: MPM3695 series PMIC, DDR4 SO-DIMM, QSFP-DD for bring-up only, PCIe, verified against real VU19P boards); final BOM quantities/rail mapping and ASIC-stage parts (OI-30-17) still open | Partially resolved — narrowed |
| OI-31-03 | 5 | Voltage rails and regulator parts confirmed against real MPS VU19P document (Table 5-1, 8 rails); board-level PSU topology (single vs. redundant feed) still depends on WP#29 OI-29-05 | Partially resolved — narrowed |
| OI-31-08 | 5.2 | PMBus addressing, bus speed, master controller assignment not fixed; non-PMBus rails (MGTYAVAUX/VCCO/DDR_VTT) need a separate monitoring path if NPU visibility is required | Open — new |
| OI-31-09 | 5.1a | PMIC headroom confirmed against an order-of-magnitude ASIC estimate (110-155W), not synthesis-derived; re-confirm once WP#30 foundry selection and real synthesis power are available | Open — new |
| OI-31-10 | 1 | Master/Standby BOM variant mechanism (separate BOM vs. populate/no-populate option) not decided — reflects WP#29 R10's Mode B requirement for Master/Standby nodes | Open — new |
| OI-31-04 | 6.1/6.2 | Clock architecture confirmed (OIL input → PTP internal distribution, Table structure per §6.1–6.2); PLL/buffer part selection, fan-out topology, skew budgets not drafted — depends on §4 | Partially resolved — narrowed |
| OI-31-05 | 7.2 | Lane grouping confirmed (Table 7-1); transceiver part selection and PCS/FEC placement remain open, blocked on WP#26 OI-07 | Partially resolved — narrowed |
| OI-31-06 | 8 | BOM populated (Table 8-1, 12 items); memory config, QSFP-DD bring-up lane count, and ASIC-stage part remain open | Partially resolved — narrowed |
| OI-31-07 | 3.3 | Control-bus topology, pin assignments, voltage levels, and specific protocol not fixed — depends on §4 (component selection) and WP#30 OI-30-16 | Open — new |
| OI-31-11 | 4, 8 | QSFP-DD (Table 4-1, Table 8-1 item 10) is a bring-up/verification-stage interface borrowed from the real VU19P reference board — it is not MOCT-native and cannot be, since MOCT is factory-fused with no field connectors (WP#03 §4.1–4.2, WP#24 §1.4). The production-stage MOCT-native PCB-level electrical/mechanical interface (pin/pad definition, ribbon termination at the board edge) is not specified anywhere in the series. Until it is, QSFP-DD stands in only for bring-up testing. | Open — new |
| OI-31-12 | 3 | Table 3-1 (10 blocks, incl. “Edge Header, CH1–CH16 pin block”) and the §1 Master/Standby paragraph cite “WP#24 Figure 7-1” as their source. Verified against the current WP#24 (Cognition Cube Physical Spec) on file: no Figure 7-1, no §7 PCB component block list, and no “Edge Header” term exist in that document — WP#24’s actual §7 is “Hardware Verification Strategy Cross-Reference,” and its §4.2 pin-level interface map (CH1–CH16 electrical sensor/actuator pins, not MOCT optical) already states those pins are internal PCB headers/pads, not user-facing connectors, consistent with MOCT’s connector-less philosophy. The citation is dangling — either WP#24 was restructured after WP#31 was drafted and the figure moved/was removed, or WP#31 invented the citation. Table 3-1’s block list itself is not necessarily wrong, but its stated source does not exist in the current document set. | Open — new |

Table 9-1. WP#31 Open Items — v1.2 (12 open items; OI-31-11 registers the QSFP-DD bring-up-vs-MOCT-native gap, OI-31-12 flags Table 3-1's dangling WP#24 Figure 7-1 citation)

# 10. Conclusion and Next Steps

This revision additionally reflects WP#29 R10's new Mode B requirement for Master/Standby nodes: §1 states that a Master/Standby-designated node's BOM omits the Dedicated Equipment (PAI/EAI) sub-assembly entirely, since that role requires Mode B (no PAI/EAI attached). The specific mechanism for this BOM variant is newly tracked as OI-31-10, not yet decided. This revision also corrects Table 4-1 and Table 8-1 item 10 to flag QSFP-DD explicitly as a bring-up/verification-stage interface only, not the production MOCT-native interface (OI-31-11), and flags Table 3-1's "WP#24 Figure 7-1" citation as unverifiable against the current WP#24 document (OI-31-12). Twelve open items remain in total; WP#31 remains a complete first draft, now consistent with both WP#30's latest state (16nm-class ASIC) and WP#29's latest state (Mode B Master/Standby rule).

Velsanet Architecture Group  —  Internal Technical Document (Skeleton Draft)

VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

WP#32 — PCB Layout Design

Board-Level Component Placement · High-Speed Trace Routing · Thermal Design

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Internal Draft — §3, §4, §5, §6, §7 drafted (§3/§6: Option A, this session); QSFP-DD flagged as bring-up-stage only per WP#31 OI-31-11 (OI-32-08) | v0.7 │ 2026 | Node equipment PCB-level design: board-level component placement, high-speed (800G PAM4) trace routing, Data Path / Control Path physical separation, thermal design — distinct from WP#31's schematic (component-connection) level and WP#30's die-level layout | Extends WP#24 §7/Figure 7-1 (Dedicated Equipment card PCB layout — different scale, not superseded); WP#31 (this document's schematic input); WP#30 §7 (die-level layout, different scale); WP#29 §10.3 (thermal design strategy); scope fixed at WP#29 R7 (Table 2-1/12-1) |

Republic of Korea | Velsanet Architecture Group

Status note

This document is a skeleton, per 창호's stated method (WP#30/WP#31 pattern): section scopes and open items fixed now; actual placement/routing/thermal detail filled in section by section in later sessions, grounded in prior documents rather than invented. No dimension, layer count, or trace-width value is invented in this draft.

# 1. Purpose and Scope

WP#32 is the PCB-level design document for Velsanet network equipment (O8/D12/I20 nodes), realizing WP#31's schematic (component selection and connections) as an actual board layout. It was proposed at WP#29 §12 (Table 12-1) and its scope was fixed at WP#29 R7: board-level component placement and trace routing — explicitly distinct from WP#30's die-level (semiconductor) layout, and from WP#24 §7/Figure 7-1's Dedicated Equipment card layout, which this document does not redesign but treats as a fixed sub-assembly to be placed on the node-equipment board.

New decision, per 창호 this session — Option A (shared FPGA/ASIC footprint): this document assumes the 16nm-class ASIC target (WP#30 Table 7-1a, Phase 1) and adopts, as a new PCB-level design decision, a single PCB footprint shared between the VU19P FPGA (prototype stage) and the future 16nm-class ASIC (production stage), pin-compatible so one board design supports both stages. This is a genuinely new decision made this session, not previously established — pin-level compatibility with an as-yet-unselected ASIC (foundry still open, WP#30 OI-30-17) is an assumption to be verified once a specific ASIC pinout exists, not a confirmed fact.

New, per 창호 this session — Master/Standby PCB variant: per WP#29 R10 (§10.7.10) and WP#31 §1's corresponding BOM-variant note (OI-31-10), a Master/Standby-designated node's PCB does not populate the Dedicated Equipment (PAI/EAI) sub-assembly connector/slot. Whether this is a fully separate board variant or a populate/no-populate option on one board layout follows WP#31 OI-31-10's still-open decision — this document does not resolve that mechanism independently.

Scope includes: board component placement (Section 3); high-speed (800G PAM4) trace routing rules (Section 4); Data Path / Control Path physical separation at the board level, realizing WP#31 §3.3's connectivity classes (Section 5); thermal design (Section 6); and layer stack-up (Section 7).

Out of scope: schematic-level component selection (WP#31, unchanged here); die-level semiconductor layout (WP#30 §7, unchanged here); the Dedicated Equipment card's own internal PCB layout (WP#24 §7/Figure 7-1, unchanged here — treated as a fixed input sub-assembly); integration test planning (WP#33).

# 2. Positioning in the Document Sequence

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces, Dedicated Equipment card PCB layout (Figure 7-1) | Complete |
| WP #26 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, FPGA→ASIC migration | Complete — 12 open items |
| WP #29 | Network Equipment SAD | O8/D12/I20 block architecture, placement, Core Stack negotiation, O8 operational architecture, NPU-Matrix execution model | Complete — v1.8 |
| WP #30 | Network Equipment RTL & Physical Design (Switching/Routing) | Matrix switching/routing RTL, register interfaces, semiconductor physical implementation (die-level) | All sections drafted — v1.1 |
| WP #31 | Schematic & BOM | System block diagram, component selection, power/clock distribution, 800G interface, BOM | In progress — v0.5, §3/§6/§7 drafted |
| WP #32 (this document) | PCB Layout Design | Board-level component placement, high-speed trace routing, thermal design | In progress — v0.5, §3/§4/§5/§6/§7 drafted |
| WP #33 (proposed) | Integration Test Plan | Node-to-node connectivity, fault recovery, performance testing | Proposed |

Table 2-1. Document Sequence — Updated at WP#32 (skeleton)

# 3. Board Component Placement

Physical placement of the components confirmed in WP#31 §4/§8 onto the node-equipment board, including where the Dedicated Equipment card(s) (WP#24 §7/Figure 7-1, fixed sub-assembly) sit relative to the Matrix Switch Fabric, MONITOR_CH, and NPU blocks (WP#31 §3.2).

## 3.1 Placement Strategy: Option A (Shared Footprint)

Per 창호's decision (§1), the PCB accepts either device:

| Device | Stage | Key Parameters |
| --- | --- | --- |
| VU19P FPGA | Prototype | GTM-class 58Gbps SerDes, 0.85V VCCINT, ≈225W total envelope (WP#31 §5.1a) |
| 16nm-class ASIC | Production | Pin-compatible footprint (this decision), estimated 110–155W (WP#30 §7.2), foundry/part TBD |

Table 3-0. Shared Footprint — Two Device Stages, One PCB

## 3.1.1 Pin Compatibility Requirements

For Option A to succeed, the ASIC pinout must be a superset or functional subset of the VU19P pinout at the PCB connection level: SerDes lanes (800G PAM4, QSFP-DD-facing) must share the same lane-to-pin mapping on both devices; power/ground pins (VCCINT, VCCAUX, MGTYAVCC/MGTYAVTT/AVAUX, VCCO, DDR_VTT — WP#31 Table 5-1) must map to the same or compatible positions; control interface pins (MONITOR_CH's monitor_control_word[31:0], PMBus, PCIe, configuration) must share pin assignments; and VU19P pins the ASIC does not implement must be left No-Connect, with PCB pads present-but-unpopulated or tied appropriately.

Open — OI-32-06: VU19P-to-ASIC pin compatibility is not yet verified against any specific ASIC pinout, since foundry/part selection remains open (WP#30 OI-30-17). This section assumes compatibility is achievable; verification is deferred until a specific ASIC pinout exists.

## 3.1.2 Placement Zones

| Zone | Components | Notes |
| --- | --- | --- |
| Zone A — FPGA/ASIC (core) | VU19P or 16nm ASIC (shared footprint) | Central placement for shortest SerDes trace lengths to QSFP-DD connectors |
| Zone B — PMIC | MPM3695-100 ×2, MPM3695-25, MPM3695-10, MPM3632C, MPM3650, MP20075 | Adjacent to core device to minimize IR drop on VCCINT (up to 200A). Layer assignment for high-current vias kept separate from Option A's shared-footprint signal lines (see note below) — this is the most common Option A layout fault, addressed proactively rather than left for later debug. |
| Zone C — Memory | DDR4 SO-DIMM ×4 (per VU19P precedent, WP#31 §4) | Near the FPGA/ASIC, following DDR4 layout rules (fly-by topology, VTT termination at far end) |
| Zone D — 800G Optical Interface (bring-up/verification stage only, see WP#31 OI-31-11) | QSFP-DD connectors (count TBD, pending WP#31 §3 signal flow, OI-31-01) | Placed along one board edge for optical cable access; production board will terminate the MOCT ribbon here instead once WP#31 OI-31-11 is resolved |
| Zone E — Host/Control | PCIe edge connector (Gen3/Gen4) | Opposite edge from optical connectors |

Table 3-1. Component Placement Zones — Option A, confirmed per 창호

Zone B layer-assignment note, per 창호: high-current rail vias must be assigned to layers separate from the shared footprint's inner signal lines. VCCINT (MPM3695-100 ×2, up to 100A/rail) power vias are placed on internal layers (e.g. Layer 3/4), while signal lines are assigned to outer or non-adjacent layers (e.g. Layer 1/6), preventing physical interference and electrical coupling. This pre-empts the most common Option A layout fault — a high-current via breaking a signal line's reference plane, or a signal line blocking a power via's current path.

[상세 미정 — 차후 세션에서 확정] Exact component coordinates, keep-out zones, and mechanical constraints (board outline, mounting holes) depend on the chassis/rack design (WP#29 OI-29-05, OI-29-07), still open.

# 4. High-Speed Trace Routing Rules

The 800G PAM4 Data Path (WP#31 §3.3) shall be routed according to high-speed PCB design rules to ensure signal integrity at 800Gbps/core (WP#26 §7.1).

Note: the values below are industry-standard conventions for 800G-class PAM4 SerDes, not yet confirmed as Velsanet-specific requirements. Presented as a reference starting point per 창호's instruction, tracked under OI-32-02.

## 4.1 Controlled Impedance

Differential pair impedance: targeted at 100Ω ± 10% (industry standard for 800G PAM4 SerDes). Single-ended impedance: targeted at 50Ω ± 10% for reference.

## 4.2 Differential Pair Spacing

Intra-pair spacing: determined by the chosen dielectric material and stack-up (§7). Inter-pair spacing: ≥3× the intra-pair spacing to minimize crosstalk.

## 4.3 Length Matching

Intra-pair length mismatch: ≤0.5ps (≈0.08mm in typical PCB materials) — an industry-standard guideline for 800G-class SerDes interfaces, not a Velsanet-specific requirement.

Inter-pair (lane-to-lane) skew: ≤5ps is a typical industry target for 800G PAM4 implementations, but it is not yet confirmed as a Velsanet requirement. Once OI-30-16 (Matrix switching latency budget, WP#30 §8.3) is resolved, the inter-pair skew target shall be re-evaluated against that budget — the 5ps value is a reference starting point, not a value derived from OI-30-16, which itself has no fixed figure yet.

Open — OI-32-02: Impedance targets (100Ω/50Ω), differential-pair spacing, and length-matching tolerances (0.5ps intra-pair, 5ps inter-pair) are industry-standard reference values, not yet confirmed as Velsanet-specific requirements by 창호. The inter-pair skew figure additionally requires re-evaluation once WP#30 OI-30-16 (Matrix switching latency budget) is resolved.

# 5. Data Path / Control Path Physical Separation

Board-level realization of WP#31 §3.3's Data Path vs. Control Path classes, reusing the isolation technique already established at the Dedicated Equipment card level (WP#24 Figure 7-1: CH11 "isolation slot — no shared trace") rather than inventing a new technique.

| Technique | Application to Data/Control Path | Precedent |
| --- | --- | --- |
| Isolation slot | A routed keep-out gap in the board substrate between the Data Path (800G PAM4) region and the Control Path (MONITOR_CH/NPU bus) region, preventing any shared copper or via | WP#24 Figure 7-1's CH11 Relay Island — same technique, different signal class |
| Layer separation | Data Path high-speed differential pairs routed on dedicated internal layers, separate from Control Path's lower-speed single-ended/bus signals, reducing the chance of layer-adjacency coupling | Standard multi-layer PCB practice; not previously specified in a Velsanet document, proposed here as a complement to the isolation slot |
| Guard traces / ground stitching | Grounded guard traces or via-stitched ground planes along the isolation slot boundary, to absorb any residual coupling | Standard EMI-mitigation practice; not previously specified in a Velsanet document, proposed here |

Table 5-1. Data/Control Path Separation Techniques — isolation slot per WP#24 precedent; layer separation and guard traces proposed as standard-practice complements

Consistent with WP#31 §3.3's rationale (signal integrity, fault isolation): the isolation slot prevents a Data Path fault or noise event from coupling onto the Control Path that carries MONITOR_CH's self-healing signaling (WP#29 §10.7.2) — the same logic that motivated CH11's isolation at the Dedicated Equipment level, applied here to the Matrix/NPU control path instead of a safety relay.

Open — OI-32-03: (Narrowed) The isolation-slot technique itself is confirmed (reused from WP#24 CH11 precedent). Layer separation and guard-trace/ground-stitching are proposed as standard-practice complements but not yet confirmed by 창호. Exact slot width, guard-trace spacing, and which specific layers carry which path remain undetermined.

# 6. Thermal Design

Board-level thermal design, following WP#29 §10.3's strategy (heat-pipe general diffusion + liquid cooling for high-performance optical cores/processors, per Velsanet Architecture Compendium §6.1), now quantified against WP#30 §7.2's power estimate and WP#31 §5.1a's PMIC data.

## 6.1 Power Dissipation Summary

| Component | Power Draw | Notes |
| --- | --- | --- |
| FPGA/ASIC (core device) | VU19P: ≈225W (FPGA envelope); ASIC: 110–155W (estimated, not synthesis-derived) | WP#30 §7.2 / WP#31 §5.1a |
| PMIC (all rails) | ≈10–15W (estimated, per MPS reference data) | DC/DC conversion losses at full load |
| Memory (DDR4 ×4) | ≈5–10W (estimated, per JEDEC reference) | Per SO-DIMM ~1.5–2.5W typical |
| 800G transceivers (4× QSFP-DD, bring-up stage only) | ≈10–15W total (estimated) | Per QSFP-DD module ~3–4W typical; production MOCT-native interface power TBD (WP#31 OI-31-11) |
| Total (FPGA stage) | ≈250–265W | Upper bound for thermal design |
| Total (ASIC stage) | ≈135–195W | Lower bound; PMIC overhead roughly constant |

Table 6-1. Power Dissipation Summary (order-of-magnitude estimates) — based on WP#30 §7.2 and WP#31 §5.1a, confirmed per 창호

## 6.2 Thermal Budget and Junction Temperature

Assuming T_ambient = 40°C (typical data center / E2E center environment, WP#27/WP#28 deployment contexts) and target T_junction ≤ 85°C for reliability margin, required package-to-ambient thermal resistance: FPGA stage (225W) → Θ_JA ≤ (85−40)/225 ≈ 0.20°C/W; ASIC stage (155W worst-case) → Θ_JA ≤ (85−40)/155 ≈ 0.29°C/W. Since bare-PCB natural convection typically provides only 10–20°C/W, active cooling is required in all cases.

## 6.3 Cooling Solution Candidates

| Approach | Θ_JA Achievable | Pros / Cons |
| --- | --- | --- |
| Aluminum heat-sink + forced convection | ≈2–5°C/W | Passive, silent; may be large/heavy; requires chassis airflow |
| Heat-sink + axial fan (active air cooling) | ≈0.5–1.5°C/W | Compact; added cost, noise, fan MTBF reliability concern |
| Vapor chamber / heat pipe + fan | ≈0.3–0.8°C/W | High performance; more expensive, complex assembly |
| Liquid cooling (cold plate) | ≈0.1–0.3°C/W | Highest performance; high cost/plumbing complexity — overkill for 225W |

Table 6-2. Cooling Solution Candidates — draft, not final

Given the power range (135–265W total system power), an active heat-sink + axial fan or vapor-chamber heat-sink + fan appears most appropriate for a standard 1U/2U chassis form factor. Liquid cooling is not recommended given the power level and the current program stage.

## 6.4 Thermal Interface Material (TIM)

A high-performance TIM (thermal grease, phase-change pad, or graphite sheet) between the FPGA/ASIC package and the heat-sink is required. Typical Θ_TIM ≈ 0.1–0.3°C/W depending on thickness and conductivity.

## 6.5 PMIC Thermal Considerations

The VCCINT PMIC (MPM3695-100 ×2, up to 100A each) dissipates approximately 5–8W total at full load. These devices require adequate copper pour for heat spreading, thermal vias to internal/backside copper layers, and airflow shared with the FPGA/ASIC cooling solution. Control-path PMIC rails (lower current, lower loss) do not require dedicated heat-sinks but should still have adequate PCB copper area for heat spreading per the MPS reference design.

PMIC thermal noise and sense-line isolation, per 창호: PMIC heat generates thermally induced noise that can affect VCCINT's remote-sense line DC accuracy. The copper pattern from PMIC output to the VCCINT sense line is routed thermally isolated from the PMIC's hot region: (1) the sense line routes around the PMIC body's heat zone, and (2) a thermal relief slot or thermally isolated copper island surrounds the PMIC so package temperature does not affect sense-line DC precision. This is necessary to maintain the high-current (200A) VCCINT rail's voltage precision (typically ±1–2%), and is applied as a general design principle given that Option A's two silicon options (VU19P vs. ASIC) may have different sense-line requirements.

Open — OI-32-04: Final cooling solution selection (heat-sink part number, fan specification, TIM selection) is not yet made — depends on final ASIC power (once synthesis-derived, WP#31 OI-31-09) and chassis mechanical design (WP#29 OI-29-05, OI-29-07).

# 7. Layer Stack-Up

PCB layer count and stack-up, sized to carry both the high-speed Data Path layers and the lower-speed Control Path layers (WP#31 §3.3) without cross-coupling.

## 7.1 Layer Count and Stripline Structure

A 16-layer stack-up is a reasonable starting point for 800G PAM4 designs. The critical structural strategy is to route high-speed signal layers as stripline — embedded between solid ground planes — rather than microstrip. This provides inherent shielding from external noise and physically reinforces the Data/Control Path separation defined in §5, preventing coupling into the MONITOR_CH/NPU control bus without relying solely on keep-out gaps.

Open — OI-32-05a: (Confirmation needed) Exact layer assignment (e.g. which layers are dedicated to 800G PAM4 vs. control vs. power) is pending component placement (§3) and will be drafted once OI-32-01 is resolved.

## 7.2 Dielectric Material and Loss Budget

Materials such as Megtron 6 or Megtron 7 provide the very low dissipation factor (Df) required for 800G PAM4 operation. When OI-32-05 is resolved, the material selection shall be accompanied by an insertion loss verification step — confirming that the material's loss-vs-frequency data meets the signal integrity requirements implied by §4.1's impedance targets, not only by material name.

Open — OI-32-05b: (Confirmation needed) Specific material grade and loss budget are pending confirmation as a Velsanet requirement (part of OI-32-05).

## 7.3 Copper Weight and IR Drop

The 1–2 oz copper weight proposed for power/ground layers is directly tied to OI-30-17 (die-level power estimate). Once the system's total power consumption and peak current draw are known, the selected copper weight must be validated through an IR drop simulation — confirming that voltage drop across the power delivery network remains within the regulator's tolerance margin.

Open — OI-32-05c: (Confirmation needed) Copper weight and IR drop budget depend on WP#30 OI-30-17 and will be finalized once that figure is available — not derived from it now, since OI-30-17 itself has no fixed value yet.

# 8. Open Items Register

| ID | Section | Open Item | Status |
| --- | --- | --- | --- |
| OI-32-01 | 3.1.2 | Placement zones fixed in principle (Table 3-1, Option A); exact coordinates, keep-out zones, mechanical constraints depend on chassis design (WP#29 OI-29-05/OI-29-07) | Partially resolved — narrowed |
| OI-32-02 | 4 | Industry-standard reference values noted (100Ω/50Ω impedance, 0.5ps/5ps length matching) but not confirmed as Velsanet requirements; inter-pair skew also pending WP#30 OI-30-16 | Partially resolved — narrowed |
| OI-32-03 | 5 | Isolation-slot technique confirmed (reused from WP#24 CH11); layer separation/guard traces proposed but unconfirmed; exact dimensions undetermined | Partially resolved — narrowed |
| OI-32-04 | 6.3/6.5 | Power dissipation estimated (Table 6-1) and Θ_JA targets fixed (§6.2); final cooling part number/fan spec/TIM selection depends on synthesis-derived ASIC power (WP#31 OI-31-09) and chassis design (WP#29 OI-29-05/07) | Partially resolved — narrowed |
| OI-32-05 | 7 | 16-layer/stripline, Megtron 6-7, and 1-2oz copper proposed as starting points; verification steps open — stripline layer assignment (pending OI-32-01), insertion loss vs. §4.1 impedance targets, IR drop vs. WP#31 OI-31-09 (ASIC power) | Partially resolved — narrowed |
| OI-32-06 | 3.1.1 | VU19P-to-ASIC pin compatibility (Option A's core assumption) not yet verified against any specific ASIC pinout — deferred until WP#30 foundry selection | Open — new |
| OI-32-07 | 1 | Master/Standby PCB variant mechanism (separate board vs. populate/no-populate) not decided — tied to WP#31 OI-31-10, reflects WP#29 R10 | Open — new |
| OI-32-08 | 2, 6.3 | Zone D and the power budget currently use QSFP-DD, which WP#31 OI-31-11 has flagged as a bring-up/verification-stage interface only — MOCT is factory-fused with no field connectors (WP#03 §4.1–4.2, WP#24 §1.4). The production-stage board edge, ribbon termination method, and power draw for a MOCT-native 800G interface are not defined; this layout will need rework once WP#31 OI-31-11 is resolved. | Open — new |

Table 8-1. WP#32 Open Items — v0.7 (8 open items; OI-32-08 new, registers the QSFP-DD bring-up-vs-MOCT-native gap carried over from WP#31 OI-31-11)

# 9. Conclusion and Next Steps

§3 (Board Component Placement) and §6 (Thermal Design) are now drafted, completing all seven sections of WP#32's first full pass. Both build on a new decision made this session, per 창호: Option A — a single PCB footprint shared between the VU19P FPGA (prototype) and the future 16nm-class ASIC (production), pin-compatible so one board design covers both stages. §3 fixes placement zones (Table 3-1) and a Zone B layer-assignment rule separating high-current VCCINT vias from shared-footprint signal lines, pre-empting the most common Option A layout fault. §6 quantifies power dissipation (Table 6-1: ≈250–265W FPGA stage, ≈135–195W ASIC stage) and thermal budget (Θ_JA ≤0.20–0.29°C/W), narrows cooling solution candidates (Table 6-2) toward active heat-sink+fan or vapor-chamber approaches, and adds a PMIC sense-line thermal-isolation principle to preserve VCCINT's ±1–2% voltage precision against PMIC-generated thermal noise — applied generally since Option A's two silicon options may have different sense-line requirements.

Option A's core assumption — VU19P/ASIC pin compatibility — is explicitly unverified against any real ASIC pinout and is tracked as new OI-32-06, since foundry selection (WP#30 OI-30-17) remains open. This revision additionally reflects WP#29 R10's Mode B requirement for Master/Standby nodes: §1 notes that a Master/Standby-designated node's PCB omits the Dedicated Equipment sub-assembly connector/slot, with the exact mechanism (separate board vs. populate/no-populate variant) tied to WP#31 OI-31-10 and newly tracked here as OI-32-07. This revision also flags Zone D and the transceiver power-budget line as bring-up/verification-stage QSFP-DD only, not the production MOCT-native interface, and registers that gap as OI-32-08 (carried over from WP#31 OI-31-11). Eight open items remain in total; WP#32 is now a complete first draft across all seven sections.

Velsanet Architecture Group  —  Internal Technical Document (Skeleton Draft)

VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

WP#33 — Integration Test Plan

Multi-Node Connectivity · Fault Recovery Integration · Performance · Cross-Document Acceptance

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Internal Draft — skeleton, section scopes and open items fixed | v1.2 │ 2026 | System-level acceptance testing of multiple assembled O8/D12/I20 nodes operating together — node-to-node connectivity, multi-node fault recovery, throughput/latency, and cross-document consistency — distinct from WP#30 §9's single-node RTL/FPGA/ASIC validation and WP#28's single-EAI field validation | Extends WP#29 §4 (hypercube topology), §10.7.2/§10.7.10 (self-healing, Master-Standby); WP#26 §7/§9 (800G, validation pattern); WP#28 Table 6-1 (field validation pattern); WP#30 §9 (single-node validation, not duplicated); WP#31/WP#32 (schematic/PCB, cross-checked not redesigned); scope fixed at WP#29 R7 (Table 2-1/12-1) |

Republic of Korea | Velsanet Architecture Group

Status note

This document is complete as a design specification (v1.2). Its scope was deliberately reworked earlier in drafting, per 창호, to avoid duplicating WP#30 §9's single-node RTL→FPGA→ASIC validation stages: WP#33 covers what only becomes testable once multiple assembled nodes exist — multi-node connectivity, multi-node self-healing/failover, system throughput, and consistency across WP#26/30/31/32. No test procedure, pass criterion, or numeric target is invented in this draft. All five remaining open items are Ready for Hardware — see Section 8.

| # | Change | Rationale |
| --- | --- | --- |
| R1-1 | Table 2-1 (Document Sequence) synced to current file versions: WP#29 v1.9→v1.10 (OI-29 terminology corrections), WP#30 v1.2→v1.3, WP#31 v0.8→v1.2 (this reference was already stale before this revision — the actual WP#31 file was v1.0, not v0.8), WP#32 “In progress §4/§5/§7 drafted”→v0.7 (§3–§7 drafted). | Per 창호 — cross-document version sync. |
| R1-2 | Document version updated to v1.1. | Reflects the scope of this revision. |

Table R-1. Revision Log — R1

| # | Change | Rationale |
| --- | --- | --- |
| R2-1 | Added §4.2 (MAC Auto-Control Fallback Test) and a new Table 6-1 row (MOCT-DWDM Alignment Coupler verification), reflecting WP#34 §3.4.3's MOCT-DWDM Alignment Coupler (MAC). Registered as OI-33-06/OI-33-07 (both Ready for Hardware). Per WP#34 OI-34-20's cross-document registration request. | Per 창호 — closes WP#34 OI-34-20. |
| R2-2 | Document version updated to v1.2. | Reflects the scope of this revision. |

Table R-2. Revision Log — R2

# Philosophy of Validation

WP#33 is the last bastion of the Velsanet network equipment design series (WP#24–32). Individual documents each guarantee consistency within their own scope, but whether that consistency survives in an actual assembled multi-node system can only be confirmed through this integration test.

This document is the final gate verifying that every requirement fixed at the design stage actually holds on physical hardware — catching system-level defects that no prior simulation or single-node validation could surface. The test items specified here are executable only once hardware is assembled, and their results are the final judgment of design completeness.

# 1. Purpose and Scope

WP#33 is the system-level integration and acceptance test plan for Velsanet network equipment, testing multiple assembled O8/D12/I20 nodes operating together as a network — not a single node's internal RTL/FPGA/ASIC validation (WP#30 §9, unchanged and not duplicated here) and not a single EAI unit's field validation (WP#28 Table 6-1, unchanged and not duplicated here).

Scope includes: multi-node hypercube connectivity testing (Section 3); multi-node fault recovery and self-healing integration testing, extending WP#29 §10.7.2/§10.7.10 beyond the single-node case WP#30 §9.4 covers (Section 4); system-level performance/throughput testing (Section 5); and cross-document acceptance — confirming WP#26/WP#30/WP#31/WP#32 are mutually consistent in an actual assembled unit, not just individually correct on paper (Section 6).

Out of scope: single-node RTL simulation, FPGA bring-up, and ASIC post-silicon validation (WP#30 §9, unchanged); single-EAI-unit field validation (WP#28 Table 6-1, unchanged); the Matrix RTL, schematic, and PCB designs themselves (WP#30/31/32, unchanged — this document tests them, does not redesign them).

# 2. Positioning in the Document Sequence

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #26 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, FPGA→ASIC migration | Complete — 12 open items |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation (single unit) | Complete |
| WP #29 | Network Equipment SAD | O8/D12/I20 block architecture, placement, Core Stack negotiation, O8 operational architecture, NPU-Matrix execution model | Complete — v1.10 |
| WP #30 | Network Equipment RTL & Physical Design (Switching/Routing) | Matrix switching/routing RTL, register interfaces, semiconductor physical implementation, single-node validation (§9) | All sections drafted — v1.3 |
| WP #31 | Schematic & BOM | System block diagram, component selection, power/clock distribution, 800G interface, BOM | All sections drafted — v1.2 |
| WP #32 | PCB Layout Design | Board-level component placement, high-speed trace routing, thermal design | All sections drafted — v0.7 |
| WP #33 (this document) | Integration Test Plan | Multi-node connectivity, multi-node fault recovery, system performance, cross-document acceptance | Complete — v1.2, 7 open items (Ready for Hardware) |

Table 2-1. Document Sequence — Updated at WP#33 (skeleton)

# 3. Multi-Node Connectivity Test

Verifying actual hypercube-conformant connectivity between assembled nodes against the topology fixed at WP#29 §4 and the deployment contexts established at WP#27/WP#28 (AI Data Center and E2E Center).

## 3.1 Node Combination and Mode Rules (New — reflects WP#29 R10)

Per 창호, connectivity tests must additionally confirm each node's Mode A/B status (WP#29 §10.7.4) is compatible with its role in the tested connection, per §10.7.10's new Master/Standby restriction (WP#29 R10 — Master/Standby nodes must be Mode B; Mode A remains valid for ordinary same-tier traffic).

| Node Combination | Connection Type | Mode Requirement |
| --- | --- | --- |
| O8 ↔ O8 | Same tier (city–city) | Either node may be Mode A (PAI/EAI attached) for ordinary traffic; a node acting as Master/Standby for this link must be Mode B |
| O8 ↔ D12 | Upward (city → region) | The O8 node, when serving as Master/Standby for this uplink, must be Mode B — not simply because it connects upward, but because the Master/Standby role itself requires Mode B (WP#29 §10.7.10) |
| D12 ↔ D12 | Same tier (region–region) | Either node may be Mode A (AAI attached) for ordinary traffic; Master/Standby role requires Mode B |
| D12 ↔ I20 | Upward (region → nation) | The D12 node, when serving as Master/Standby for this uplink, must be Mode B |
| I20 ↔ I20 | Same tier (nation–nation) | Either node may be Mode A (AsAI attached) for ordinary traffic; Master/Standby role requires Mode B |

Table 3-1. Node Combination and Mode A/B Requirements — confirmed per 창호, reflects WP#29 R10

[상세 미정 — 차후 세션에서 확정] Specific test node counts, topology configurations, and pass criteria (e.g. what constitutes verified hypercube-conformant connectivity at the physical layer) are not yet drafted — depends on an actual multi-node test bed, not yet defined at the architecture-record level.

# 4. Fault Recovery / Self-Healing Integration Test

Multi-node exercise of WP#29 §10.7.2 (self-healing, alternate-path activation) and §10.7.10 (Master-Standby active failover), which are inherently multi-node mechanisms — a hypercube-neighbor standby assuming a failed master's role cannot be verified on a single node in isolation, unlike WP#30 §9.4's single-node field validation of the underlying MONITOR_CH/NPU mechanism.

## 4.1 Mode B Verification for Master/Standby (New — reflects WP#29 R10)

Per 창호, this test explicitly verifies: (a) O8↔D12 uplinks — the O8 node designated Master or Standby for the uplink has no PAI/EAI attached (pure Mode B, no neuron/nervous-system structure present, per 창호's rationale); (b) D12↔I20 uplinks — the D12 node designated Master or Standby has no AAI attached; (c) same-tier connections (O8↔O8, D12↔D12, I20↔I20) — connection succeeds normally regardless of the connected nodes' Mode A/B status, since Mode A validity is not restricted for ordinary same-tier traffic, only for the Master/Standby role.

[상세 미정 — 차후 세션에서 확정] Fault-injection method for multi-node scenarios (e.g. how a link is actually severed between two assembled nodes in a test bed), recovery-time measurement procedure, and pass criteria are not yet drafted — this extends WP#30 OI-30-20 (single-node self-healing pass criteria, still open) to the multi-node case. Mode B verification method (e.g. querying MONITOR_CONFIG_REG's attachment-detection state, WP#29 §10.7.4.3) is also not yet drafted.

## 4.2 MAC Auto-Control Fallback Test (New — reflects WP#34 §3.4.3.5/OI-34-16/OI-34-20)

For D12/I20 units equipped with the MOCT-DWDM Alignment Coupler (MAC, WP#34 §3.4.3), this test verifies that a MAC auto-control failure (actuator fault, sensor disagreement, or non-convergent correction) correctly surfaces as a MONITOR_EVENT_REG event and triggers the existing MONITOR_CH → NPU → alternate-path/handover pipeline (WP#29 §10.7.11, WP#30 §3.1) — not a separate, untested escalation path, per WP#34's OI-34-16 correction that no new architecture is required here.

[상세 미정 — 차후 세션에서 확정] Fault-injection method for MAC auto-control failure (e.g. deliberately inducing actuator fault or sensor disagreement in a test bed), the specific MONITOR_EVENT_REG bit/ACTION value it should map to (still open per WP#34 OI-34-16), and pass criteria are not yet drafted. This extends §4.1's Mode B verification pattern to the MAC-specific case.

# 5. Performance / Throughput Test

Scope: system-level 800G throughput and latency measurement across multiple connected nodes, building on WP#26 §7's 800Gbps/core physical-layer contract and WP#30 §8's timing budgets — but measured end-to-end across an assembled multi-node path, not per-component as in WP#30 §8.

[상세 미정 — 차후 세션에서 확정] Test topology, traffic generation method, and throughput/latency pass criteria are not yet drafted — depends on WP#30 OI-30-16 (Matrix switching latency budget, still open), since system-level latency cannot be meaningfully bounded until the component-level budget is fixed.

# 6. Cross-Document Acceptance

Scope: confirming WP#26, WP#30, WP#31, and WP#32 are mutually consistent in an actual assembled unit, following WP#28 Table 6-1's field-validation-of-a-specification-guarantee pattern (that document confirmed Dual E2E isolation held in installed hardware, not just on paper; this section applies the same logic across four documents instead of one).

| Check | Documents | What It Confirms |
| --- | --- | --- |
| SEU1 RTL ↔ Matrix RTL interface | WP#26 §4 vs. WP#30 §4 | The SEU1 engine actually interfaces correctly with the Matrix switch fabric, per WP#30 §4's register mapping (Tables 4-1–4-4) |
| BOM ↔ PCB placement | WP#31 §8 vs. WP#32 §3 | Every part listed in the BOM is actually placeable per WP#32's board layout, once WP#32 §3 is resolved |
| Timing budget ↔ signal integrity | WP#30 §8 vs. WP#32 §4 | 800G PAM4 traces on the actual PCB meet WP#30's timing budget, not just WP#32's industry-standard routing targets (WP#32 OI-32-02) |
| Single-unit field validation ↔ multi-node scenario | WP#28 Table 6-1 vs. WP#33 §4 | WP#28's single-EAI Dual E2E isolation guarantee still holds when that unit is one of several nodes under multi-node fault conditions |
| MOCT-DWDM Alignment Coupler (MAC) verification | WP#34 §3.4.3 vs. WP#33 §4 | MAC alignment/insertion-loss at DWDM installation meets WP#34 Table 3-4c's targets in an actual assembled D12/I20 unit, and MAC auto-control fallback (WP#34 OI-34-16) correctly surfaces through the existing MONITOR_CH/NPU pipeline rather than only on paper |

Table 6-1. Cross-Document Acceptance Checks — confirmed per 창호

[상세 미정 — 차후 세션에서 확정] Specific test procedures and pass/fail criteria for each row of Table 6-1 are not yet drafted — this table fixes what must be checked, not yet how.

# 7. Open Items Register

| ID | Section | Open Item | Status |
| --- | --- | --- | --- |
| OI-33-01 | 3 | Multi-node test bed configuration, node counts, and connectivity pass criteria not defined | Ready for Hardware — structure fixed at §3; parameters confirmed once test bed exists |
| OI-33-02 | 4 | Multi-node fault-injection method, recovery-time measurement, and pass criteria not defined — extends WP#30 OI-30-20 to multi-node case | Ready for Hardware — fault scenarios fixed at §4.1; injection method/instrumentation is hardware-environment-dependent |
| OI-33-05 | 4.1 | Mode B verification method for Master/Standby nodes (e.g. querying MONITOR_CONFIG_REG attachment state) not defined — reflects WP#29 R10/OI-29-24 | Ready for Hardware — tied to WP#29 OI-29-24; verification method follows once hardware register access is defined |
| OI-33-03 | 5 | System-level throughput/latency test topology and pass criteria not defined — blocked on WP#30 OI-30-16 (component-level latency budget) | Blocked (Ready for Hardware) — drafting possible once both hardware assembly and WP#30 OI-30-16 resolve |
| OI-33-04 | 6 | Specific test procedures for each Cross-Document Acceptance check (Table 6-1) not yet drafted | Ready for Hardware — check items fixed at Table 6-1; specific procedures defined against an actual assembled unit |
| OI-33-06 | 6 | MAC verification test procedure (Table 6-1's new row) — how to measure MAC alignment/insertion-loss against WP#34 Table 3-4c's targets on an actual assembled D12/I20 unit — not yet drafted. | Ready for Hardware — check item fixed at Table 6-1; procedure defined against an actual assembled unit |
| OI-33-07 | 4.2 | MAC auto-control fallback fault-injection method, target MONITOR_EVENT_REG mapping (depends on WP#34 OI-34-16, still open), and pass criteria not yet drafted. | Ready for Hardware — scope fixed at §4.2; depends on WP#34 OI-34-16 resolving first |

Table 7-1. WP#33 Open Items — v1.2 (7 open items; OI-33-06/OI-33-07 new, reflect WP#34 §3.4.3 MAC integration)

# 8. Conclusion and Next Steps

§3.1 and §4.1 carry WP#29 R10's Mode B requirement for Master/Standby nodes: Table 3-1 fixes which node combinations require Mode B (Master/Standby role only) versus permitting Mode A freely (ordinary same-tier traffic), and §4.1 states the specific verification intent for O8↔D12 and D12↔I20 uplinks. This reworked scope (originally would have duplicated WP#30 §9's single-node validation) carries the multi-node-specific constraints that single-node testing structurally cannot verify.

This revision (v1.2) adds two items reflecting WP#34's MOCT-DWDM Alignment Coupler (MAC, §3.4.3): OI-33-06 registers a MAC verification procedure in Table 6-1 (Cross-Document Acceptance), and OI-33-07 adds a new §4.2 MAC auto-control fallback test, confirming a MAC failure correctly reuses the existing MONITOR_CH/NPU pipeline (WP#34 OI-34-16) rather than requiring separate validation of a new escalation path. Both are Ready for Hardware, consistent with this document's existing items. Seven open items remain in total, all Ready for Hardware — their resolution is contingent on the availability of an actual multi-node test bed, not on further architectural design work. These items constitute the preconditions for commencing integration testing, not gaps in the design documentation itself. WP#33 remains v1.2 (Complete) as the living integration-test document in the WP#24–34 design series, establishing the validation framework that will be executed once physical hardware is assembled. The document series' governing principle — that all specifications are finalized through staged physical hardware validation — is hereby reflected in WP#33's status as the last bastion of system-level consistency verification.

Velsanet Architecture Group  —  Internal Technical Document

VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

WP#34 — CPO Package & Optical Fusion Process Specification

Chip-Level MOCT–Matrix Bonding · MEMS Alignment Process · Optical Fusion Process · Supersedes Board-Edge Assumption in WP#31/WP#32

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Internal Draft — skeleton, section scopes and open items fixed; CPO chip-level bonding model confirmed per 창호, supersedes the board-edge QSFP-DD assumption implicit in WP#31 §7.2 / WP#32 Zone D; v0.3 added tier-specific ribbon reach/DWDM scaling; v0.4 added diagrams/front-panel/fusion-process detail/§6.1; v0.5 added §3.4.3 MAC and tier-dependent §3.4.1 termination; v0.6 added §3.4.3.5 slot-level auto-monitoring/control; v0.7 added durability/thermal/replaceability/WP#33 gap closure; v0.8 corrects OI-34-16 (MAC failure reuses existing MONITOR_CH/NPU pipeline, per 창호) | v0.8 │ 2026 | PCB/package-level realization of WP#03's MOCT architecture as CPO (Co-Packaged Optics): ASIC-to-MOCT chip-level bonding, MEMS alignment process, optical fusion (splicing) process — registers the gaps closing WP#31 OI-31-11 and WP#32 OI-32-08 | Extends WP#03 §6 (Mechanical & Optical Integration Design), §8 (Layered Matrix–Transceiver Physical Structure); WP#29 §4.6 Table 4-4 (hierarchical core scaling, R8 — tentative basis, see §2); registers against WP#31 OI-31-11, WP#32 OI-32-08 |

Republic of Korea | Velsanet Architecture Group

Status note

This document is a skeleton, per 창호's stated method (WP#30/31/32/33 pattern): section scopes and open items are fixed now; CPO package pin-map, MEMS alignment tolerances, and fusion-process parameters are filled in section by section in later sessions, grounded only in what WP#03/WP#29 actually specify. No dimension, tolerance, or process parameter is invented in this draft. This document also formally establishes that MOCT bonds directly to the semiconductor photonic matrix at chip level (CPO) rather than to a board-edge connector, per WP#03 §6.2/§8.3 and 창호's explicit confirmation this session — WP#31 §7.2 and WP#32 Zone D's board-edge QSFP-DD framing remains valid only as the bring-up/verification-stage interface already flagged at OI-31-11/OI-32-08, not as the eventual production interface.

# 1. Purpose and Scope

WP#34 translates WP#03's architecture-level MOCT definition into a physical package specification that an actual ASIC/FPGA can be fabricated and bonded with. Where WP#03 establishes the principle (fixed ribbon, MEMS alignment, no field connectors) and WP#29 §4.6 establishes the numerical face/layer/bundle scaling, WP#34 defines how those two are physically realized as a Co-Packaged Optics (CPO) structure: the MOCT transceiver and ribbon bonding directly onto the Matrix switch fabric's semiconductor package, rather than connecting to it via board-edge SerDes traces and a pluggable module.

In scope: CPO package architecture and core-to-matrix junction mapping (§3); MEMS alignment process (§4); optical fusion (splicing) process (§5); the relationship of this document to WP#31/WP#32's existing bring-up-stage QSFP-DD content (§6).

Out of scope: RTL/logic design (WP#30, unchanged); non-optical board-level PCB placement and routing (WP#32 Zones A/B/C/E, unchanged); BOM structure outside the 800G optical interface line item (WP#31, unchanged); FPGA/ASIC selection itself (WP#26/WP#30, unchanged).

# 2. Positioning in the Document Sequence

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #03 | MOCT Physical Spec | Architecture-level MOCT definition: fixed ribbon, MEMS alignment, factory fusion, no field connectors | v1.1 — unchanged, this document's basis |
| WP #24 | Cognition Cube Physical Spec | Register maps, electrical interfaces (§4); still states "O8 = 192 optical cores" (§5.2/§7.2) — not yet synced to WP#29 R8's 1,536 figure | Unchanged — see OI-34-01 |
| WP #29 | Network Equipment SAD | O8/D12/I20 hierarchical core scaling (§4.6 Table 4-4, R8): 1,536/9,216/61,440 total cores | v1.10 — unchanged, this document's numeric basis |
| WP #31 | Schematic & BOM | §7.2 describes a board-edge transceiver-to-FPGA SerDes connection; Table 4-1/8-1 item 10 use QSFP-DD, flagged bring-up-only at OI-31-11 | v1.2 — unchanged; see §6 of this document |
| WP #32 | PCB Layout Design | Zone D places QSFP-DD connectors as the 800G interface, flagged bring-up-only at OI-32-08 | v0.7 — unchanged; see §6 of this document |
| WP #34 (this document) | CPO Package & Optical Fusion Process Spec | Chip-level MOCT–Matrix bonding, MEMS alignment process, optical fusion process | Skeleton — v0.8, 20 open items |

Table 2-1. Document Sequence — WP#34 skeleton

# 3. CPO Package Definition

## 3.1 Package Architecture Overview (restates WP#03 §6.2/§8.3)

Per WP#03 §6.2 and §8.3, the MOCT transceiver does not present a discrete, board-mounted module connected to the ASIC/FPGA by board-edge SerDes traces. Instead, the polymer waveguide layer, the ultra-fine mechanical contact layer, and the semiconductor/MEMS alignment layer form a single bonded stack directly on the Matrix switch fabric's own semiconductor package — a Co-Packaged Optics (CPO) structure. The MEMS Alignment Interface (MAI) aligns the transceiver's polymer waveguide array with the fixed multi-core ribbon at the point of manufacture; because the waveguide, transceiver output, and ribbon cores form one planar integrated structure, no separate round-to-ribbon or board-edge conversion step exists.

This confirms the model 창호 identified this session: MOCT bonds to the Matrix at chip/package level (Option A from the prior discussion), not via a board-edge connector (Option B). WP#31 §7.2's "optical transceiver converts the MOCT ribbon's optical signals to electrical SerDes lanes" language describes the bring-up-stage QSFP-DD path only, not the production CPO path this document defines.

## 3.2 Core-to-Matrix Junction Mapping

Per 창호, MOCT's multi-layer/multi-bundle structure is numerically designed to match the Matrix's hierarchical core-scaling rule fixed at WP#29 §4.6 Table 4-4: O8 = 4 layers × 4 bundles (192 cores/face, 1,536 total across 8 faces); D12 = 8×8 (768 cores/face, 9,216 total across 12 faces); I20 = 16×16 (3,072 cores/face, 61,440 total across 20 faces). If factory-level MEMS alignment and optical fusion are achieved per §4–§5, the physical fit between MOCT and the Matrix at each tier is structurally guaranteed by this numeric correspondence — no additional adaptation layer is required between MOCT's face/layer/bundle structure and the Matrix's face/layer/bundle structure.

[상세 미정 — 차후 세션에서 확정] The exact package-level pin/pad/ball grid, die-to-package interconnect method, and how the 192/768/3,072 cores/face map onto discrete package I/O are not yet drafted — see OI-34-02. This depends on WP#30 §7.2.1's foundry qualification requirements (Table 7-1b) being met, not on a specific named foundry — WP#30 OI-30-17 was reframed this session from "select a foundry" to "define what any qualifying foundry must satisfy" (per 창호), so this package footprint remains open until a qualifying vendor is actually engaged, not until one is chosen from a shortlist.

***Open — OI-34-01:*** This document uses WP#29 R8's 1,536-total-core figure for O8 (Table 4-4) as its basis. WP#24 §5.2/§7.2 still states "O8 = 192 optical cores" and has not been synced to WP#29 R8 — this inconsistency was logged in WP#29 itself (cross-document note, §11) as pending 창호's decision on propagation. Per 창호's tentative direction this session, WP#34 proceeds on the 1,536 figure; if 창호 later decides differently, this document's §3.2 and any package pin-count derived from it will need revision.

## 3.3 Hierarchical Scaling: MOCT Ribbon Variants by Node Tier

Per 창호's confirmation this session: WP#29 §4.6 Table 4-4 fixes core count per tier, but physical realization must also scale by reach, not core count alone. Velsanet's optical physical layer is therefore a tier-specific family of factory-fused assemblies. Total bandwidth figures below are 800Gbps/core (WP#26 §7, physical transmission reference) × WP#29 Table 4-4's core counts — independently verified, not restated from an external source.

| Parameter | O8 | D12 | I20 |
| --- | --- | --- | --- |
| Reach (qualitative — see OI-34-06) | ≤ 10 km | tens to hundreds of km | hundreds to thousands of km |
| Faces | 8 | 12 | 20 |
| Cores/face | 192 | 768 | 3,072 |
| Total optical cores | 1,536 | 9,216 | 61,440 |
| Total bandwidth (800Gbps/core × cores) | 1.2288 Pbps (≈1.2 Pbps) | 7.3728 Pbps (≈7.4 Pbps) | 49.152 Pbps (≈49.2 Pbps) |
| Ribbon composition | Pure MOCT ribbon | MOCT + DWDM + amplifier (fused) | MOCT + DWDM + high-power amplifier + dispersion compensation (fused) |
| DWDM required | Not required | Required | Required (ultra-high-density) |
| Amplifier | None | EDFA/SOA (fused) | High-power EDFA + dispersion compensation |
| Manufacturing method | Direct CPO bonding | CPO + DWDM/amplifier ribbon fusion | CPO + DWDM/amplifier/compensation ribbon fusion |

Table 3-3a. Tier-Specific MOCT Ribbon Variants and Bandwidth Scale — bandwidth figures verified against WP#29 Table 4-4 × WP#26 §7; reach figures qualitative pending OI-34-06

O8 nodes, operating within the ≤10km reach, receive the pure MOCT ribbon — no DWDM, no amplifier, no dispersion compensation — consistent with WP#03's original passive-medium definition and keeping O8 cost/complexity minimal as the edge/access tier. D12 and I20 receive active-component "smart ribbon" variants, scaling in both core count and optical complexity with reach and aggregate capacity. This is a departure from WP#03 §9.6's characterization of MOCT as uniformly passive — see OI-34-09.

Visualized below: each tier's ribbon composition, from bare CPO bonding (O8) through progressively more integrated active-component smart ribbons (D12, I20).

[O8] 1.2 Pbps / <=10km
    +---------------------------------------+
    |  ASIC (Matrix)                        |
    |  `-- CPO Direct Bonding                |
    |      `-- MOCT Pure Ribbon              |
    |          (800G x 1,536 cores)          |
    +---------------------------------------+

        [D12] 7.4 Pbps / tens-to-hundreds of km
    +------------------------------------------------------------+
    |  ASIC (Matrix)                                              |
    |  `-- CPO Direct Bonding                                     |
    |      `-- MOCT Ribbon + [DWDM Mux/Demux] + [EDFA/SOA]        |
    |          `-- Factory-Fused "Smart Ribbon"                   |
    |              (800G x 9,216 cores -> 7.4 Pbps)               |
    +------------------------------------------------------------+

        [I20] 49.2 Pbps / hundreds-to-thousands of km
    +---------------------------------------------------------------------------+
    |  ASIC (Matrix)                                                            |
    |  `-- CPO Direct Bonding                                                   |
    |      `-- MOCT Ribbon + [DWDM Mux/Demux] + [High-Power EDFA] + [DCF]       |
    |          `-- Factory-Fused "Ultra Smart Ribbon"                           |
    |              (800G x 61,440 cores -> 49.2 Pbps)                          |
    +---------------------------------------------------------------------------+

Figure 3-3b. Tier-Specific Ribbon Composition — schematic, not to scale; restates Table 3-3a

## 3.4 MOCT-Compliant DWDM Interface Definition

Per 창호's direction: DWDM equipment for D12/I20 must not be connected to the MOCT ribbon through a conventional field connector (LC/SC) — its optical I/O must itself be redesigned to accept the MOCT ribbon by direct factory fusion, consistent with the no-field-connector principle applied everywhere else in the series (WP#03 §4.1–4.2).

## 3.4.1 Termination Specification

The DWDM equipment's optical input/output termination method is tier-dependent, per 창호's confirmation this session:

• O8 tier: terminates directly in a factory-fused MOCT ribbon connection — one ribbon per face (8 faces). This is consistent with WP#03's original passive-medium, no-field-connector principle and keeps O8 cost/complexity minimal.

• D12 and I20 tiers: terminate via the MAC (MOCT Alignment Coupler) defined in §3.4.3 — a factory-calibrated, field-insertable alignment and locking mechanism. This enables DWDM equipment replacement, upgrade, and maintenance without factory-level rework, which is required for regional/national-scale nodes where equipment lifecycle management is a practical concern.

The MAC is not a conventional field connector (e.g., MPO/LC) — it is a precision mechanical alignment fixture that maintains insertion loss within ≤0.1–0.2dB, approaching fusion-splice performance while allowing deliberate detachment when required.

| Aspect | Conventional DWDM | MOCT-Compliant DWDM |
| --- | --- | --- |
| Front panel | LC/SC connector array (tens of ports) | MOCT ribbon terminations (12–20 ribbons) |
| Connectors | Field-pluggable | None (factory-fused) |
| Alignment | Manual plug | Integrated MEMS stage |
| Maintenance | Connector cleaning/replacement | Ribbon not field-replaceable (factory-level only) |

Table 3-4b. Front-Panel Physical Difference — Conventional vs. MOCT-Compliant DWDM

## 3.4.2 MEMS Alignment Structure within DWDM Equipment

The same MEMS Alignment Interface (MAI) that aligns MOCT's polymer waveguide array to the ribbon (WP#03 §8.3) applies inside the DWDM equipment itself: the DWDM mux/demux optics must be factory-aligned to the incoming MOCT ribbon cores using the identical static MEMS alignment principle, not a separately engineered alignment mechanism. This is restated in §4 below.

| Parameter | Conventional DWDM | MOCT-Compliant DWDM (D12) | MOCT-Compliant DWDM (I20) |
| --- | --- | --- | --- |
| Throughput | ≈128 Tbps (approximate industry reference — see OI-34-07) | 7.4 Pbps | 49.2 Pbps |
| Optical I/O | LC/SC connectors | MOCT ribbon, direct fusion (12 faces) | MOCT ribbon, direct fusion (20 faces) |
| Alignment method | Manual, field | Factory MEMS alignment (MAI) | Factory MEMS alignment (MAI) |
| Core-to-wavelength mapping | None | 9,216 cores → DWDM wavelength mapping | 61,440 cores → DWDM wavelength mapping |
| Amplifier | Separate module | Ribbon-fused | Ribbon-fused |
| Dispersion compensation | Separate equipment | Optional | Ribbon-fused |
| Control interface | SNMP/NetConf | Channel 1 + NPU integration | Channel 1 + NPU integration |

Table 3-4a. MOCT-Compliant DWDM Equipment Specification (D12/I20) — throughput column for D12/I20 verified against Table 3-3a; conventional-DWDM baseline and derived scale multiples are approximate, see OI-34-07

***Open — OI-34-10:*** Core-to-wavelength mapping (9,216→wavelengths for D12, 61,440→wavelengths for I20) is stated as a requirement but not yet defined — depends on DWDM channel plan (channel count, spacing, C/L-band split), which is itself part of OI-34-08.

## 3.4.3 MOCT-DWDM Alignment Coupler (MAC)

The MOCT-to-DWDM connection for D12/I20 must satisfy three conditions simultaneously: (1) not factory fusion — DWDM equipment must remain replaceable/upgradeable; (2) not a conventional optical connector (e.g., MPO) — insertion loss must stay near fusion-level; (3) must not violate Velsanet's no-field-connector principle. The MOCT Alignment Coupler (MAC) is defined to satisfy all three.

3.4.3.1 Alignment Method

The MAC aligns MOCT ribbon cores to DWDM-internal waveguides using one or more of the following mechanical alignment means:

• V-groove alignment: ribbon fibers seat into V-shaped grooves for passive alignment

• Pin-hole alignment: mechanical keying between ribbon and DWDM equipment

• MEMS stage: ultra-precision MEMS actuation for active/passive alignment (optional)

3.4.3.2 Locking Method

Once aligned, the MAC performs physical locking by one or more of:

• Clamp: mechanical fastening that maintains alignment state

• Screw fastening: not permanent, but robust

• Magnetic latch: facilitates detachment

3.4.3.3 Insertion Loss Target

| Connection Method | Insertion Loss (Target) | Note |
| --- | --- | --- |
| Fusion | ≤ 0.05 dB | Permanent connection, highest reliability |
| MAC | ≤ 0.1–0.2 dB | Alignment-and-lock method, detachable |
| Conventional optical connector (MPO) | ≤ 0.5 dB | Field-detachable, higher insertion loss |

Table 3-4c. Insertion Loss Targets by Connection Method — targets stated, not yet verified; see OI-34-13

The MAC targets insertion loss approaching fusion-splice performance while retaining deliberate detachability.

MAC Durability (Lifecycle)

MAC durability — insertion/removal cycle count, insertion-loss drift over repeated cycles, and replacement interval — is a vendor manufacturing-capability question, appropriately handled as a requirement in Table 6-1a rather than specified in this document.

***Open — OI-34-17:*** MAC durability target (rated insertion/removal cycles, insertion-loss drift limit over that lifetime, and expected replacement interval) is not set. Velsanet must set the target; the DWDM vendor must demonstrate it can be met (Table 6-1a, extended).

3.4.3.4 MAC Operating Principle

The MAC operates in three stages:

• 1. Factory calibration: X/Y/Z alignment between the MOCT ribbon and DWDM-internal waveguide is performed at the factory; the alignment state is mechanically fixed/recorded within the MAC.

• 2. Field insertion: a field technician pushes the MOCT ribbon into the MAC's insertion port; the factory-calibrated alignment guide directs the ribbon to the correct position.

• 3. Field locking: once fully inserted, the ribbon is physically locked via clamp/screw/latch — connection is complete by "push and lock" alone, with no electrical or optical adjustment required.

Thermal Expansion Matching

Because factory calibration (step 1) is performed at a fixed reference temperature but field operation spans a wider range, differential thermal expansion between the MAC body, the MOCT ribbon, and the DWDM-internal waveguide can shift the calibrated alignment. Material CTE (coefficient of thermal expansion) matching, the rated operating temperature range, and any active temperature compensation are therefore part of the MAC design, not a detail that can be deferred to field adjustment (which the design explicitly excludes).

***Open — OI-34-18:*** Material CTE matching strategy, rated operating temperature range, and whether temperature compensation is passive (matched materials) or active (compensating mechanism) are undefined. This is primarily a vendor material-selection and mechanical-design question (Table 6-1a candidate addition), but Velsanet must specify the required operating temperature range as an input.

[상세 미정 — 차후 세션에서 확정] MAC mechanical specifications (dimensions, tolerances, materials) and the insertion loss targets above are not yet verified against an actual design — see OI-34-13. Long-term reliability (temperature cycling, vibration, repeated insertion/removal) is also undefined — see OI-34-14.

## 3.4.3.5 Slot-Level Auto-Monitoring and Control

The D12/I20-tier MAC must include the following automated alignment monitoring and control functions:

(1) Auto-Monitoring

• Measures each slot (face)'s optical core alignment state in real time or periodically

• Measured items: insertion loss, optical power, alignment error (X/Y/Z axes)

• Measurement method: optical feedback (power monitor, interferometry) or mechanical sensors

(2) Auto-Control

• Actively corrects alignment state based on measurement results

• Fine X/Y/Z adjustment via MEMS actuators or mechanical adjustment mechanisms

• Closed-loop control maintains alignment state continuously

(3) Slot-Level Independence

• Independent measurement and control channel per slot (face)

• D12: 12 slots; I20: 20 slots, each independently controllable

(4) Relationship to WP#03 §8.3

WP#03 §8.3 establishes static MEMS (no active control) as the general principle. D12/I20's auto-control here is a tier-based exception to that principle — extending the same exception already registered at OI-34-09 (§3.3's active-component smart ribbon), rather than introducing a new, separately-flagged conflict. O8 retains static MEMS per WP#03's original definition; only D12/I20 apply auto-control.

***Open — OI-34-15:*** Slot-level auto-monitoring and control mechanism is not yet defined — sensor type (optical feedback vs. mechanical), control algorithm, actuator response time/range, and closed-loop stability are all unresolved. This is a further specific instance of the WP#03 §8.3/§9.6 static-passive-medium exception already tracked at OI-34-09; that item's eventual WP#03 amendment should cover both §9.6 (passive medium) and §8.3 (static MEMS) for D12/I20.

(5) Control-Failure Behavior

Per 창호's correction this session: this does not require a new escalation architecture. WP#29 §10.7.11/§10.7.2 and WP#30 §3.1 already define the mechanism — MONITOR_CH detects a link-status condition (LOS/BER) and reports it to the NPU, which executes a pre-defined, deterministic response (alternate-path activation or Master-Standby handover, WP#30 Table 3-3) without NPU judgment being invented case-by-case. A MAC auto-control failure is simply a new fault source that should surface through this existing pipeline, not a reason to design a separate one.

***Open — OI-34-16:*** How a MAC auto-control failure maps onto MONITOR_CH's existing MONITOR_EVENT_REG (WP#30 Table 3-2b) is undefined — whether it reuses the existing LOS or BER_THRESHOLD bits, or requires a new bit/ACTION value (Table 3-3 currently has no MAC-specific case). This is a narrow register-mapping question, not a new architecture item — the detection-to-response pipeline itself (MONITOR_CH → NPU → alternate-path/handover) already exists and is reused as-is.

## 3.5 Tier Summary: Ribbon Composition and DWDM Integration

O8 — 1.2 Pbps / 1,536 cores / ≤10km reach: pure MOCT ribbon via direct CPO bonding; no DWDM required.

D12 — 7.4 Pbps / 9,216 cores / tens-to-hundreds-of-km reach: MOCT + DWDM + EDFA fused into a single smart-ribbon assembly; per 창호, this implies DWDM equipment must be newly designed at roughly 58× a conventional ≈128Tbps unit's throughput (approximate, see OI-34-07).

I20 — 49.2 Pbps / 61,440 cores / hundreds-to-thousands-of-km reach: MOCT + DWDM + high-power EDFA + dispersion compensation fused into a single smart-ribbon assembly; per 창호, this implies DWDM equipment must be redesigned at roughly 384× a conventional ≈128Tbps unit's throughput (approximate, see OI-34-07), i.e. an ultra-high-density redesign, not an incremental scale-up of an existing product.

# 4. MEMS Alignment Process

Per 창호's stated principle this session: if factory-level MEMS alignment and optical fusion are secured, the physical fit between MOCT and the multi-layer multi-plane Matrix is structurally already guaranteed by the numeric correspondence in §3.2 — the alignment process itself is therefore the load-bearing manufacturing step this document must define precisely.

## 4.1 Static MEMS Alignment Elements (per WP#03 §8.3)

WP#03 §8.3 establishes that the semiconductor-based photonic matrix combines with MEMS alignment structures using lithographic precision, high-density optical contact arrays, and static (non-actuated) MEMS elements that guarantee — rather than actively maintain — accurate core-to-matrix junctions once bonded.

[상세 미정 — 차후 세션에서 확정] Specific alignment tolerance budget (µm-level positional accuracy target), MEMS actuator/anchor part class or process (e.g. which lithography node, which bonding technique), and the verification/test method used to confirm alignment at manufacture are not yet drafted — see OI-34-03.

## 4.2 MEMS Alignment in DWDM Equipment (D12/I20 — New per §3.4.2)

The static MEMS alignment principle of §4.1 applies identically inside D12/I20's DWDM equipment: the DWDM mux/demux optics are factory-aligned to the incoming MOCT ribbon cores using the same MAI mechanism, not a separately engineered alignment process. This is stated as a requirement, not yet as a manufacturing procedure — see OI-34-03, which now also covers this DWDM-internal alignment case.

[DWDM Equipment Internal Optical Path]

        MOCT Ribbon (12/20 faces)
    |
    v
[MAI MEMS Alignment Stage]   <- aligns ribbon cores to DWDM-internal waveguides
    |
    v
[DWDM Mux/Demux]             <- wavelength separation/combination
    |
    v
[Amplifier (EDFA/SOA)]       <- signal amplification
    |
    v
[External Optical Network]

Figure 4-2a. DWDM-Internal Optical Path and MEMS Alignment Point (D12/I20)

Why MEMS alignment is required at this point:

— Each of the MOCT ribbon's cores (hundreds to thousands) must align to the DWDM-internal waveguide at sub-micron precision.

— Misalignment increases insertion loss, which can collapse the overall link budget.

— Once aligned at the factory, the junction is permanently fixed — no field re-alignment, consistent with WP#03's static-MEMS principle (§8.3).

# 5. Optical Fusion (Splicing) Process

## 5.1 Factory Fusion Principle (per WP#03 §4.2/§9.4)

WP#03 establishes that MOCT's fibers are pre-attached and fused at the factory as an inseparable extension of the transceiver (5m+ length), eliminating connector mating cycles, insertion-loss variation, and field-installation error by construction. This section is the process-level counterpart: it must define how that factory fusion is actually performed.

[상세 미정 — 차후 세션에서 확정] The specific fusion process (e.g. laser welding, thermal fusion splicing, or adhesive-free direct bonding), process temperature/time parameters, and post-fusion yield/inspection criteria are not yet drafted — see OI-34-04.

## 5.2 Smart-Ribbon Fusion Process — D12 (DWDM + EDFA)

For D12, the factory fusion process of §5.1 extends to cover the DWDM mux/demux and EDFA/SOA amplifier stage: these are fused into the ribbon assembly at manufacture as a single subassembly, per §3.3's "smart ribbon" principle and WP#03 §4.2/§9.4's no-field-installation mandate.

[상세 미정 — 차후 세션에서 확정] The specific process for fusing an active gain-medium component (EDFA/SOA) into the same factory assembly as the passive MOCT fiber — as opposed to the purely passive fusion of §5.1 — is not yet drafted, including whether it uses the same fusion technique or requires a distinct process step. See OI-34-04 (extended).

## 5.3 Smart-Ribbon Fusion Process — I20 (DWDM + High-Power EDFA + Dispersion Compensation)

For I20, the same fusion principle extends further to include high-power EDFA and dispersion compensation (DCF or electronic) in the same factory-fused assembly.

[상세 미정 — 차후 세션에서 확정] The specific fusion process for a three-component active assembly (DWDM + high-power amplifier + dispersion compensation, all fused with the passive MOCT ribbon) is not yet drafted. Whether DCF (a physical fiber-based component) can be factory-fused using the same technique as amplifier integration, or requires a distinct process, is unresolved. See OI-34-04 (extended).

## 5.4 Tier-Specific Fusion Process Comparison

| Process Step | O8 (Pure Ribbon) | D12 (Smart Ribbon) | I20 (Ultra Smart Ribbon) |
| --- | --- | --- | --- |
| Fusion targets | Fiber + PIC | Fiber + PIC + DWDM + EDFA | Fiber + PIC + DWDM + high-power EDFA + DCF |
| Number of fusion passes | 1 (see OI-34-12) | 2–3, sequential (see OI-34-12) | 3–4, sequential (see OI-34-12) |
| Alignment targets | Core–waveguide | Core–waveguide + DWDM–core + amplifier–waveguide | Core–waveguide + DWDM–core + amplifier–waveguide + DCF–waveguide |
| Test items | Insertion loss | Insertion loss + wavelength response + gain | Insertion loss + wavelength response + gain + dispersion compensation amount |

Table 5-1. Tier-Specific Fusion Process Comparison — process steps and targets are descriptive; exact pass counts are placeholders pending OI-34-12

***Open — OI-34-12:*** The specific number of sequential fusion passes (1 for O8; 2–3 for D12; 3–4 for I20) is a placeholder reflecting component count, not a confirmed process specification — actual pass count depends on whether multiple active components can be co-fused in a single step or require separate steps, which is unresolved (related to OI-34-04's open question on active-component fusion technique).

# 6. Relationship to WP#31/WP#32 (Supersession Note)

This document does not invalidate WP#31's Table 4-1/Table 8-1 item 10 or WP#32's Zone D — their QSFP-DD content remains correct as the bring-up/verification-stage interface for FPGA dev-board testing, per OI-31-11/OI-32-08. What this document supersedes is the implicit assumption, in WP#31 §7.2's "transceiver-to-FPGA" language and WP#32 Zone D's board-edge placement, that a board-edge connector is also the eventual production interface. Per §3.1, it is not — the production interface is chip-level CPO bonding.

***Open — OI-34-05:*** WP#31 §7.2 and WP#32 Zone D should eventually be revised to reference this document's CPO package instead of describing a board-edge connector as the production path, once §3–§5 of this document mature past skeleton stage. Per the addition-only revision principle, that revision is not made here — this item only registers the need.

## 6.1 MOCT-Compliant DWDM Vendor Qualification Requirements (Vendor-Agnostic)

Following the same philosophy as WP#30 §7.2.1: rather than naming a DWDM vendor, this section fixes what any vendor must be able to do to supply MOCT-compliant DWDM equipment. Whichever vendor meets every row below qualifies; none is pre-selected here.

| Requirement | Description |
| --- | --- |
| Direct ribbon fusion | Capability to design an optical I/O port that factory-fuses directly to a MOCT ribbon, not LC/SC connectors |
| Integrated MEMS alignment | Manufacturing capability to integrate the same MEMS alignment structure as MAI inside the DWDM unit |
| Silicon photonics PDK | Capability to design/fabricate DWDM-internal waveguides using the same SiPho PDK class as MOCT (WP#30 Table 7-1b) |
| Tier-scale support | Scalability to support both D12 (9,216 cores) and I20 (61,440 cores) configurations |
| Channel 1 integration | Software/firmware capability to integrate DWDM control with Velsanet NPU's Channel 1 control path |
| MAC design/manufacturing capability | Capability to design and manufacture the MOCT-DWDM Alignment Coupler (§3.4.3) — V-groove/pin-hole/MEMS alignment, clamp/screw/magnetic-latch locking mechanisms |
| Slot-level auto-monitoring and control | Capability to design/manufacture a system that automatically measures each slot (face)'s alignment state and actively corrects it via MEMS actuators or equivalent (§3.4.3.5) |
| MAC durability & thermal stability | Demonstrated insertion/removal cycle rating, insertion-loss drift limit over that lifetime, and CTE-matched or actively compensated thermal stability across the rated operating temperature range (OI-34-17, OI-34-18) |
| MAC module replaceability | Support for MAC-module-level replacement (as distinct from full DWDM chassis replacement) without requiring factory-level re-alignment, if Velsanet decides this is required (OI-34-19) |

Table 6-1a. MOCT-Compliant DWDM Vendor Qualification Requirements — vendor-agnostic, mirrors WP#30 Table 7-1b's approach

Whether the MAC itself should be a separately replaceable module (distinct from replacing the whole DWDM unit it terminates) is a Velsanet architecture decision, not a vendor capability question — it determines what "replaceable" means at this boundary, and whether MAC replacement requires re-running factory calibration (§3.4.3.4 step 1) or can rely on the coupler's own retained alignment.

***Open — OI-34-19:*** Whether the MAC is a separately replaceable module, and if so whether replacement requires factory-level re-calibration, is undecided. This is a Velsanet design decision (not vendor capability) — Table 6-1a's corresponding vendor requirement is contingent on this decision.

***Open — OI-34-11:*** No specific vendor — domestic or international, existing DWDM supplier or new entrant — has been identified as capable of developing MOCT-compliant DWDM equipment against Table 6-1a. This is a vendor-agnostic requirement list only, consistent with WP#30 OI-30-17's reframing; no shortlist exists.

## 6.2 Relationship to WP#33 (Integration Test Plan)

WP#33's cross-document acceptance framework (Table 6-1) does not currently include a MAC-specific check row — WP#33 was completed (v1.1) before this document's §3.4.3 existed. Per the addition-only revision principle, WP#33 is not modified here; this section only registers that a future WP#33 revision should add MAC alignment/insertion-loss verification, auto-control fallback behavior (§3.4.3.5(5)), and MAC durability testing to its cross-document acceptance checks and/or multi-node fault recovery test scope.

***Open — OI-34-20:*** WP#33's Table 6-1 (Cross-Document Acceptance Checks) and §4 (Fault Recovery/Self-Healing Integration Test) do not yet cover MAC-specific scenarios: alignment verification at DWDM installation, auto-control fallback triggering (OI-34-16), and durability/lifecycle testing (OI-34-17). This is WP#33's scope to add, not WP#34's — registered here for cross-document tracking only.

# 7. Open Items Register

| ID | Section | Open Item | Status |
| --- | --- | --- | --- |
| OI-34-01 | 3.2 | O8 face core-count basis (192, WP#24/WP#20/WP#25/Compendium vs. 1,536, WP#29 R8) not resolved across the series; this document tentatively uses 1,536 per 창호's direction this session | Open — new |
| OI-34-02 | 3.2 | CPO package pinout, ball/pad grid, and die-to-package interconnect method not yet defined — depends on a qualifying foundry being engaged against WP#30 §7.2.1's vendor-agnostic requirements (Table 7-1b), not on selecting among named candidates | Open — new |
| OI-34-03 | 4.1–4.2 | MEMS alignment tolerance budget, actuator/anchor part class or bonding process, and alignment verification/test method not yet defined; extended this revision to also cover the DWDM-internal MEMS alignment case (§4.2, D12/I20) | Open — new |
| OI-34-04 | 5.1–5.3 | Optical fusion process method, temperature/time parameters, and post-fusion yield/inspection criteria not yet defined for O8's pure-ribbon fusion (§5.1); extended this revision to cover D12's DWDM+EDFA smart-ribbon fusion (§5.2) and I20's DWDM+high-power-EDFA+dispersion-compensation smart-ribbon fusion (§5.3) — whether active-component fusion uses the same process as passive fiber fusion is itself unresolved | Open — new |
| OI-34-05 | 6 | WP#31 §7.2 and WP#32 Zone D still describe a board-edge model as if it were the production interface; revision to reference this document is registered but not made here (addition-only principle) | Open — new |
| OI-34-06 | 3.3 | Reach targets (O8 ≤10km, D12 tens-hundreds of km, I20 hundreds-thousands of km) are qualitative, not numerically fixed. A formal optical link-budget analysis (fiber attenuation, splice loss, amplifier gain/noise figure) is needed before exact km thresholds can be set. | Open — new |
| OI-34-07 | 3.4, 3.5 | The "≈128 Tbps" conventional-DWDM baseline (Table 3-4a) and the derived ≈58× (D12) / ≈384× (I20) redesign-scale figures (§3.5) are approximate industry reference values, not independently verified against a specific real DWDM product or citation. By contrast, the 1.2/7.4/49.2 Pbps totals themselves (Table 3-3a) are independently verified: 800Gbps/core (WP#26 §7) × WP#29 Table 4-4 core counts. | Open — new |
| OI-34-08 | 3.3, 3.4 | Amplifier/dispersion-compensation technology choice (EDFA vs. SOA, C-band vs. L-band, DCF vs. electronic dispersion compensation) is not yet selected — §3.3/§3.4 name the options without deciding among them. DWDM channel plan (count, spacing) is also undefined, which OI-34-10 depends on. | Open — new |
| OI-34-09 | 3.3, 3.4.3.5 | WP#03 §9.6 characterizes MOCT as a uniformly passive, "solid-state" optical medium. §3.3's tier-based active-component "smart ribbon" for D12/I20 departs from that characterization. A tier-based exception (O8 = pure passive per WP#03's original definition; D12/I20 = active-component variant per this document) has not yet been amended into WP#03 itself. Registered here per the addition-only principle; WP#03 is not modified by this document. | Open — new |
| OI-34-10 | 3.4.2 | Core-to-wavelength mapping (9,216 cores → DWDM wavelengths for D12; 61,440 cores → DWDM wavelengths for I20) is stated as a requirement but not yet defined — depends on the DWDM channel plan (OI-34-08). | Open — new |
| OI-34-11 | 6.1 | No specific vendor has been identified as capable of developing MOCT-compliant DWDM equipment against Table 6-1a's vendor-agnostic requirements; no shortlist exists, consistent with WP#30 OI-30-17's reframing. | Open — new |
| OI-34-12 | 5.4 | Number of sequential fusion passes for smart-ribbon manufacture (1 for O8; 2–3 for D12; 3–4 for I20, Table 5-1) is a placeholder reflecting component count, not a confirmed process specification. | Open — new |
| OI-34-13 | 3.4.3.3 | MAC mechanical specifications (dimensions, tolerances, materials) and insertion loss target (≤0.1–0.2dB) are stated but not yet verified against an actual design. | Open — new |
| OI-34-14 | 3.4.3.4 | MAC long-term reliability — alignment retention under temperature cycling, vibration, and repeated field insertion/removal — is undefined. | Open — new |
| OI-34-15 | 3.4.3.5 | Slot-level auto-monitoring and control mechanism (sensor type, control algorithm, actuator response) is undefined; extends the WP#03 §8.3/§9.6 exception already tracked at OI-34-09 to also cover static-MEMS for D12/I20. | Open — new |
| OI-34-16 | 3.4.3.5 | How a MAC auto-control failure maps onto MONITOR_CH's existing MONITOR_EVENT_REG (WP#30 Table 3-2b) is undefined — the detection-to-response pipeline (MONITOR_CH → NPU → alternate-path/handover, WP#29 §10.7.11, WP#30 §3.1) already exists and is reused as-is; only the fault-code mapping is open. | Open — new |
| OI-34-17 | 3.4.3.3 | MAC durability target (rated insertion/removal cycles, insertion-loss drift limit, replacement interval) is not set — Velsanet must set the target; vendor must demonstrate it. | Open — new |
| OI-34-18 | 3.4.3.4 | Material CTE matching strategy, rated operating temperature range, and passive vs. active temperature compensation for the MAC are undefined. | Open — new |
| OI-34-19 | 6.1 | Whether the MAC is a separately replaceable module (vs. full DWDM-unit replacement), and whether replacement requires factory re-calibration, is undecided — a Velsanet design decision, not a vendor capability question. | Open — new |
| OI-34-20 | 6.2 | WP#33's cross-document acceptance checks and fault-recovery test scope do not yet cover MAC-specific scenarios (alignment verification, auto-control fallback, durability testing) — WP#33's scope to add, not WP#34's; registered for cross-document tracking only. | Open — new |

Table 7-1. WP#34 Open Items — v0.8 (20 open items; OI-34-16 corrected this revision — MAC failures reuse the existing MONITOR_CH/NPU pipeline rather than needing a new escalation path)

# 8. Conclusion and Next Steps

WP#34 confirms, per 창호's explicit direction this session, that MOCT bonds to the Matrix switch fabric at chip/package level (CPO) rather than via a board-edge connector — resolving the architectural ambiguity flagged when WP#31 OI-31-11 and WP#32 OI-32-08 were first registered. This document fixes the numeric basis for that bonding (WP#29 §4.6's face/layer/bundle scaling, tentatively at the 1,536-core O8 figure pending OI-34-01) and scopes the three process-level definitions still required: CPO package pin/pad mapping (§3.2, OI-34-02), MEMS alignment process (§4, OI-34-03), and optical fusion process (§5, OI-34-04). No process parameter, tolerance, or pin count is invented in this draft — WP#34 is a skeleton, consistent with the WP#30/31/32/33 drafting pattern. This revision (v0.3) adds §3.3's tier-specific ribbon reach scaling (O8 pure ribbon; D12/I20 active-component "smart ribbon" with DWDM and amplification), §3.4's MOCT-compliant DWDM interface definition, and §5.2/§5.3's corresponding fusion-process extensions, all per 창호's direction this session. The 1.2/7.4/49.2 Pbps bandwidth totals underlying this scaling are independently verified against WP#29 Table 4-4 and WP#26's 800Gbps/core figure; the specific reach thresholds, DWDM technology choices, and conventional-DWDM baseline used for scale comparison are not — see OI-34-06 through OI-34-08. This revision also surfaces that WP#03 §9.6's passive-medium characterization now requires a tier-based exception, registered as OI-34-09 but not yet made in WP#03 itself. This revision (v0.4) adds visual diagrams for §3.3/§3.4's tier-specific ribbon composition and §4.2's DWDM-internal MEMS alignment path, a front-panel physical comparison (§3.4.1, Table 3-4b), a tier-specific fusion-process comparison (§5.4, Table 5-1 — process step counts flagged as placeholders, OI-34-12), and a new §6.1 defining vendor-agnostic MOCT-compliant DWDM supplier requirements, mirroring WP#30 §7.2.1's approach (no vendor identified, OI-34-11). This revision (v0.5) resolves last turn's O8/D12/I20 MOCT-ASIC bonding concern by scoping the alignment-based connection strictly to the MOCT-DWDM boundary (§3.4.3, MAC — MOCT Alignment Coupler), not to MOCT's chip-level CPO bonding itself, which remains factory-fused per §3.1/§3.2. §3.4.1 now states this tier-dependent termination explicitly: O8 stays factory-fused (no DWDM equipment exists at that tier); D12/I20 use the MAC, since regional/national DWDM equipment lifecycle management requires replaceability that pure fusion would prevent. Table 6-1a is extended with a MAC design/manufacturing requirement. This revision (v0.6) adds §3.4.3.5's slot-level auto-monitoring and control for D12/I20's MAC (closed-loop MEMS correction based on real-time insertion-loss/alignment-error measurement per slot) — an explicit extension of the WP#03 §8.3 (static MEMS) exception already tracked at OI-34-09, not a new unflagged conflict. Table 6-1a gains a corresponding auto-monitoring/control vendor requirement. This revision (v0.7) closes five identified gaps: MAC-failure fault-code mapping into the existing MONITOR_CH/NPU pipeline (§3.4.3.5(5), OI-34-16 — narrowed per 창호's correction: the detection-to-response mechanism already exists at WP#29 §10.7.11/WP#30 §3.1, no new escalation architecture needed); MAC durability/lifecycle target (§3.4.3.3, OI-34-17); MAC thermal expansion matching (§3.4.3.4, OI-34-18); MAC module replaceability as a Velsanet design decision, not a vendor question (§6.1, OI-34-19); and a new §6.2 registering WP#33's not-yet-updated cross-document acceptance/fault-recovery coverage for MAC scenarios (OI-34-20, WP#33's scope to resolve, not modified here). Twenty open items remain in total, to be resolved in later sessions grounded in WP#03/WP#29's existing specifications and, where a specific ASIC or DWDM vendor is required, WP#30's foundry qualification framework (§7.2.1).

Velsanet Architecture Group  —  Internal Technical Document