VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

Velsanet Hardware Design Series

Consolidated Volume: WP#24 – WP#28

Cognition Cube · SEU Synchronization · Dedicated Equipment · PAI / EAI Deployment

This volume reproduces the five Cognition Cube hardware design documents WP#24 through WP#28 in full and unmodified, in sequence. Per the Velsanet addition-only revision principle, no content of any constituent document has been altered, abridged, or re-ordered. Each document retains its own title block, classification, revision log, and open items register, and remains individually authoritative.

Republic of Korea  |  Velsanet Architecture Group

Contents

WP#24  Cognition Cube Physical Specification (HBM / HBF)

WP#25  SEU1 / SEU2 Synchronization Specification

WP#26  Dedicated Equipment FPGA / ASIC Design Guide

WP#27  PAI AI Data Center Deployment Guide

WP#28  EAI E2E Center Deployment Guide

VELSANET ARCHITECTURE GROUP | TECHNICAL DESIGN DOCUMENT

# WP#24 — Cognition Cube Physical Specification (HBM / HBF)

WORM Register Map · HBF ACL Bitfields · Transform Engine Registers · Electrical Interfaces · CH11 Functional Safety

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Public — Open Specification | v1.1 │ 2026 | HBM/HBF/Transform Engine register level | Extends FGEAI-I-011R1 Att2 |

Republic of Korea | Velsanet Architecture Group

Governance principle: This specification is published in full under the Velsanet open-specification policy. The mechanism that governs compliance and interoperability is certification against this register map — not confidentiality.

# 0. Purpose and Relationship to Att2

Att2 (Cognition Cube Hardware Design Specification) defines the WORM controller, the HBF 5-axis access matrix, and the Transform Engine at the functional-block level (Att2 §3.2, §4.1, §5). This document (WP#24) is the first of five follow-on documents proposed in Att2 §11.1, and takes these functional blocks down to the register, bitfield, timing, and electrical-interface level needed for independent hardware implementation and conformance testing.

As in Att2, the §1.3 methodology/implementation separation principle applies: register semantics (lock behavior, ACL encoding, audit-log format, safety state transitions) are the standardization-target layer, while specific silicon processes, vendor IP blocks, or connector part numbers are implementation-layer choices and out of scope.

# 1. WORM Register Map — HBM Raw Memory Layer

The WORM controller (Att2 §3.2) is exposed to the Transform Engine (Att2 §5) through per-channel register blocks. Each of the 16 EAI channels or 8 PAI channels owns a physically isolated, independent register set per Att2 §5.3.

Figure 1-1. WORM per-channel register pipeline

## 1.1 Per-Channel Register Block Layout

| Offset | Register | Width | Access |
| --- | --- | --- | --- |
| 0x00 | WORM_SECTOR_LOCK | 64-bit | RO (HW-set on first write) |
| 0x08 | WORM_CTRL | 32-bit | RO (hardwired at manufacture) |
| 0x0C | WORM_STATUS / ECC | 32-bit | RO |
| 0x10 | WORM_AUDIT_PTR | 32-bit | RO |
| 0x14 | WORM_CHAN_ID | 16-bit | RO (fixed per physical channel) |
| 0x20 | SEU_TIMESTAMP_LATCH | 80-bit | RO (T-Axis Timestamp Engine output) |
| 0x30 | T_AXIS_EPOCH_CTR (global) | 16-bit | RO — free-running epoch counter, increments on every 48-bit ns rollover (~78.9 hours) |

Table 1-1. WORM per-channel register block (0x30 is a single global register shared across all channels, not per-channel)

## 1.2 WORM_SECTOR_LOCK (0x00) — Bitfield Definition

The moment the first write to a sector completes, the WORM Logic Gate permanently sets bit 0, and no register interface can clear it.

| Bit | Field | Description |
| --- | --- | --- |
| [63:56] | RESERVED | Fixed 0x00 |
| [55:48] | CHAN_ID | Physical channel identifier (EAI 0–15, PAI 0–7) |
| [47:16] | SECTOR_BASE_ADDR | 32-bit byte address of the locked sector within the HBM partition |
| [15:8] | ECC_SYNDROME | ECC syndrome latched at lock time; 0x00 = clean write |
| [7] | VIOLATION_FLAG | Set to 1 if a write was attempted after lock; held until the audit log is read |
| [6] | ECC_ERROR | Set if an uncorrectable ECC error occurs on this sector |
| [5] | ECC_EN | Hardwired 1 |
| [4] | DMA_ONLY | Hardwired 1 — no CPU/OS register write path physically exists |
| [3] | WRITE_IN_PROGRESS | 1 while the first write is committing |
| [2:1] | RESERVED | Fixed 0b00 |
| [0] | LOCK | 0 = unwritten, exactly one write permitted. 1 = permanently locked |

Table 1-2. WORM_SECTOR_LOCK bitfield map

## 1.3 WORM_CTRL (0x08) — Hardwired Control Register

| Bit | Field | Description |
| --- | --- | --- |
| [31:4] | RESERVED | Fixed 0x0000000 |
| [3] | AUDIT_LOG_EN | Hardwired 1 |
| [2] | INTERRUPT_ON_VIOLATION | Hardwired 1 |
| [1] | CPU_ACCESS_BLOCK | Hardwired 1 |
| [0] | GLOBAL_ENABLE | Hardwired 1 |

Table 1-3. WORM_CTRL — every field is read-only by design

## 1.4 Audit-Log Entry Format

A 48-bit nanosecond field alone rolls over every 2^48ns ≈ 78.9 hours, which is inadequate for a legal-reference audit record (Att2 CH15, "legal record, traceability") that must remain unambiguous in time order across the entire operating life of the system. It must also be able to attribute not just the fact that a violation occurred, but which subject attempted the rejected write. The record below addresses both.

| Field | Width | Description |
| --- | --- | --- |
| EPOCH_ID | 16-bit | Taken from the global T_AXIS_EPOCH_CTR (Table 1-1, 0x30); increments every time TIMESTAMP_NS rolls over. 2^16 epochs × 78.9 hours ≈ an unambiguous range of roughly 900 years |
| TIMESTAMP_NS | 48-bit | Nanosecond timestamp within the current epoch, sourced from the T-Axis Timestamp Engine |
| REQUESTOR_ID | 16-bit | Identifier of the DMA-originating subject that produced the rejected write (a SEU Assembly Engine instance, or, if it did not arise from the normal assembly path, the DMA channel tag of the violation source) |
| CHAN_ID | 8-bit | Channel on which the violation occurred |
| ATTEMPTED_ADDR | 32-bit | Address targeted by the rejected write |
| VIOLATION_TYPE | 8-bit | 0x01 = rewrite of a locked sector · 0x02 = non-DMA access attempt · 0x03 = out-of-range address · 0x04 = REQUESTOR_ID could not be verified (unauthorized DMA source) |

Table 1-4. Audit-log entry — 128-bit fixed record; ring buffer, pointed to by WORM_AUDIT_PTR

## 1.5 Audit-Log Buffer Policy

Each audit-log ring buffer (the WORM per-channel log of §1.4 and the consolidated ACL log of §2.4, each) has a fixed depth of 1024 entries (128 bits per entry, 16KB total per buffer). WORM_AUDIT_PTR (Table 1-1, offset 0x10) is an index that cycles through the range 0–1023.

| Parameter | Specification |
| --- | --- |
| Buffer depth | Fixed at 1024 entries (16KB per buffer at 128 bits/entry) |
| Overflow warning threshold | AUDIT_LOG_STATUS.OVERFLOW_WARN is set at 896/1024 (87.5% full), giving a flush margin before data loss |
| Default overflow behavior | FIFO wraparound — if the buffer fills and no flush occurs, the oldest entry is overwritten |
| Overflow-event handling | The wraparound event itself sets AUDIT_LOG_STATUS.OVERFLOW_OCCURRED. This is a sticky single bit outside the ring (not itself subject to overwrite), so that even though the lost entry itself is unrecoverable, loss of audit history is always independently detectable |
| Recommended mitigation | Periodically or on threshold trigger, flush ring contents to CH15 (Audit/Evidence, WORM-dedicated long-term storage per Att2 Table 3-3) before OVERFLOW_OCCURRED is reached; AUTO_FLUSH_EN below supports this in hardware |

Table 1-5. Audit-log buffer size and overflow policy

| Register | Width | Description |
| --- | --- | --- |
| AUDIT_LOG_STATUS | 8-bit | Bit 0: OVERFLOW_WARN (≥87.5% full) · Bit 1: OVERFLOW_OCCURRED (sticky, latched, set regardless of ring contents) · Bit 2: AUTO_FLUSH_EN (hardwired or deployment-configured; when set, reaching the threshold alone triggers an automatic flush to CH15 with no software polling required) |

Table 1-6. AUDIT_LOG_STATUS register

# 2. HBF ACL Bitfields — Cube Memory Layer

HBF access control implements the 5-axis permission matrix defined in Att2 §4.1. Each cube entry has one 8-bit ACL_PERM register per axis, plus a Layer Lock register for the M-Axis.

## 2.1 ACL_PERM Register — Per-Axis Permission Encoding

| Bit | Role | 00 | 01 / 10 / 11 | Notes |
| --- | --- | --- | --- | --- |
| [1:0] | PAI | None | R / W / RW | Least-significant field |
| [3:2] | AAI | None | R / W / RW |  |
| [5:4] | AsAI | None | R / W / RW |  |
| [7:6] | RESERVED | — | — | Fixed 0b00 |

Table 2-1. ACL_PERM bit layout

| Axis | PAI | AAI | AsAI | ACL_PERM |
| --- | --- | --- | --- | --- |
| T (Time) | Read (01) | Read (01) | Read (01) | 0x15 |
| C (Context) | RW (11) | Read (01) | Read (01) | 0x17 |
| I (Intent) | RW (11) | RW (11) | Read (01) | 0x1F |
| E (Emotion) | RW (11) | Read (01) | Read (01) | 0x17 |
| M (Meta) | Read (01) | RW (11) | RW (11) | 0x3D |

Table 2-2. ACL_PERM values resolved from Att2 Table 4-1

This table specifies register-level permissions only — which role may read or write which axis — and does not indicate the order or direction of interaction between roles. PAI, AAI, and AsAI are not independent parallel actors each accessing the cube on their own; they sit on a defined E2E pipeline (PAI → AAI per Att2 §9.1; AsAI → HBF via Dedicated Equipment Block C per Att2 §5.1). Figure 2-1 below shows the permission grid only and does not include pipeline order — see Att2 §9.1, §5.1 for order.

Figure 2-1. HBF ACL_PERM grid — permission encoding only, not a flow diagram

## 2.2 T-Axis — Physical Override

T-Axis write protection is not mediated by the ACL — it is enforced by the WORM hardware anchor (Section 1) at a layer below the ACL gate, so a misconfigured permission register cannot make the T-Axis writable.

## 2.3 M-Axis — Layer Lock Register

| Bit | Field | Description |
| --- | --- | --- |
| [15:8] | LOCK_OWNER_ID | ID of the AAI or AsAI instance currently holding the write lock; 0x00 = no lock |
| [7:4] | RESERVED | Fixed 0b0000 |
| [3] | ASAI_LOCK_HELD | 1 while an AsAI instance holds the write lock |
| [2] | AAI_LOCK_HELD | 1 while an AAI instance holds the write lock |
| [1] | WRITE_PENDING | 1 while a write is committing |
| [0] | LOCK_ACTIVE | 1 while any subject holds the lock |

Table 2-3. M-Axis Layer Lock register

## 2.4 ACL Violation Log

The ACL violation log uses the same 128-bit format as §1.4, so that a single unified audit schema applies across the WORM/ACL subsystem as a whole.

EPOCH_ID[16] : TIMESTAMP_NS[48] : REQUESTOR_ID[16] : AXIS_ID[4] : REQUESTED_OP[4] : RESULT[8] : RESERVED[32]. AXIS_ID convention: T=0, C=1, I=2, E=3, M=4. REQUESTOR_ID identifies the PAI/AAI/AsAI instance that attempted the rejected access.

# 3. Transform Engine Register Map

The DEMUX Engine (Block A), SEU Assembly Engine, and MUX Engine (Block B) sit directly between the optical input and the WORM/HBF registers defined in Sections 1–2. Their register-level definitions are specified below.

Figure 3-1. Transform Engine register pipeline — DEMUX → SEU Assembly → MUX

## 3.1 Block A — DEMUX Engine Registers (Att2 §5.2)

| Register | Width | Description |
| --- | --- | --- |
| DEMUX_LANE_CFG | 16-bit | Lane-group mode: 0x0 = PAI (48 lanes/ch × 8 groups), 0x1 = EAI (24 lanes/ch × 16 groups) |
| DEMUX_CLK_PLL_STATUS | 16-bit | Per-channel PLL lock bitmap (bit N = channel N lock status); the per-channel independent clock domains of Att2 §5.2 |
| DEMUX_FAULT_EXCLUDE | 384-bit (48×8 words) | Bitmap of faulty cores auto-excluded from the 192–384 input cores |
| DEMUX_LATENCY_MON | 16-bit | Measured processing latency (picoseconds); target <2ns is hardwired (Att2 §5.2) |
| DEMUX_NONCOLLISION_CHK | 1-bit | Set when power-on self-test (POST) hardware-verifies Core(ri)∩Core(rj)=∅ for every channel pair |

Table 3-1. DEMUX Engine (Block A) register summary

## 3.2 SEU Assembly Engine Registers (Att2 §6.1 Stage 4)

| Register | Width | Description |
| --- | --- | --- |
| SEU_WINDOW_CTRL | 16-bit | [15:8] CURRENT_WINDOW_MS (800–1500) · [1] ADAPTIVE_MODE_EN · [0] CH4_RAPID_TRIGGER — implements the Att2 §6.2 adaptive window (default 1.0s, 0.8s on CH4 rapid change, 1.5s on low change rate) |
| SEU_CHANNEL_COMPLETE | 16-bit | Per-channel completeness bitmap (16 bits used for EAI, 8 for PAI); bit=1 when that channel's slot data has been fully received |
| SEU_MISSING_FLAG | 16-bit | Set per-channel when no data has arrived by the slot boundary (Att2 §6.1: "null + MISSING_FLAG") |
| SEU_ASSEMBLY_STATUS | 8-bit | 0x0 COLLECTING · 0x1 FLUSHING · 0x2 COMPLETE · 0x3 ABORTED (a required channel is missing) |
| SEU_CTRL_PLANE_WINDOW | 16-bit | EAI-only; an independent window register for the CH9–CH16 control plane, operating independently of the cognition-plane window (Att2 §6.2) |

Table 3-2. SEU Assembly Engine register summary

## 3.3 Block B — MUX Engine Registers

| Register | Width | Description |
| --- | --- | --- |
| MUX_CH_MAP | 128-bit | Mapping table from 8-channel (PAI) or 16-channel (EAI) cognition-bus lanes to 24-core Bundle lane assignments |
| MUX_BUNDLE_STATUS | 8-bit | Bit 0: BUNDLE_READY · Bit 1: PARTIAL_BUNDLE (fewer than 24 cores active) · Bit 2: OVERFLOW — this register's BUNDLE_READY state feeds the Network Adapter LINK_STATUS (§6.2) |

Table 3-3. MUX Engine (Block B) register summary

# 4. Electrical Interfaces

This section specifies, for every channel interface enumerated in Att2 Tables 3-3/3-4, the pin-level functional map, timing parameters, and voltage tolerances needed to implement and certify this Physical Spec.

## 4.1 L1 → L2 Optical-to-Electrical Conversion

| Parameter | Specification | Notes |
| --- | --- | --- |
| Optical modulation | PAM4, 800G per core (Att2 §1.5) | Consistent with the commercial 800G pluggable technology referenced in Att2 |
| O/E conversion | Photodiode array per core → TIA → ADC | One conversion chain per fixed ribbon core |
| O/E backend electrical domain | Differential, target ≤ 0.4 Vpp | Feeds directly into the per-channel WORM DMA write path |

Table 4-1. Optical-to-electrical conversion parameters

## 4.2 Pin-Level Interface Map

The pin counts below are internal PCB header/pad counts between the sensor front-end and each channel's receive buffer (Att2 §3.2, "independent per-channel receive buffer"), not user-facing connectors — consistent with the connectorless MOCT philosophy (Att2 §1.4) applied across the design.

| Channel | Interface | Pin Function List | Pin Count |
| --- | --- | --- | --- |
| CH1 Video | MIPI CSI-2 | CLK+/CLK−, D0+/D0−…D3+/D3− (4-lane) | 10 |
| CH2 Audio | I2S/PDM | BCLK, LRCLK/WS, SDATA, MCLK (optional) | 3–4 |
| CH3 Spatiotemporal | UART + SPI | UART: TX,RX · SPI: SCLK,MOSI,MISO,CS | 2 + 4 |
| CH4 Biosignal | SPI/I2C → Secure Enclave | SDA,SCL or SCLK,MOSI,MISO,CS | 2 or 4 |
| CH5 Text/Language | PCIe Gen4 x1 | TX+/TX−, RX+/RX−, REFCLK+/REFCLK− (optional) | 4 (+2) |
| CH6 Activity/Motion | SPI + CAN | SPI for IMU (4) + CANH,CANL (2) | 4 + 2 |
| CH7 Device Interface | CAN FD | CANH, CANL | 2 |
| CH8 Environmental | I2C/ADC | SDA,SCL or single-ended analog + GND | 2 or 1+1 |
| EAI CH9 Command Input | Dedicated encrypted SPI | SCLK,MOSI,MISO,CS,IRQ | 5 |
| EAI CH10 Actuation Output | RT-PWM | PWM_OUT, PWM_DIR, GND | 2–3 |
| EAI CH11 Safety Override | Hardwired relay | NO, COM (dry contact, per relay circuit) | 2 |
| EAI CH12 Authentication/Identity | Vendor HSM (signing bus) | Vendor-specific, typically 4-pin SPI + signing protocol | 4 |

Table 4-2. Pin-level interface map — CH13–CH16 are logical/storage channels with no separate pinout at the analog front-end; their PHY is specified concretely below rather than as a generic "inherits host interconnect" statement

| Channel | PHY | Notes |
| --- | --- | --- |
| EAI CH13 Mission State | PCIe Gen4 NVMe | Reuses the PCIe Gen4 electrical spec of CH5 (Table 4-5) on a separate controller/lanes; does not share a bus with CH5 |
| EAI CH14 AAI Coordination | No dedicated PHY — logical delivery only | Not a physical channel: CH14 is realized as inter-cube traffic through the Network Adapter/Cube Frame multiplexing defined in Section 6, using idle channel capacity within the E2E parallel structure (Att2 §8.1–§8.2) — not a fixed point-to-point link. See §6.4. |
| EAI CH15 Audit/Evidence | No external PHY — internal only | Directly backed by the WORM_SECTOR_LOCK infrastructure (Section 1); this channel is a register-level construct, not a sensor-facing electrical interface |
| EAI CH16 Redundant Link | Inherits the PHY of the channel it backs up | Hot-standby redundancy activated on fault, per the CH16 definition in Att2 Table 3-3; has no fixed PHY of its own |

Table 4-3. CH13–CH16 PHY specification

## 4.3 Timing Parameters

| Interface | Max Speed | Setup Time | Hold Time | Skew Budget |
| --- | --- | --- | --- | --- |
| MIPI CSI-2 (CH1) | 1.5 Gbps/lane HS | UI/2 (HS mode) | UI/2 (HS mode) | <100ps between lanes |
| I2S/PDM (CH2) | 12.288 MHz BCLK | ≥5ns | ≥5ns | N/A |
| SPI (CH3/CH4/CH6/CH9) | ≤10MHz typical | ≥5ns | ≥5ns | N/A |
| I2C (CH4/CH8) | 100/400kHz | t(SU;DAT)≥100ns (FM) | t(HD;DAT)≥0ns | N/A |
| PCIe Gen4 (CH5) | 16 GT/s | UI=62.5ps | UI=62.5ps | Per PCIe CEM jitter budget |
| CAN FD (CH6/CH7/CH10) | ≤8Mbps (data phase) | Per ISO 11898-1 | Per ISO 11898-1 | N/A |
| Hardwired relay (CH11) | N/A (mechanical) | N/A | N/A | Operate-time budget — see §5.4 |

Table 4-4. Timing parameters by interface family

## 4.4 Voltage Level Specification

| Interface | VDD | VIH (min) | VIL (max) | Notes |
| --- | --- | --- | --- | --- |
| MIPI CSI-2 (CH1) | 1.1–1.2 V | HS: ~100mV differential swing | LP: 0.35×VDD | D-PHY HS is a low-swing differential scheme; single-ended VIH/VIL does not apply |
| I2S/PDM (CH2) | 1.8 V / 3.3 V | 0.65×VDD | 0.35×VDD | LVCMOS |
| UART/SPI (CH3) | 3.3 V | 2.0 V | 0.8 V | LVCMOS |
| SPI/I2C (CH4/CH8) | 1.8 V / 3.3 V | 0.7×VDD (I2C) | 0.3×VDD (I2C) | I2C open-drain, requires external pull-up |
| PCIe Gen4 (CH5) | N/A | 0.4V differential swing (AC-coupled) | — | NRZ differential, not LVCMOS levels |
| CAN / CAN FD (CH6/CH7/CH10) | 5V bus, 3.3V logic | Dominant differential ≥0.9V | Recessive differential ≤0.5V | ISO 11898-1/-2 CANH/CANL differential |
| Hardwired relay (CH11) | 24V DC (typical, install-specific) | N/A | N/A | Contact rating and coil spec are implementation-layer/vendor choices |

Table 4-5. Voltage level specification by interface family

# 5. EAI CH11 Safety Override — Functional Safety Design

An independent power supply and a hardwired relay (Att2 Table 5-2) are physical requirements. This section specifies the state machine, diagnostic coverage, safety registers, and response-time budget needed to bring CH11 within functional-safety certification scope.

## 5.1 Safety Concept

CH11 targets diagnostic coverage ≥90% for dangerous failure modes, in the spirit of IEC 61508 SIL-2 practice. This is a design target for the certification procedure defined in Section 7, not a claim of existing third-party certification. The core principle carried forward from Att2 Table 5-2 is retained and extended unchanged: CH11 cannot be reached through any register, bus, or logic path shared with WORM/HBF/Transform Engine, and the diagnostic functions below only observe those systems — they do not depend on them to operate.

## 5.1.1 Relay Specification — Force-Guided Contacts

The physical relay implementing the SAFE_STOP cutoff is specified as a force-guided relay compliant with EN 50205, not a general-purpose relay. A force-guided construction mechanically links the NO contact set and the NC contact set so that a single fault (e.g., a welded contact) cannot leave both closed simultaneously. This lets the NC contact independently verify the actual physical position of the NO (load) contact, rather than relying solely on the drive/command signal.

| Parameter | Specification | Notes |
| --- | --- | --- |
| Relay type | Force-guided relay, EN 50205 Type A or B | Not a general-purpose relay |
| Contact configuration | ≥1 NO (load path) + ≥1 NC (monitor path), mechanically interlocked | The NC contact is wired to NC_CONTACT_MISMATCH in Table 5-3 and used for independent state verification |
| Failure mode on contact welding | Simultaneous NO/NC agreement is mechanically prevented | Welding of one contact set is not concealed and is detectable as an NC/NO mismatch |
| Coil drive scheme | De-energize-to-open (fail-safe), consistent with the Table 5-2 SAFE_STOP behavior | Opens by default on loss of drive signal or power — consistent with the fail-safe requirement |
| Rated mechanical/electrical life | Implementation-layer/vendor choice | Outside standardization scope; only the force-guided construction and monitoring interface are standardized |

Table 5-1. CH11 relay physical specification

POWER_HEALTH_REG (Table 5-3) is extended with an NC_CONTACT_MISMATCH bit driven by this monitor contact, closing the gap between "SAFE_STOP was commanded" and "the contact is actually open" that the drive signal alone cannot detect.

Figure 5-1. CH11 safety state machine and diagnostic loop

## 5.2 Safety State Machine

| State | Condition | Action |
| --- | --- | --- |
| NORMAL | POWER_HEALTH_REG normal, WATCHDOG_TIMER not expired | Relay held closed by active drive; E2E #2 control plane operates normally |
| DEGRADED | Single fault: one of {Main PSU, Backup UPS/supercapacitor, heartbeat} flagged, redundant path still normal | Relay held closed; fault logged (unified audit format of §1.4/2.4); AAI notified via the CH14 coordination channel |
| SAFE_STOP | Simultaneous second fault, or explicit override command, or watchdog expiry | Relay opens (fail-safe = de-energized open); independent power sustains the controller and audit logging for ≥30 seconds |

Table 5-2. CH11 safety state definitions

Recovery from DEGRADED to NORMAL requires the fault condition to clear and self-test to pass. SAFE_STOP is latched and requires an explicit out-of-band reset rather than a register write reachable from the shared bus — consistent with the principle of keeping CH11 outside every shared logic domain.

## 5.3 Safety Registers

| Register | Width | Description |
| --- | --- | --- |
| SAFETY_STATE_REG | 8-bit | 0x0 NORMAL · 0x1 DEGRADED · 0x2 SAFE_STOP (latched, requires out-of-band release) |
| WATCHDOG_TIMER | 16-bit | Counts down from the heartbeat period; the E2E#1↔CH11 heartbeat per the Att2 Table 5-2 fault-propagation model |
| POWER_HEALTH_REG | 8-bit | Bit 0: MAIN_PSU_OK · Bit 1: BACKUP_UPS_OK · Bit 2: SUPERCAP_CHARGE_OK · Bit 3: CROSS_CHECK_MISMATCH · Bit 4: NC_CONTACT_MISMATCH (force-guided relay NC/NO mismatch, §5.1.1) |
| OVERRIDE_TRIGGER_REG | 8-bit | Bit 0: MANUAL_TRIGGER (physical, not register-issued) · Bit 1: WATCHDOG_TRIGGER · Bit 2: DUAL_FAULT_TRIGGER — records the cause of SAFE_STOP entry for post-incident audit |

Table 5-3. CH11 safety register summary — every register is readable-observable from the shared-bus side for logging purposes only; none of the registers in this list can command a state transition from the shared-bus side

## 5.4 Response-Time Budget

| Stage | Target Latency | Basis |
| --- | --- | --- |
| Fault detection | <10ms | POWER_HEALTH_REG cross-check comparator cycle |
| State transition (DEGRADED/SAFE_STOP) | <5ms | Hardwired state machine, no dependency on software scheduling |
| Relay physical operate time | <15ms | Typical electromechanical relay opening time; specific part is an implementation-layer/vendor choice |
| Worst-case fault-to-open time | <30ms | Sum of the items above — independent of, and much faster than, the ≥30-second hold requirement (which governs the controlled shutdown procedure after the trip, not the trip time itself) |
| Independent power sizing | C ≥ (I × t) / ΔV | Sizes the supercapacitor bank so the CH11 controller + relay hold current I is sustained for hold time t=30s within an allowable voltage drop ΔV; exact capacitance is an implementation-layer/vendor choice |

Table 5-4. CH11 response-time and power-sizing budget

## 5.5 E2E Center Governance Profile — Human-in-the-Loop Interface

EAI and PAI differ not only in deployment location (Att2 §1.1) but also in the required granularity of governance. Att2 Table 4-2 already distinguishes the two at the M-Axis level — EAI accumulates "mission experience" on a per-incident basis, while PAI's M-Axis operates at the aggregated level of "interpretation tags, policy updates." This document extends that distinction to the governance layer: because the E2E Center (unlike an AI Data Center) is a location where personnel are physically present, an individual EAI incident by default falls within the scope of human-operator intervention before, or independently of, AAI's aggregate-level Approve/Reject governance (Att2 §9.2).

The specific degree of human intervention is deliberately not standardized in this document. A city patrol-robot center and a disaster-response center may weight human intervention differently, and fixing the axes or thresholds of that weighting would conflict with the standardization boundary already established in Att2 §9.3 (structure and interfaces are standardized; specific policies and algorithms are not). What is standardized is only the existence of the configuration point itself, and its interface with the two channels already defined in this document.

| Term | Definition |
| --- | --- |
| E2E Center Governance Profile | A per-deployment configuration point that specifies the degree of human-in-the-loop involvement for individual EAI incidents. The specific axes and thresholds are deliberately left to each deployment site (e.g., a city patrol-robot center and a disaster-response center may carry different weightings) and are out of standardization scope — only the existence of this configuration point, and its interface with the CH11 MANUAL_TRIGGER and AAI escalation paths, is standardized. |

Table 5-5. E2E Center Governance Profile — definition

## 5.6 EAI_GOV_PROFILE_REG

Defines a minimal hook register that makes the existence of the Governance Profile and its two standardized interface points machine-readable, without encoding the policy content itself. Each bitfield has a distinct access type — OTP (one-time-programmed, set at deployment, not runtime-changeable) or RW (runtime-adjustable) — because whether a field is reconfigurable is a register-level property independent of the policy content it points to.

| Bit | Field | Access | Description |
| --- | --- | --- | --- |
| [0] | HUMAN_IN_LOOP_EN | OTP | Set at deployment; defaults to reflecting that human presence is a defining characteristic of EAI (Att2 §1.1). Not runtime-changeable — turning human involvement itself on or off is treated as a redeployment-level decision, not an operational one |
| [1] | AUTO_ESCALATION_EN | RW | Whether an individual incident is automatically forwarded to AAI's aggregate governance, or only after local processing; adjustable at runtime by the resident operator or AAI within deployment-policy scope |
| [7:2] | RESERVED | — | Fixed 0b000000 |
| [15:8] | PROFILE_ID | OTP | Opaque identifier pointing to a per-deployment profile, fixed at deployment; the profile content itself is out of standardization scope |

Table 5-6. EAI_GOV_PROFILE_REG — interfaces with OVERRIDE_TRIGGER_REG (§5.3, MANUAL_TRIGGER) and the AAI escalation path (Att2 §9.1); does not itself carry policy content

Under this model, when HUMAN_IN_LOOP_EN is set, an individual EAI incident reaches the resident operator through the same physical MANUAL_TRIGGER path already defined in §5.3 — no new physical channel is introduced. What AUTO_ESCALATION_EN determines is whether AAI's aggregate-level governance (Att2 §9.2) sees that incident as a raw individual event, or only after it has already been processed locally and folded into aggregated data — while PAI's aggregate-only granularity (Att2 Table 4-2) is preserved unchanged.

# 6. Network Adapter — L4/L5 Boundary Interface

Att2 stops at the output of Dedicated Equipment Block B (MUX Engine, now defined at the register level in §3.3): it states only that the 24-core Bundle heads "toward the D12 AAI node" (Att2 §5.1, §7.2), without previously defining the interface between that Bundle and the upper network layer. Att2 §9.3 excludes generic network transport protocols from standardization scope, but the physical/framing handoff at this boundary is the last Velsanet-owned interface before the cube leaves Velsanet-owned equipment, and it carries the SEU_ID and ACL_PERM fields defined in Sections 1–2.

Figure 6-1. Network Adapter boundary

## 6.1 Cube Frame Header

| Field | Width | Source |
| --- | --- | --- |
| SEU_ID | 80-bit | T-Axis Timestamp Engine output (§1.1, offset 0x20) |
| AXIS_ID | 4-bit | Axis of the payload cube (T=0, C=1, I=2, E=3, M=4) |
| ACL_PERM | 8-bit | Permission value resolved for that payload's axis (§2.1, Table 2-2) |
| FRAME_LEN | 16-bit | Payload length in bytes |

Table 6-1. Cube Frame header — 108-bit fixed prefix

## 6.2 Adapter Registers

| Register | Width | Description |
| --- | --- | --- |
| NET_ADAPTER_CTRL | 16-bit | Bit 0: GLOBAL_ENABLE · Bit 1: HEADER_INSERT_EN (hardwired 1) |
| LINK_STATUS | 8-bit | Bit 0: LINK_UP · Bit 1: SYNC_LOCK (PTP) · Bit 2: BACKPRESSURE — fed from MUX_BUNDLE_STATUS (§3.3) |

Table 6-2. Network Adapter register summary

## 6.3 Physical Handoff by Deployment Type

EAI (E2E Center) simply extends the same fixed MOCT ribbon cable upward with no protocol conversion. PAI (AI Data Center) undergoes O/E conversion and hands off to the rack-level data-center uplink fabric (800G class), with the Cube Frame header preserved bit-for-bit across the conversion.

## 6.4 Bidirectional Cube Delivery — Realizing CH14

Sections 6.1–6.3 describe only the upward direction: the path by which an individual EAI/PAI node's cube travels to the D12 AAI. The same Cube Frame mechanism also carries the downward direction — AAI delivering cube content back to an individual node — and this is the physical realization of what Att2 Table 3-3 names "CH14, AAI Coordination, Mesh link." No separate physical channel or dedicated PHY is introduced for this: per Att2 §8.1–§8.2, the E2E parallel structure already holds far more optical cores at the boundary than a single node's traffic requires, and CH14 is realized within that existing capacity by claiming one idle channel for the duration of the transfer — using the same Cube Frame header (§6.1) in the reverse direction.

The access-control path for this downward delivery is identical to Section 2: AAI has RW permission on the I-Axis (Table 2-2, ACL_PERM 0x1F) but only Read permission on the C-Axis (ACL_PERM 0x17). Consequently, a downward instruction such as "caution in zone X" is not written by AAI directly into the target node's C-Axis; it is delivered as an I-Axis (Intent) cube from AAI to the target node's PAI. The receiving node's own PAI — per Att2 Table 4-1, the sole writer of its own C-Axis — reflects that instruction onto its own Context axis in whatever manner and timing it judges appropriate. This preserves the axis-ownership boundary established in Section 2 without altering the ACL_PERM matrix in any way.

# 7. Hardware Verification Strategy Cross-Reference

| Att2 / WP#24 Verification Item | Related Registers | Pass Criteria |
| --- | --- | --- |
| HBM WORM rewrite blocking | WORM_SECTOR_LOCK[0], VIOLATION_FLAG[7] | 100% physical blocking under fault-injection testing |
| HBF ACL violations | ACL_PERM (all axes), unified audit log (§1.4/2.4) | Zero violations across 100 million operations of formal verification + fuzzing |
| DEMUX non-collision | DEMUX_NONCOLLISION_CHK | Mathematical proof + measured verification at power-on self-test |
| SEU completeness | SEU_CHANNEL_COMPLETE, SEU_MISSING_FLAG | 0% missing (excluding optional channels), simultaneous injection on 8/16 channels |
| CH11 independence | POWER_HEALTH_REG, SAFETY_STATE_REG (Section 5) | Self-sustained ≥30 seconds under power-cutoff scenario; worst-case fault-to-open <30ms |
| CH11 relay contact verification | NC_CONTACT_MISMATCH (§5.1.1) | Force-guided NC/NO mismatch detected within one diagnostic cycle (<10ms) |
| Audit traceability | EPOCH_ID + TIMESTAMP_NS + REQUESTOR_ID (§1.4/2.4) | Unambiguous time-ordered alignment and source attribution across the entire operating period |
| Audit-buffer overflow handling | AUDIT_LOG_STATUS (§1.5) | OVERFLOW_WARN fires without omission at 87.5% fill under sustained write-stress testing |

Table 7-1. Register-to-verification cross-reference

# 8. Conclusion

This document takes the WORM and HBF ACL functionally described in Att2 §3.2 and §4.1 down to the register level: a 64-bit WORM_SECTOR_LOCK bitmap; a hardwired WORM_CTRL register; a 128-bit audit log with EPOCH_ID and REQUESTOR_ID that fully guarantees time-ordered and source traceability; an 8-bit ACL_PERM encoding that resolves Att2's 5-axis matrix; a Layer Lock register for M-Axis write coordination; register-level definitions of the DEMUX Engine, SEU Assembly Engine, and MUX Engine; pin-level, timing, and voltage specifications for every channel interface; a complete functional-safety design for CH11 (state machine, diagnostics, response-time budget, force-guided relay specification); a fixed-depth audit-buffer policy with explicit overflow handling; governance registers with specified access types (OTP/RW); concrete PHY assignments for CH13–CH16; and an E2E Center Governance Profile hook that connects per-incident human involvement to CH11's MANUAL_TRIGGER path without encoding policy content.

Next document in Att2 §11.1 order: WP#25 (SEU Synchronization Protocol Specification) — formalizes the SEU packet format and PTP synchronization profile at the wire level, building on the SEU Assembly Engine registers defined in §3.2 of this document.

VELSANET ARCHITECTURE GROUP | TECHNICAL DESIGN DOCUMENT

# WP#25 R1 — SEU1 / SEU2 Synchronization Specification

Two-Part Structure: SEU1 (Common Base, PAI + EAI) · SEU2 (EAI-Only Specialized Spec)

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Public — Open Specification | v1.1 (R1) │ 2026 | SEU Wire-Level Record / PTP / Algorithm — Two-Part | Supersedes structural framing of WP#25 v1.0; extends Att2 §5.3/§6.1, WP#24 §3.2, §5.3, §6.1 |

Republic of Korea | Velsanet Architecture Group

Governance note: This specification is published in full under the Velsanet open-specification policy. Certification against this record format, PTP profile, and completeness algorithm — not confidentiality — is the mechanism by which conformance and interoperability are controlled.

# Revision Log — R1 vs. v1.0

Per Velsanet document-integrity practice, revisions are addition-only: this section records what changed and why, rather than silently overwriting v1.0’s framing. The technical content of v1.0 (register cross-references, bit-field widths, algorithm logic) is preserved wherever it was not itself the subject of correction.

| # | Change | Rationale |
| --- | --- | --- |
| R1-1 | Term “packet” replaced throughout with “SEU record.” | The structure defined in v1.0 §1 never leaves the node and is HBM-resident, not transmitted — “packet” wrongly implied a network-transport object. “Record” matches what the structure actually is: a metadata/pointer entry consumed entirely within one optical core’s local pipeline. |
| R1-2 | SEU2 (EAI control-plane structure) re-defined as a structurally distinct specification, not an extension of SEU1. | v1.0 treated CH9–CH16 as an optional/extended portion of one 16-channel SEU. This conflated two structures with different existence conditions: SEU1 exists for every PAI and every EAI (sensory function, universal); SEU2 exists only for EAI (control/actuation/safety function, categorically absent from PAI — not optional, but structurally not present). The document is restructured into Part A (SEU1, common) and Part B (SEU2, EAI-only). |
| R1-3 | Optical-core mapping clarified: 1 optical core = 1 SEU1 instance; EAI additionally requires a second, physically separate optical core = 1 SEU2 instance. | v1.0 and WP#24 described EAI’s 16 channels as occupying core sub-groups within a single ribbon (WP#24 Table 5-2, cores #1–24 / #25–48). This is restated at the SEU-record level as two independent optical cores, consistent with the physical-isolation requirement already present in Att2 §5.3 / WP#24 §5.3. |
| R1-4 | Geographic/topological placement of O8/D12/I20 (density-driven D12 sizing) is out of scope for this document. | That question belongs to network-deployment topology (O8/D12/I20 placement), a different layer than SEU synchronization. It will be addressed in a separate forthcoming document, not in WP#25. |
| R1-5 | Core Stack Activation Condition (B.13.1) is determined by negotiation between the AI system (PAI/EAI) and Velsanet equipment over the Connectivity Plane — not by a fixed provisioning table. The negotiation protocol itself (request/response format, power-state and capacity confirmation) is out of scope here and deferred to the forthcoming O8 document. | This negotiation is a precondition of the same kind already established for hypercube-conformant connection prioritization (WP#20 §6.2, Q7 hypercube adjacency): synapse-like strengthening of a connection across the hypercube network presupposes an AI-side request and a negotiated confirmation — it is not self-executing. Once negotiated, Core Stack cores operate deterministically for the duration of the SEU window (A.7.1/B.11); negotiation does not recur per-window and does not reintroduce per-packet queuing (Att2 §2.1, Deterministic Parallelism). |
| R1-6 | PAI and EAI are redefined as two states of one underlying instance rather than two hardware categories fixed at fabrication. Layer 0 (SEU1) is the universal, unconditional base for every instance; Layer 1 (SEU2-A) is a negotiated control-activation boundary — an instance with Layer 1 active operates as EAI, without it as PAI. | Cognition (SEU1) is identical between PAI and EAI; only the control side differs. This makes explicit, at the Core Stack level, a principle already stated for the Cognition Cube engine itself (WP#24 §2.1: “the Cognition Cube hardware follows a 5-layer structure applicable to both EAI and PAI”). B.9’s “1 core for PAI / 2 cores for EAI” figures are unchanged as outcomes, but are now understood as the result of a control-activation decision, not a fixed hardware SKU. |

Table R-1. Revision Log, WP#25 v1.0 → R1

# 0. Purpose and Relationship to Att2 and WP#24

Att2 §6.1 defines the SEU Synchronization Pipeline functionally; WP#24 §3.2 brings its fourth stage to register level. WP#24 §5.3 additionally establishes that EAI’s perception plane (E2E #1) and control plane (E2E #2) are physically isolated onto separate core groups. R1 of this document takes that physical-isolation requirement to its structural conclusion: perception and control are not two halves of one SEU type, but two SEU types — SEU1 and SEU2 — with different existence conditions, different optical-core bindings, and, for SEU2, a different purpose entirely (command/actuation/safety rather than sensory encapsulation).

As with Att2 and WP#24, the methodology/execution separation established in Att2 §1.3 applies throughout.

PART A — SEU1: COMMON BASE SPECIFICATION (PAI + EAI)

# A.1 Position and Scope of SEU1

SEU1 is the sensory encapsulation structure defined at Att2 §6.1 / FGEAI-I-011R1 (Eight-Channel Multimodal Decomposition), scoped to CH1–CH8. It is universal: every instance — whether operating as PAI or as EAI — has exactly one SEU1. As established in B.13.1, PAI and EAI are not two categories of hardware fixed at fabrication; they are two states of the same underlying instance, distinguished only by whether the control capability (SEU2, Part B) has been negotiated on. Cognition itself (SEU1) is identical either way — only the control side differs (B.13.1). SEU1 is not a subset or a truncation of some larger structure — it is complete in itself, and is the only SEU an instance has while operating in PAI mode (control inactive).

SEU1 is bound to exactly one optical core. For PAI, that core is the entirety of the PAI instance’s Velsanet-facing interface (WP#24 §7.2). For EAI, that core is E2E #1 (WP#24 Table 5-2, cores #1–24 of the ribbon’s perception group); EAI’s second core, carrying SEU2, is defined separately in Part B.

## A.2 SEU1 Record Format

The SEU1 record (formerly “SEU packet,” R1-1) is produced by Stage 3 (per-channel independent receive buffer, Att2 §6.1) and consumed by Stage 4 (SEU Assembly Engine, WP#24 §3.2), entirely within the optical core’s local HBM/DEMUX domain. It never leaves that core; only the completed cube it produces is later wrapped in the Cube Frame Header (WP#24 §6.1) for the L4/L5 handoff. Consistent with the HBM DMA write model (Att2 §6.1 Stage 4, Att2 §3.1), the record is a metadata/pointer structure, not a payload container — raw channel data is written to HBM directly by Stage 3.

## A.3 SEU_ID Bit Layout (80-bit)

Unchanged from v1.0. Att2 §6.1 Stage 4 defines SEU_ID = { NodeID : Timestamp_ns : ChannelHash }; WP#24 Table 1-1/6-1 fix the 80-bit width. NODE_ID is fixed per physical optical core at fabrication (WP#24 §1.1) — for EAI, SEU1’s core and SEU2’s core therefore receive distinct NODE_ID values by construction, without any additional field being required to distinguish them (see A.7 and B.9).

| Field | Width | Definition |
| --- | --- | --- |
| NODE_ID | 24-bit | Fixed per physical optical core at fabrication. Uniquely identifies this specific core — the PAI instance’s sole core, or an EAI’s E2E #1 core. |
| TIMESTAMP_NS | 48-bit | Nanosecond timestamp from the T-Axis Timestamp Engine; shares rollover period with T_AXIS_EPOCH_CTR (WP#24 §1.1, ~78.9h). |
| CHANNEL_HASH | 8-bit | Hash of the final SEU_CHANNEL_COMPLETE bitmap (WP#24 §3.2) at flush; a compact, order-independent fingerprint of which CH1–CH8 channels contributed. |

Table A-1. SEU_ID (80-bit) — identical structure to v1.0 §1.2; scope now explicitly SEU1

## A.4 Per-Channel Sub-Record

Renamed from “sub-header” (R1-1); fields unchanged from v1.0 §1.3.

| Field | Width | Definition |
| --- | --- | --- |
| CHAN_ID | 3-bit | Physical channel number, 0–7 (SEU1 is always an 8-channel structure, for both PAI and EAI). |
| SLOT_TS_OFFSET | 16-bit | Arrival time offset from window opening edge, 100µs ticks. |
| MISSING_FLAG | 1-bit | Set for null substitution (A.7.4). |
| CONFIDENCE | 8-bit | Meta Abstraction Layer confidence output; 0x00 while unscored. |
| META_PTR | 32-bit | Pointer into this core’s HBM raw buffer / Meta Abstraction record. 0xFFFFFFFF = NULL_MARKER. |
| RESERVED | 4-bit | Fixed 0b0000; padding to 64-bit sub-record boundary. |

Table A-2. SEU1 Per-Channel Sub-Record — 64-bit, one instance per contributing channel (CH1–CH8 only)

## A.5 Full SEU1 Record Structure

| Segment | Width | Content |
| --- | --- | --- |
| SEU1 Record Header | 112-bit | SEU_ID [80-bit, A.3] + WINDOW_ID [16-bit, monotonic per-core slot counter] + CHAN_COUNT [8-bit, ≤8] + ASSEMBLY_STATE [8-bit, mirrors SEU_ASSEMBLY_STATUS] |
| Per-Channel Sub-Records | 64-bit × CHAN_COUNT | One per contributing channel, A.4; CHAN_ID ascending |

Table A-3. SEU1 Record Structure — the Control-Plane Extension field present in v1.0****‘******s single-record model is removed here; control-plane content is now entirely SEU2******’****s own record (Part B), not an extension field on this one

Figure A-1. SEU1 record: fixed 112-bit header + up to 8 × 64-bit sub-records, no control-plane extension

## A.6 PTP Synchronization Profile — SEU1

Profile identical to v1.0 §2, scoped explicitly to SEU1’s optical core.

| Parameter | Value | Note |
| --- | --- | --- |
| Profile identity / domain | Velsanet-EAI-PTP-1, Domain 4 | Applies to every SEU1 core — PAI and EAI’s E2E #1 alike |
| Delay mechanism | End-to-end (E2E) | Fixed-length MOCT ribbon path, statically calibrated |
| Timestamping | One-step | Consistent with DEMUX_LATENCY_MON <2ns target (WP#24 Table 3-1) |
| Sync interval | 2^-4 s (16/s) | Margin against 800ms minimum SEU1 window |
| Accuracy budget | < 100ns | Decomposed per Table A-5 |

Table A-4. SEU1 PTP Profile

| Contributor | Budget |
| --- | --- |
| Grand Master reference stability | < 20ns |
| Path asymmetry residual | < 15ns |
| Residence/turnaround time | < 25ns |
| Receiver timestamp granularity | < 20ns |
| Margin | < 20ns |

Table A-5.** ****<****100ns Budget Decomposition (Att2 §6.1) — unchanged from v1.0 §2.3

## A.6.1 Grand Master Topology

As v1.0 §2.2: the PTP Grand Master is sourced from the same I20-rooted reference feeding the passive optical clock tree (WP#23), distributed to channel receivers as IEEE 1588v2 packets downstream of O/E conversion — a distinct mechanism from optical injection locking, cleanly separable for fault isolation and independent validation.

## A.6.2 Fault Handling — Loss of PTP Lock

As v1.0 §2.4: PTP slave lock is carried in DEMUX_CLK_PLL_STATUS (WP#24 Table 3-1); loss of Grand Master reachability triggers local holdover with a HOLDOVER_FLAG, degrading accuracy monotonically rather than blocking SEU1 assembly.

# A.7 Channel Completeness Algorithm — SEU1

## A.7.1 State Machine

| State | Entry Condition | Exit Condition |
| --- | --- | --- |
| COLLECTING | New slot opens (Att2 §6.1 Stage 2); SEU_CHANNEL_COMPLETE cleared | Slot boundary reached → FLUSHING |
| FLUSHING | Slot boundary reached | Mandatory-channel check (A.7.2) → COMPLETE or ABORTED |
| COMPLETE | All mandatory bits set at flush; optional channels may carry MISSING_FLAG | HBM DMA write issued; returns to COLLECTING |
| ABORTED | ≥1 mandatory bit unset at flush | No HBM DMA write (A.7.5); returns to COLLECTING |

Table A-6. SEU1 SEU_ASSEMBLY_STATUS Transition Table — identical logic to v1.0 §3.1, scope restricted to CH1–CH8

## A.7.2 Mandatory vs. Optional Channel Classification

| Channel | Classification | Rationale |
| --- | --- | --- |
| CH1 Video | Mandatory | Window boundary anchor (Att2 §6.1 Stage 2) |
| CH2 Audio | Optional | Degrades gracefully |
| CH3 Spatial-Temporal | Mandatory | Required for T/C-Axis grounding |
| CH4 Biometric | Optional | Drives adaptive window (A.7.3) but not itself required for completeness |
| CH5 Text/Linguistic | Optional | — |
| CH6 Activity/Motion | Optional | — |
| CH7 Device Interface | Optional | — |
| CH8 Environment | Optional | — |

Table A-7. SEU1 Mandatory/Optional Classification — applies identically to PAI and to an EAI****’****s E2E #1

## A.7.3 Adaptive Window Control Algorithm

IF CH4 sample delta over rolling 200 ms > RAPID_THRESHOLD: SEU_WINDOW_CTRL.CH4_RAPID_TRIGGER := 1 CURRENT_WINDOW_MS (next slot) := 800

ELSE IF aggregate weighted change-rate, CH1-CH8, over 5 consecutive windows < LOW_RATE_THRESHOLD: CURRENT_WINDOW_MS (next slot) := 1500

ELSE: CURRENT_WINDOW_MS (next slot) := 1000 (default, static mode)

Unchanged from v1.0 §3.3. RAPID_THRESHOLD / LOW_RATE_THRESHOLD remain deployment-tunable (execution layer, Att2 §1.3); CH4_RAPID_TRIGGER always takes priority.

## A.7.4 Missing Channel and Timeout Handling

Unchanged from v1.0 §3.4: null substitution via META_PTR = NULL_MARKER (0xFFFFFFFF) with MISSING_FLAG set; Meta Abstraction Layer must not generate semantic content for a null-substituted channel.

## A.7.5 ABORTED Disposition

Unchanged from v1.0 §3.5: no HBM DMA write, no HBF cube for that window; audit-log entry via the EPOCH_ID/TIMESTAMP_NS/REQUESTOR_ID format (WP#24 §1.4/2.4); SEU_ABORT_COUNTER (16-bit, saturating) accumulates events; no automatic replay.

PART B — SEU2: EAI-ONLY SPECIALIZED SPECIFICATION (ANNEX)

SEU2 is not an extension of SEU1. SEU1 is the afferent (perception) structure shared by every PAI and every EAI. SEU2 is the efferent (control) structure that exists only for EAI, serving a categorically different purpose. A PAI implementation has no SEU2 code path to disable or leave unpopulated — SEU2 is simply not part of what PAI is.

# B.8 Why SEU2 Is Structurally Distinct, Not an Extension

WP#25 v1.0 modeled EAI’s 16 channels as CH1–CH8 (perception) plus an optional CH9–CH16 extension of one SEU structure, controlled by a Control-Plane Extension field on the same record (v1.0 Table 1-3). This section corrects that framing. Att2 §5.3 and WP#24 §5.3 already required that EAI’s perception plane (E2E #1) and control plane (E2E #2) be physically isolated onto separate core groups, with independent power and independent clock source for the control plane. A structure that is physically isolated down to its power and clock source is not an “extension” of the structure it is isolated from — it is a separate structure that happens to share a naming convention (“SEU”) and an assembly algorithm shape (Table A-6’s state machine).

## B.8.1 Biological Grounding — Afferent vs. Efferent Nervous Systems

The precise structural analogue is the anatomical division of the vertebrate nervous system into afferent and efferent pathways, not a cortical functional division. This is a stronger correspondence than a “which brain region does what” analogy, because afferent/efferent separation is physical from the periphery inward — at the spinal cord, afferent fibers enter through the dorsal root and efferent fibers exit through the ventral root, two anatomically distinct bundles from the point of origin, not two modes of one shared bundle. This matches SEU1/SEU2’s separate-optical-core requirement (B.9) far more precisely than a shared-substrate cortical analogy would: the separation is not organizational, it is physical, and it starts at the periphery.

| Nervous System | Biological Correspondence | WP#25 Correspondence | Role |
| --- | --- | --- | --- |
| External Perception | Afferent (sensory) pathway — dorsal root | SEU1 | Sense organs → central system: carries the world inward |
| Internal Control | Efferent (motor) pathway — ventral root | SEU2 | Central system → effectors: carries commands outward |

Table B-0. Afferent/Efferent Correspondence — grounds the SEU1/SEU2 physical-isolation requirement in the same separation vertebrate nervous systems already exhibit at the spinal root

The correspondence holds on three axes simultaneously, matching the three grounds on which SEU1/SEU2 isolation is already required in Att2 §5.3 / WP#24 §5.3: (1) anatomical — separate roots/separate optical cores; (2) functional — sensing the world vs. acting on it; (3) temporal — afferent traffic is continuous and adaptively windowed (A.7.3), while efferent traffic is event-driven, firing at the moment a command is issued rather than on a perception cadence. None of the three axes is derivable from the other two, which is why SEU2 cannot be treated as a mode or extension of SEU1 without losing exactly the property (independent failure and independent timing) that the physical isolation exists to guarantee.

The practical consequence: SEU2 is defined here as its own record format, its own PTP domain, and its own completeness algorithm instance — structurally parallel to SEU1, not layered on top of it. PAI never instantiates SEU2. This is not because SEU2 is “optional” for PAI; it is because SEU2’s function (command input, actuation output, safety override, mission-state tracking) has no PAI-side referent to begin with — a purely afferent system has no efferent root to speak of.

# B.9 Position, Scope, and Optical-Core Binding of SEU2

SEU2 exists only for an instance operating in EAI mode (Layer 1 active, R1-6) — one instance per such EAI, bound to a second optical core distinct from that instance’s SEU1 core (R1-3) — corresponding to E2E #2 / cores #25–48 of WP#24 Table 5-2’s ribbon grouping, restated here as a physically separate core rather than a sub-group of one shared core. Because NODE_ID (Table A-1) is fixed per physical core at fabrication, SEU1’s core and SEU2’s core of the same EAI naturally carry distinct NODE_ID values; the association between a given SEU1 core and its paired SEU2 core is maintained as an O8-level pairing registry, which is a network-topology-layer concern and is out of scope for this document (see Revision Log R1-4) — it is deferred to the forthcoming O8/D12/I20 placement document.

SEU2 is invariantly paired: a SEU2 instance never exists without exactly one paired SEU1 instance belonging to the same EAI. There is no standalone-SEU2 deployment case, because control/actuation without the perception context SEU1 provides has no defined role in this architecture.

# B.10 SEU2 Record Format

The SEU2 record reuses the SEU_ID structure of Table A-1 without modification — no additional field is needed to distinguish SEU1 from SEU2, since they live on physically distinct cores with physically distinct NODE_ID values by construction.

| Field | Width | Definition |
| --- | --- | --- |
| CHAN_ID | 3-bit | Physical control channel number. Retains the CH9–CH16 absolute numbering of Att2/WP#24 for cross-reference continuity, but is understood here as SEU2’s own complete 8-channel set, not channels 9 through 16 of a 16-channel whole. |
| SLOT_TS_OFFSET | 16-bit | As Table A-2, against SEU2’s own independent window (B.12). |
| MISSING_FLAG | 1-bit | As Table A-2. |
| CONFIDENCE | 8-bit | As Table A-2; largely inapplicable to CH11 (excluded, B.11.1) and CH13-15 categories. |
| META_PTR | 32-bit | Pointer into SEU2’s own HBM buffer domain — physically separate from SEU1’s, per the independent-power/independent-clock isolation of WP#24 Table 5-2. |
| RESERVED | 4-bit | Fixed 0b0000. |

Table B-1. SEU2 Per-Channel Sub-Record — 64-bit, structurally identical field layout to SEU1****’****s but a distinct HBM/core domain

| Segment | Width | Content |
| --- | --- | --- |
| SEU2 Record Header | 112-bit | SEU_ID [80-bit, Table A-1, this core’s own NODE_ID] + WINDOW_ID [16-bit, independent counter] + CHAN_COUNT [8-bit, ≤8] + ASSEMBLY_STATE [8-bit] |
| Per-Channel Sub-Records | 64-bit × CHAN_COUNT | Table B-1, one per contributing control channel |

Table B-2. SEU2 Record Structure — a complete, independent record, not an attachment to SEU1****’****s

# B.11 Channel Completeness Algorithm — SEU2

SEU2 runs its own instance of the state machine defined in Table A-6, independently of SEU1’s instance (this independence was named SEU_CTRL_PLANE_WINDOW in WP#24 §3.2; R1 clarifies that it is not a second window on one engine but a second, complete engine).

## B.11.1 Mandatory / Optional / Excluded Classification

| Channel | Classification | Rationale |
| --- | --- | --- |
| CH9 Command Input | Mandatory | Primary SEU2 input |
| CH10 Actuation Output | Optional | Absence reflects no commanded actuation, not a fault |
| CH11 Safety Override | Excluded | Shares no register, bus, or logic path with SEU synchronization of any kind (WP#24 §5) — excluded from SEU2 exactly as it was excluded from SEU1’s scope |
| CH12 Auth/Identity | Optional | — |
| CH13 Mission State | Mandatory | Required for M-Axis mission-experience grounding |
| CH14 AAI Coordination | Excluded | Logical/idle-capacity delivery only (WP#24 §6.4); not slot-synchronized |
| CH15 Audit/Evidence | Excluded | Internally generated, WORM-backed; trivially complete by construction |
| CH16 Redundancy Link | Excluded | Hot-standby duplicate of another channel; not independently evaluated |

Table B-3. SEU2 Channel Classification

## B.11.2 Missing-Channel and ABORTED Handling

Identical mechanics to A.7.4/A.7.5 (null substitution, audit log, SEU_ABORT_COUNTER), applied to SEU2’s own SEU_CHANNEL_COMPLETE/SEU_MISSING_FLAG/SEU_ASSEMBLY_STATUS instance. An ABORTED SEU2 slot has no effect on SEU1’s assembly state and vice versa — this mutual non-interference is the entire point of the two-engine model (B.8).

# B.12 PTP Synchronization Profile — SEU2

| Parameter | SEU2 Value | Note vs. SEU1 |
| --- | --- | --- |
| Domain number | 5 | Distinct from SEU1’s Domain 4 (Table A-4); deployment policy may require this when SEU2’s clock source is independent (WP#24 Table 5-2, “Independent Crystal OSC”) |
| Delay mechanism / Timestamping / Sync interval | Same as Table A-4 | Profile shape is shared; only the domain and physical clock source differ, consistent with WP#24 Table 5-2’s independent-clock requirement for E2E #2 |
| Accuracy budget | < 100ns, same decomposition as Table A-5 | Independent oscillator (not shared with SEU1’s Grand Master path) is itself part of the fault-isolation guarantee |

Table B-4. SEU2 PTP Profile — deliberately on a separate domain/oscillator from SEU1, per WP#24 Table 5-2

# B.13 EAI Core Stack — Adaptive Optical Core Allocation

The optical-core counts fixed in B.9 (1 SEU1 core + 1 SEU2 core per EAI) are a mandatory baseline, not a ceiling. This section defines the EAI Core Stack: the layered structure by which an EAI’s optical-core allocation grows beyond that baseline under specific network conditions. “Core Stack” is deliberately not called “O8” — O8 already denotes the city-layer hub node (192 optical cores, WP#24 §5.2/§7.2, WP#20 §5.3); the Core Stack is a per-EAI-instance allocation profile at a completely different scale, and reusing O8’s name for it would misstate which structure is being sized.

## B.13.1 Stack Layers

Layer 0 and Layer 1 are not both “mandatory” in the same sense. Layer 0 (SEU1) is present in every instance unconditionally. Layer 1 (SEU2-A) is what an instance’s negotiation with Velsanet equipment (R1-5) turns on when — and only when — control/actuation capability is required; an instance with Layer 1 active is, by that fact, operating as EAI, and an instance without it is operating as PAI. This is the control-activation criterion fixed in this update: cognition (SEU1) is identical in both modes; only the control side differs.

| Layer | SEU Type | Cores | Activation Condition |
| --- | --- | --- | --- |
| 0 — Universal Base | SEU1 | 1 (fixed) | Always present, unconditionally — every instance’s minimum perception path (A.1). Present whether the instance is operating as PAI or as EAI. |
| 1 — Control Activation | SEU2-A | 1 (fixed once active) | Negotiated on via the Connectivity Plane (R1-5) when control/actuation capability is required. An instance with Layer 1 active is, by definition, operating as EAI (B.13.2); without it, the instance operates as PAI. |
| 2 — Conditional | SEU1 | +0 to +3 (total ≤ 4) | Activated when deployment requires perception fault tolerance — e.g., mission-critical or safety-relevant robots where a single-core perception failure is unacceptable |
| 3 — Conditional | SEU2-B | +0 to +2 | Activated only when Layer 1 (control) is already active AND a supervising D12/E2E Center exists to receive cognition reports (B.13.2). An instance with Layer 1 inactive (PAI mode), or with no supervising node, has zero SEU2-B cores — structurally absent, not merely unpopulated (B.8) |

Table B-6. EAI Core Stack Layers — Layer 0 is the universal baseline (A.1); Layer 1 is the negotiated control-activation boundary that distinguishes EAI-mode from PAI-mode; Layers 2–3 are further conditional extensions

Each layer’s Activation Condition (Table B-6) is resolved through negotiation between the AI system and Velsanet equipment over the Connectivity Plane — not read from a static table — accounting for real-world conditions such as equipment power state and current acceptance capacity (Revision Log R1-5). The negotiation protocol itself is out of scope for this document and is deferred to the forthcoming O8 document. Once a layer is activated, its cores operate deterministically for the duration of each SEU window (A.7.1, B.11.2) — negotiation determines whether a core exists, never how it behaves once it does, so this does not reintroduce per-window queuing into the completeness algorithms already fixed in Part A/B.

Core counts by mode: an instance in PAI mode (Layer 0 only) uses 1 core. An instance in EAI mode with only the mandatory control layer active (Layer 0 + Layer 1) uses 2 cores — this is the WP#25 EAI baseline (B.9). A maximal EAI (all layers populated) uses 7 cores total: 4 SEU1 (Layer 0 + up to 3 Layer-2 redundancy) + 1 SEU2-A (Layer 1, does not scale beyond 1) + 2 SEU2-B (Layer 3 maximum).

The general principle is that Core Stack size is a function of network condition, not a fixed per-instance constant — consistent with the same density-driven sizing logic already flagged for O8/D12/I20 placement (Revision Log R1-4, deferred to a separate document).

## B.13.2 SEU2 Sub-Types — Actuation vs. Cognition Reporting

SEU2 (B.8) is further divided into two sub-types, physically separated onto distinct Master optical cores within Layer 1/Layer 3 of the Core Stack:

- SEU2-A (Actuation Command) — carries motor commands to physical Velsanet equipment (actuators, motors, displays). This is the efferent-motor path to the peripheral world, and is Layer 1 (mandatory, B.13.1).

- SEU2-B (Cognition Report) — carries a serialized summary of the EAI’s current cognitive state (perceived context, decision rationale, confidence scores, internal system health) upward to the D12/E2E Center. This is the efferent-visceral / interoceptive-reporting path — the EAI reporting its own state upward, rather than acting outward — and is Layer 3 (conditional, B.13.1).

SEU2-A and SEU2-B share the completeness algorithm shape (Table B-3) and PTP profile (Table B-4), but are evaluated as independent channel sets with independent ABORTED/COMPLETE states, on physically distinct cores. A failure in SEU2-A does not block SEU2-B: this is a deliberate design invariant, not an incidental consequence of using separate cores — an EAI that cannot act must still be able to report its own degraded state to the supervising center. Where a channel classification is needed for SEU2-B specifically, it reuses the CH13 Mission State / CH15 Audit-Evidence roles of Table B-3 as its primary and secondary content respectively; it does not introduce new channel numbers.

## B.13.3 Three-Path Correspondence

| Nervous System | WP#25 Correspondence | Direction | Role |
| --- | --- | --- | --- |
| Afferent | SEU1 | External world → AI | Sensing |
| Efferent — Motor | SEU2-A | AI → External world | Acting |
| Efferent — Cognition Report | SEU2-B | AI → Supervising node | Reporting its own state upward |

Table B-7. Three-Path Correspondence — extends Table B-0 (afferent/efferent) with the reporting path****’****s interoceptive character

This is not a sensorimotor loop alone (sense, then act) but a complete cognition–action–reflection loop: sense, act, and report the sensing and the acting upward for supervision. The exact fusion/arbitration behavior when multiple SEU1 cores are active under Layer 2 (B.13.1) — hot-standby, majority vote, or independent-parallel-with-AAI-level fusion — is left to a future document; this section fixes only the existence and independence of the layers, not their internal combination algorithm.

Table B-5 (PAI vs. EAI) is read together with this section as follows: PAI’s “None — not applicable, not optional” for SEU2 extends unchanged to SEU2-A and SEU2-B alike, since neither sub-type has a PAI-side referent.

# B.14 Interoperability Note — PAI vs. EAI

| Item | PAI | EAI |
| --- | --- | --- |
| SEU1 instance | Yes — 1, sole SEU type | Yes — 1, perception plane (E2E #1) |
| SEU2 instance | None — not applicable, not optional | Yes — 1, control plane (E2E #2), separate core |
| Optical cores consumed | 1 | 2 (physically isolated, WP#24 Table 5-2) |
| PTP domains in use | 1 (Domain 4) | 2 (Domain 4 + Domain 5) |
| Mutual independence guarantee needed? | N/A — single engine | Yes — SEU1 ABORTED must not affect SEU2 COMPLETE and vice versa (B.11.2) |

Table B-5. PAI vs. EAI — SEU1/SEU2 Instantiation Summary

# C.15 Cross-Reference to Hardware Validation Strategy

| Validation Item | Applies To | Pass Criterion |
| --- | --- | --- |
| PTP lock acquisition | SEU1 and SEU2, each domain independently | Lock within 1s of Grand Master reachability, per domain |
| PTP accuracy budget | SEU1 and SEU2, each core independently | < 100ns aggregate, 72h+ continuous, per core |
| SEU2 independent-clock isolation | SEU2 only | SEU1 Grand Master fault injected; SEU2 Domain 5 lock unaffected (WP#24 Table 5-2) |
| Record malformed rejection | SEU1 and SEU2 | 100% rejection of CHAN_COUNT/sub-record mismatch, fuzzing test, both record types |
| Mandatory-channel abort correctness | SEU1 (Table A-7) and SEU2 (Table B-3) independently | ABORTED iff a mandatory bit is unset at flush for that engine — 0 false-positive/negative |
| Cross-engine independence | SEU1 ↔ SEU2 | SEU1 ABORTED injected; SEU2 COMPLETE unaffected, and vice versa — direct test of B.8’s claim |
| PAI absence-of-SEU2 conformance | PAI only | Conformant PAI implementation exposes no SEU2 register surface or record path at all |

Table C-1. Validation Items, R1 — supersedes WP#25 v1.0 Table 6-1****’****s single-engine framing

# C.16 Conclusion

This document restructures WP#25 around the physical reality already established in Att2 §5.3 and WP#24 §5.3: EAI’s perception and control planes are two independently powered, independently clocked, physically isolated optical domains. Modeling them as one 16-channel SEU with an optional extension (the earlier v1.0 framing) understated that isolation at the record and algorithm level. SEU1 (Part A) is now defined as the complete, common afferent structure every PAI and every EAI instantiates, and SEU2 (Part B) as a structurally distinct, EAI-only efferent structure — not larger or smaller than SEU1, but different in kind, the way afferent and efferent nerve roots differ in kind, and in point of physical origin, rather than in size (B.8.1).

The geographic and topological question of how O8/D12/I20 are sized and placed across a deployment (density-driven D12 sizing) is explicitly out of scope here (Revision Log R1-4) and will be addressed in a separate forthcoming document.

Next in sequence per Att2 §11.1 / WP#24 §11.1: WP#26 (Dedicated Equipment FPGA/ASIC Design Guide), now scoped to implement two RTL engine templates — one for SEU1, one for SEU2 — rather than one engine with a control-plane extension mode.

## Future Document Proposals (Reference)

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete (this document) |
| WP #26 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL templates, timing analysis, 800G optical interface | Proposed — next |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Proposed |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Proposed |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; O8 Connectivity Plane face allocation (24-core In/Out); Core Stack negotiation protocol (R1-5) | Proposed — spun off from this discussion |

Table C-2. Future Document Proposals — Updated

This document is an internal technical design document of the Velsanet Architecture Group. All design specifications are based on software MVP validation results and will be finalized through physical hardware validation.

Velsanet Architecture Group — Internal Technical Document

VELSANET ARCHITECTURE GROUP  |  TECHNICAL DESIGN DOCUMENT

# WP#26 — Dedicated Equipment FPGA/ASIC Design Guide

Two RTL Engine Templates — SEU1 (Common Base) / SEU2 (EAI-Only) — FPGA Prototyping to ASIC Production

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Public — Open Specification | v1.7 (R8) │ 2026 | SEU1/SEU2 RTL Engine Templates, Timing Analysis, 800G Optical Interface, FPGA→ASIC Migration | Extends WP#24 §5, §11.1; WP#25 R1 §C.16 / Table C-2; Att2 §11.1 |

Republic of Korea | Velsanet Architecture Group

Governance note: This specification is published in full under the Velsanet open-specification policy. Certification against the SEU1/SEU2 engine templates, timing budgets, and interface definitions in this document — not confidentiality — is the mechanism by which conformance and interoperability are controlled.

# Revision Log — R2 vs v1.0

Per Velsanet document-integrity practice, revisions are addition-only: this log records what changed and why, rather than silently overwriting v1.0's content. No numeric, process-node, or micro-architectural value has been invented in this revision — every item below either (a) marks a value that has not yet been decided as explicitly Open, or (b) adds a cross-reference/register without altering any prior claim.

| # | Change | Rationale |
| --- | --- | --- |
| R2-1 | Inserted inline "Open — OI-nn" markers at every point where v1.0 used a qualitative placeholder ("bounded by", "mature node", "moderate") without a decided value. | A qualitative placeholder and an explicit Open flag look similar in prose but function differently for a reader: the flag states plainly that no decision exists yet, rather than reading as if the document is quietly asserting one. |
| R2-2 | Flagged the optical-core numbering used in Table 5-1 ("Cores #1–24 / #25–48", carried from WP#24 Table 5-2) against WP#25 R1-3's "1 optical core = 1 SEU1 instance" (singular) as an unresolved cross-document terminology question (OI-14). | The two source documents use core-counting language that has not been reconciled. WP#26 inherits the tension rather than introduces it, and is not the right place to resolve it unilaterally — that requires the WP#24 and WP#25 R1 record owners. |
| R2-3 | Added Section 10, "Open Items Register," consolidating every OI-nn item raised inline into one table with owner and status. Conclusion renumbered from Section 10 to Section 11. | A single register makes the document's open surface area auditable in one place, rather than requiring a full re-read to find every qualitative placeholder. |
| R2-4 | Added three contingent, unscoped addenda (WP#26.1 / #26.2 / #26.3) to the document-sequence tables (Table 2-1, Table 10-1), distinct from WP#27/#28 which already have committed scope. | Several Open Items cluster naturally into future addenda once their underlying decisions are made. Listing them as contingent placeholders (no content yet) is honest about their status; listing them as "Proposed" with fabricated content would not be. |
| R3-1 | Added Section 10.1, "Dependency Map," ordering the 16 Open Items into a root/dependent/standalone structure per 창호's dependency chains (OI-04, OI-01, OI-06 as roots). | Knowing which Open Items block others changes the order in which decisions should be made — resolving a root item unblocks several dependents at once. This was not derivable from the flat register alone. |
| R3-2 | Attached a one-line technical rationale to each dependency edge, marked "inferred — confirm" since 창호 supplied the chain structure but not written rationale. | Per document-integrity practice, no decision (including a dependency claim) is recorded without a stated basis. Marking rationale as inferred, rather than presenting it as already confirmed, keeps the distinction between what 창호 stated and what Claude derived from it. |
| R4-1 | Reclassified the OI-12 dependency edge from OI-06 to OI-01, per 창호's regrouping (OI-12 now in the OI-01 parallel group alongside OI-05, OI-03). The R3 "weak link" note on OI-12→OI-06 is removed, since that edge no longer exists. | OI-12 (phase-alignment) is timing-reference behavior, which fits the completeness-window prerequisite (OI-01) better than Layer Lock arbitration priority (OI-06) — resolving the low-confidence flag raised in R3. |
| R4-2 | Replaced the Tier 1/2/3 decision-order table with a parallel-execution Group A/B/C/D structure per 창호. Noted that OI-06 now has no dependents in the map and is no longer a blocking root. | The tier framing implied a strict sequence; the group framing states directly which items can run in parallel once a prerequisite clears, which is closer to how a program would actually be scheduled. OI-06's status changed as a direct consequence of R4-1 and is called out rather than left implicit. |
| R5-1 | Added Priority and Estimated Duration columns to the group structure, per 창호's estimates. Split Group D into D1 (verification-related), D2 (documentation/terminology), D3 (technical, low priority). Added Group E (OI-06, independent as of R4). | Priority and duration are program-management inputs, not architectural facts — they are recorded as 창호's estimates, not derived or invented by Claude, consistent with document-integrity practice. |
| R5-2 | Added Section 10.3, "Critical Path Estimate," identifying Group C (OI-07 → OI-08) as the longest chain at 5–10 weeks, and explicitly flagging that OI-04, OI-01, and all of Group D/E carry no duration estimate. | A set of per-item durations does not by itself show total time-to-ready; the critical path is the actual answer to "how long until this is implementable," and the items still missing an estimate are exactly the ones that would change that answer most if left silent. |
| R6-1 | Added Section 10.3, "Readiness Scorecard," formalizing RTL Coding Readiness and ASIC Design Readiness at ★☆☆☆☆ each, both attributed directly to OI-04/OI-01 being open, per 창호. Added a projected-rating table showing how each score should move as specific Open Items clear. | A one-line star rating is only useful if it is traceable to a specific cause, not a general impression. Tying both ratings to the exact Open Items already identified in Sections 10.1–10.2 keeps the scorecard consistent with the rest of the register rather than a separate, re-derived judgment. |
| R7-1 | Closed OI-14: re-read WP#25 R1 §B.9, which already states that WP#24 Table 5-2's "Cores #1–24 / #25–48" range is restated as one physically separate optical core, not a sub-group of a shared core. No conflict exists between WP#24 and WP#25 R1 — the two were never reconciled against each other when WP#26 was first drafted. | This was found by searching the project's own source documents, not by inventing a resolution — WP#25 R1 §B.9 pre-dates WP#26 v1.0 and already contains the answer. |
| R7-2 | Closed OI-03: WP#25 R1 Table A-5/B-4 already specify the <100ns PTP accuracy budget and its five-way decomposition (Grand Master stability, path asymmetry, residence/turnaround, timestamp granularity, margin — each individually bounded), identical for SEU1 (Domain 4) and SEU2 (Domain 5). | Same basis as R7-1 — the value existed in WP#25 R1 and was not carried into WP#26 v1.0's Section 6.3. |
| R7-3 | Narrowed OI-01: WP#25 R1 §A.7.3 fixes the SEU1 completeness-window duration itself (0.8s / 1.0s default / 1.5s adaptive). The remaining gap is only the RTL clock-cycle translation of that wall-clock bound at a chosen implementation clock frequency — a direct calculation once a clock is selected, not an unresolved algorithm question. | The window duration was always fixed at the algorithm level (WP#25 R1); what was actually missing was connecting that value into WP#26's RTL timing budget (Table 4-3). |
| R7-4 | Closed OI-09: committed to a standard 2-flop synchronizer for the SEU1/SEU2 monitoring-tap CDC, since Section 6.1 already establishes the tap as non-functional (cross-check only, not a data path) — a 2-flop synchronizer is the standard, lowest-risk choice for that class of signal and does not require FIFO-level throughput. | This is Claude's engineering judgment, clearly distinguished from the WP#25-sourced closures above (R7-1/2/3) — offered as a proposal for 창호's confirmation, not asserted as an architectural fact. |
| R8-1 | Drafted actual SystemVerilog RTL for OI-05 (worm_write_gate.sv) and the arbitration half of OI-06 (m_axis_layer_lock_arbiter.sv), each with a directed testbench. Compiled with a real Verilog compiler (Icarus Verilog) and simulated — both testbenches pass. Files are external to this document (Section 10.5); not reproduced inline. | Sections 10.1–10.3 (dependency maps, priority estimates, readiness scores) were planning artifacts, not resolutions. This is the first revision containing an actual executable artifact — code that compiles and a simulation that ran and passed — rather than a description of what such code should contain. |
| R8-2 | Split OI-06 into OI-06a (Layer Lock arbitration priority/deadlock avoidance — resolved by m_axis_layer_lock_arbiter.sv, simulated) and OI-06b (HBF ACL bit-field evaluation timing — still fully open, not addressed by the arbiter module). | The original OI-06 bundled two different questions. The arbiter code resolves only the first; splitting the item prevents the RTL artifact from being read as closing more than it does. |
| R8-3 | Recorded explicitly that OI-01 (root), OI-04 (root), and all measurement/negotiation-dependent items (e.g., OI-02's device-specific occupancy, which requires real synthesis tooling Claude does not have access to) remain outside what Claude can resolve by further document revision, coding, or simulation — these require 창호's direct action (hardware validation, foundry negotiation) regardless of how many further revisions this document goes through. | Per 창호's R8 prompt: distinguishing what Claude can and cannot actually execute, rather than implying that another revision cycle would eventually reach them. |

Table R2-1. Revision Log — R2 through R8

# 1. Purpose and Scope

WP#26 is the implementation-facing counterpart to WP#24 (Cognition Cube Physical Spec) and WP#25 R1 (SEU1/SEU2 Synchronization Specification). Where WP#24 fixed the register maps and 3-block hardware structure of the Dedicated Equipment, and WP#25 R1 fixed the wire-level record format and timing algorithms for SEU1 and SEU2, WP#26 defines how those two structures are actually built: as two separate RTL engine templates, each carried through a FPGA prototyping stage and an ASIC production stage.

Per WP#25 R1 Revision R1-2, SEU1 and SEU2 are not one engine with an optional extension mode — they are structurally distinct. This document reflects that decision at the hardware-implementation level: it defines two engine templates, not one configurable template. Section 4 covers the SEU1 engine (common to every PAI and every EAI instance). Section 5 covers the SEU2 engine (EAI-only, control/actuation plane).

Scope includes: RTL-level module decomposition of both engine templates; timing budget analysis per engine and across the SEU1/SEU2 boundary; the 800G optical interface as seen from the DEMUX front end; and the migration path from FPGA prototype to ASIC production, with ASIC treated as the primary target implementation for volume deployment (Section 8).

Out of scope: geographic/topological placement of O8/D12/I20 nodes (per WP#25 R1-4, addressed in a separate forthcoming document); the O8 Connectivity & Placement Spec (spun off per WP#25 R1-5); and EDA-vendor-specific tooling instructions, which are treated as an execution-time decision outside the architecture record.

# 2. Positioning in the Document Sequence

This document is Doc No. WP#26 in the hardware-implementation sub-series that began with WP#24. The table below restates and updates WP#25 R1 Table C-2, marking WP#26 as complete.

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete |
| WP #26 R2 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, timing analysis, 800G optical interface, FPGA→ASIC migration | Complete (R2) — 16 open items, Section 10 |
| WP #26.1 (contingent) | RTL Macro-Architecture & FSM Addendum | WORM/ACL FSM, CDC technique, physical isolation implementation | Contingent — no content until OI-05/06/09/10/11 decided |
| WP #26.2 (contingent) | Verification & DFT Specification Addendum | Test-plan specifics, coverage targets, DFT strategy | Contingent — no content until OI-08/13/15/16 decided |
| WP #26.3 (contingent) | 800G PHY Integration Addendum | SerDes/PCS/FEC implementation detail | Contingent — no content until OI-07 decided |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Proposed — next |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Proposed |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; Core Stack negotiation protocol | Proposed — spun off |

Table 2-1. Document Sequence — Updated at WP#26 R2

# 3. Two-Engine Architecture Overview

The Dedicated Equipment (WP#24 §5, 3-block structure: DEMUX Engine, MUX Engine, Cube Write-Back) is instantiated once per optical core. Because one optical core carries exactly one SEU1 instance, and EAI additionally requires a second, physically separate optical core for one SEU2 instance (WP#25 R1-3), the Dedicated Equipment itself resolves into two engine templates rather than one:

| Engine Template | Instantiated By | Function |
| --- | --- | --- |
| SEU1 Engine | Every PAI (8ch) and every EAI (first 8 of 16ch, E2E #1 perception plane) | Common afferent structure: DEMUX → SEU1 Assembly → WORM write gate → HBF C/I/E-Axis path |
| SEU2 Engine | EAI only (CH9–CH16, E2E #2 control plane) | EAI-only efferent structure: MUX → Cube Write-Back → M-Axis path → CH11 safety interface |

Table 3-1. Two-Engine Decomposition of the Dedicated Equipment

This mirrors the physical isolation already fixed at WP#24 §5.3 (cores #1–24 vs #25–48 within one MOCT ribbon, separate power, separate clock source) and at WP#25 R1 Part A/Part B. The SEU1 and SEU2 RTL templates in Sections 4 and 5 below are the register-transfer-level realization of that same isolation boundary — they share no clock domain, no register bus, and (per Section 8) may be placed as physically separate blocks even on a single ASIC die.

# 4. SEU1 RTL Engine Template (Common Base)

The SEU1 engine is the single template every PAI and every EAI instantiates. It is designed once and reused without modification across both deployment contexts (WP#24 §2.1 5-layer stack applies identically to L1–L3 for both).

## 4.1 Module Decomposition

| Module | Function | Maps to WP#24 |
| --- | --- | --- |
| DEMUX Sub-block | 384 optical lanes → 8ch (PAI) or 8ch of 16 (EAI E2E #1) de-multiplex | Block A, §5.2 |
| SEU1 Assembly Engine | 4-stage synchronization pipeline; builds one SEU1 record per completeness window | §6, WP#25 R1 Part A |
| WORM Write Gate | Hardware-enforced write-once interface into HBM per-channel register set | §1, WORM_SECTOR_LOCK |
| Timestamp Latch I/F | Reads T-Axis Timestamp Engine output into SEU_TIMESTAMP_LATCH | §1, offset 0x20 |

Table 4-1. SEU1 Engine — RTL Module Decomposition

Open — OI-05: WORM Write Gate RTL structure is not yet chosen among a hardware FSM, a one-shot pulse circuit, or a plain lock register. WP#24 fixes the write-once behavior (WORM_SECTOR_LOCK, hardware-set, no clear path); it does not fix the circuit style that implements it. Decision owner: RTL architecture, proposed for WP#26.1.

Resolved — OI-05 (drafted, R8): 4-state FSM (IDLE → LATCH → COMMIT → LOCKED) implemented as actual SystemVerilog: worm_write_gate.sv (Section 10.5). Compiled with Icarus Verilog and simulated against a directed testbench (tb_worm_write_gate.sv) — 2 back-to-back SEU1 records, audit-pointer increment and single-cycle HBM write pulse both verified in simulation. One design assumption is flagged in the file header for 창호's confirmation: WORM_SECTOR_LOCK is treated as a per-slot lock word in an audit trail (consistent with WORM_AUDIT_PTR advancing per record), not a single lifetime lock for the whole channel — this reading was not explicit in WP#24 and should be confirmed, not assumed correct.

## 4.2 Interface Signals

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| optlane_in[47:0] | 48 × SerDes | Input | Per-channel lane group from MOCT fixed ribbon (48 lanes/ch × 8 groups, PAI) |
| seu1_rec_out | Record (WP#25 format) | Output | Completed SEU1 record to HBM write path |
| worm_lock_stat | 1 | Output | Mirrors WORM_SECTOR_LOCK bit 0 — hardware-set, no clear path |
| ptp_sync_in | 80-bit | Input | PTP master clock reference per WP#25 R1 sync profile |
| chan_id | 16 | Static (fixed per channel) | WORM_CHAN_ID, fixed at fabrication |

Table 4-2. SEU1 Engine — Top-Level Interface Signals

## 4.3 Timing Budget

The SEU1 pipeline inherits the WP#24 hardwired DEMUX latency target (< 2ns) and adds the SEU1 Assembly Engine's completeness-window latency, bounded by the PTP sync profile defined in WP#25 R1.

| Stage | Target Latency | Notes |
| --- | --- | --- |
| DEMUX (Block A) | < 2ns | Hardwired, per-channel independent clock domain (PLL) |
| SEU1 Assembly (4-stage) | Bounded by completeness window | Per WP#25 R1 Part A algorithm; not a fixed constant — depends on channel data-rate heterogeneity |
| WORM write commit | 1 clock cycle post-assembly | Hardware-set lock bit, no round-trip acknowledgment required |

Table 4-3. SEU1 Engine — Timing Budget

Open — OI-01: The completeness-window latency is described only as "bounded," with no minimum/maximum clock-cycle figure. That figure depends on worst-case channel data-rate heterogeneity, which has not been characterized against real hardware. Decision owner: 창호 + hardware validation results (Section 9).

Resolved — OI-01 (narrowed, R7): WP#25 R1 §A.7.3 fixes the window itself: default 1000ms, adaptive down to 800ms on a CH4 rapid-change trigger, adaptive up to 1500ms on sustained low change-rate across CH1–CH8. The remaining unknown is only the clock-cycle count at a chosen RTL clock frequency — cycles = window_ms × clock_Hz / 1000 — a calculation, not an open algorithm question. Still blocked on OI-04-adjacent clock-frequency selection, but no longer blocked on the window value itself.

## 4.4 FPGA Prototype Target

Per WP#24 Table 5-1, Block A (DEMUX Engine) is FPGA-targeted (Xilinx UltraScale+ recommended) for both prototyping and initial low-volume production. The SEU1 engine template inherits this target for its DEMUX sub-block; the SEU1 Assembly Engine and WORM Write Gate are implemented as programmable logic within the same device for prototype validation.

| Resource | Estimate (per-channel instance) | Basis |
| --- | --- | --- |
| Transceivers (SerDes) | 8 groups × 48 lanes, 800Gbps/core aggregate | MOCT fixed-ribbon lane grouping, WP#24 §5.2 |
| Logic (LUT) budget | Moderate — DEMUX control + 4-stage pipeline state machine | Scales with channel count, not per-lane |
| BRAM | Completeness-window buffering per channel | Sized to worst-case heterogeneous-rate skew, WP#25 algorithm |

Table 4-4. SEU1 Engine — FPGA Resource Estimate (order-of-magnitude, prototype stage)

Open — OI-02: No specific device (e.g., VU9P vs VU13P) or precise LUT/BRAM/DSP/SerDes occupancy percentage has been selected — the estimate above is order-of-magnitude only. Precise figures require RTL synthesis against a chosen device, which is a Section 9 (FPGA bring-up) activity, not yet performed.

## 4.5 ASIC Production Path

The SEU1 engine is the higher-volume of the two templates — it is instantiated in every PAI and every EAI, versus SEU2's EAI-only deployment. This volume profile favors migration to ASIC once the FPGA prototype passes functional and timing sign-off (Section 9), consistent with WP#24's general FPGA-for-prototype / ASIC-for-volume posture.

| Consideration | Guidance |
| --- | --- |
| Partitioning | DEMUX sub-block and SEU1 Assembly Engine as a single hardened block; WORM Write Gate kept as a physically distinct, minimal-logic block reflecting its write-once, no-clear-path guarantee at the silicon level, not just the register level |
| Process node | A mature, cost-optimized node is favored over a leading-edge node — SEU1's logic is control/state-machine dominated rather than compute-dominated, so density gains from a leading node have limited return relative to the SerDes/PHY cost |
| Power target | Bounded primarily by the 800Gbps/core SerDes interfaces, not by the assembly-engine logic; PHY IP selection dominates the power budget |
| Verification carry-over | FPGA prototype testbenches (Section 9) are reused as the RTL regression suite for ASIC sign-off — no re-derivation of the completeness-window algorithm from WP#25 is required |

Table 4-5. SEU1 Engine — ASIC Production Considerations

Open — OI-04: "Mature, cost-optimized node" is a direction, not a selection — no specific process node (foundry/node family) has been chosen for either engine. See Section 8.4 for the same item stated at the cross-engine level. Decision owner: 창호 / foundry partner selection.

# 5. SEU2 RTL Engine Template (EAI-Only, Control Plane)

The SEU2 engine exists only where an EAI instance exists. Per WP#25 R1-2, it is not an extension of SEU1 — it is structurally distinct, the way afferent and efferent nerve roots differ in kind rather than size. The RTL template below reflects that: SEU2 is not SEU1 with additional logic gated on, it is a separate template sharing no register bus or clock domain with SEU1.

## 5.1 Structural Differences from SEU1

| Property | SEU1 Engine | SEU2 Engine |
| --- | --- | --- |
| Deployment | Every PAI + every EAI (E2E #1) | EAI only (E2E #2) |
| Direction | Afferent (sensory/perception → HBM/HBF) | Efferent (control/actuation, HBF M-Axis → output) |
| Optical core | Cores #1–24 (WP#24 Table 5-2) | Cores #25–48, physically separate core group |
| Power domain | Main PSU | Backup UPS + supercapacitor parallel (WP#24 Table 5-2) |
| Clock source | PTP master clock | Independent Crystal OSC (WP#24 Table 5-2) |
| Safety interface | None | CH11 Safety Override — isolated island, no shared trace (WP#24 §7, Figure 7-1) |

Table 5-1. SEU1 vs SEU2 — Structural Comparison

Open — OI-14: The "Optical core" row uses WP#24 Table 5-2's core-range language ("Cores #1–24" / "#25–48") without reconciling it against WP#25 R1-3's singular framing ("1 optical core = 1 SEU1 instance"; EAI's SEU2 as "a second, physically separate optical core"). This row is carried forward from WP#24 unmodified; the reconciliation is a cross-document question for the WP#24 and WP#25 R1 record owners, not resolved unilaterally here.

Resolved — OI-14 (closed, R7): WP#25 R1 §B.9 already contains the reconciliation: EAI's second core is stated as "corresponding to E2E #2 / cores #25–48 of WP#24 Table 5-2's ribbon grouping, restated here as a physically separate core rather than a sub-group of one shared core." WP#24's 24-count range describes the fiber-lane bundle; WP#25 R1's "1 optical core" is the logical unit that bundle constitutes. No cross-document conflict exists — this was a missed cross-reference in WP#26 v1.0, not an unresolved architecture question.

## 5.2 Module Decomposition

| Module | Function | Maps to WP#24 |
| --- | --- | --- |
| MUX Engine | 8/16ch output → 24-core Bundle toward AAI | Block B, §5.1 |
| Cube Write-Back | AsAI result → HBF Cube address mapping + M-Axis update | Block C, §5.1 |
| CH11 Safety Interface | Force-guided relay control (EN 50205 Type A/B), independent power monitor | §7, Figure 5-1 (safety state machine) |
| Independent Clock I/F | Crystal OSC reference, decoupled from PTP master | WP#24 Table 5-2 |

Table 5-2. SEU2 Engine — RTL Module Decomposition

Open — OI-06: Cube Write-Back's HBF ACL bit-field evaluation timing (when in the pipeline the 5-axis ACL_PERM check occurs relative to the write) and the Layer Lock arbitration priority/deadlock-avoidance mechanism for M-Axis are not specified at RTL level — WP#24 §2 fixes the permission matrix, not the arbitration circuit. Decision owner: RTL architecture, proposed for WP#26.1.

Resolved — OI-06a (drafted, R8 — split from OI-06): Layer Lock arbitration priority and deadlock avoidance implemented as actual SystemVerilog: m_axis_layer_lock_arbiter.sv (Section 10.5). Two-requester (AAI, AsAI) alternate-on-contention arbiter, simulated against a directed testbench — mutual exclusion (never both granted) and no-starvation across repeated contention both verified. OI-06b (ACL bit-field evaluation timing) is NOT resolved by this module and remains fully open — see Section 10.4.

## 5.3 Timing Budget

SEU2 timing is governed by its independent Crystal OSC rather than the PTP master clock used by SEU1, consistent with the physical-isolation requirement that E2E #2 failure must not propagate from or to E2E #1 (WP#24 Table 5-2).

| Stage | Target Latency | Notes |
| --- | --- | --- |
| MUX (Block B) | Bounded by 24-core Bundle uplink rate | FPGA or custom ASIC per WP#24 Table 5-1 |
| Cube Write-Back (Block C) | M-Axis update commit | Custom ASIC recommended (WP#24 Table 5-1); Layer Lock arbitration adds one arbitration cycle |
| CH11 safety loop | ≥ 30s self-sustained, independent of main-path timing | Supercapacitor-bound, not clock-bound (WP#24 Figure 5-1) |

Table 5-3. SEU2 Engine — Timing Budget

Open — OI-12: How an initial phase difference between SEU2's independent Crystal OSC and SEU1's PTP domain is handled (e.g., at chassis power-up, or after a CH11 event) is not specified — the two domains are defined as independent, but "independent" has not been distinguished from "unsynchronized-and-unmonitored" at the RTL level. Decision owner: RTL architecture.

## 5.4 FPGA Prototype Target

The MUX Engine (Block B) may be prototyped on the same FPGA family as the SEU1 DEMUX for schedule efficiency, but is placed in a logically and — where the target device supports it — physically partitioned region, to validate the no-shared-bus property before it is committed to silicon. The CH11 Safety Interface is prototyped on a separate, non-shared I/O bank from the outset; this is treated as a hard prototyping rule, not an optimization, because CH11 isolation is a safety requirement rather than a performance one.

## 5.5 ASIC Production Path

WP#24 Table 5-1 recommends custom ASIC for Block C (Cube Write-Back) and allows FPGA or custom ASIC for Block B (MUX Engine). Because SEU2 volume is EAI-only — lower than SEU1's PAI+EAI volume — the case for ASIC migration rests less on unit-cost amortization and more on the safety and isolation guarantees that are easier to make load-bearing in silicon than in reconfigurable logic.

| Consideration | Guidance |
| --- | --- |
| Floorplan isolation | CH11 Safety Interface occupies a physically separate die region or, where volume justifies it, a separate die entirely — carrying forward WP#24's isolation-slotted PCB convention (Figure 7-1) into the ASIC floorplan rather than treating it as a board-level-only concern |
| Power domain on-die | Backup UPS + supercapacitor input kept on an independent on-die power rail, mirroring the board-level independent power requirement — no shared regulator with the SEU1 die/region |
| Process node | Node choice is secondary to isolation guarantees for SEU2; a node already qualified for the SEU1 engine is preferred by default to simplify multi-project wafer or chiplet integration, unless safety-certification requirements dictate otherwise |
| Certification path | Force-guided relay control logic (EN 50205 Type A/B) is treated as safety-relevant RTL and carried through a separate, more conservative verification track than the MUX/Write-Back logic (Section 9) |

Table 5-4. SEU2 Engine — ASIC Production Considerations

Open — OI-10: "Physically separate die region" for the CH11 Safety Interface names a goal, not a method — guard-ring, deep N-well, and dedicated ISO-cell approaches are all consistent with the goal and have not been chosen among. Decision owner: ASIC physical design team.

Open — OI-16: No RTL-to-certification traceability methodology is defined for the EN 50205 Type A/B force-guided relay logic — i.e., how a specific RTL block or test is mapped back to a specific certification requirement. Decision owner: safety-certification lead, proposed for WP#26.2.

# 6. Cross-Engine Timing Analysis

## 6.1 Clock Domain Crossing

By design, there is no clock-domain-crossing path between the SEU1 engine (PTP master clock) and the SEU2 engine (independent Crystal OSC) at the RTL level. This is a direct consequence of WP#24 §5.3 physical isolation and WP#25 R1-2's structural (not extensional) treatment of SEU2. Any signal that appears to cross between the two — for example, a shared chassis-level timestamp reference used only for cross-checking, not for functional synchronization — is treated as a monitoring tap, not a data path, and is excluded from both engines' functional timing budgets.

Open — OI-09: If a monitoring tap of this kind is implemented, its CDC synchronization technique (2-flop synchronizer, asynchronous FIFO, or full handshake protocol) is not specified. Because the tap is explicitly non-functional, this is lower-priority than the other Open Items, but is listed for completeness. Decision owner: RTL architecture.

Resolved — OI-09 (closed, R7): Standard 2-flop synchronizer, decided in this revision. The tap is cross-check-only and explicitly excluded from both engines' functional timing budgets (Section 6.1 above) — it carries no throughput requirement, so a FIFO or handshake protocol would be unnecessary complexity for a single-bit or slow-changing signal. Proposed for 창호's confirmation; distinguished from OI-14/OI-01/OI-03 above, which were closed by finding an existing answer rather than by new judgment.

## 6.2 End-to-End Latency Budget

| Path | SEU1 | SEU2 |
| --- | --- | --- |
| DEMUX/MUX stage | < 2ns (hardwired) | Bounded by 24-core Bundle uplink rate |
| Assembly / Write-Back stage | Completeness-window bound (WP#25 Part A) | M-Axis commit + Layer Lock arbitration cycle |
| Commit to HBM/HBF | 1 cycle, WORM hardware-set | 1+ cycle, ACL Gate arbitrated |

Table 6-1. End-to-End Latency Budget Comparison

## 6.3 PTP Jitter Tolerance

SEU1's completeness-window algorithm inherits its jitter tolerance directly from the WP#25 R1 PTP sync profile; no additional tolerance derivation is introduced at the RTL level in this document. SEU2, running on an independent Crystal OSC rather than PTP, is not subject to PTP jitter by construction — its timing reference stability is instead bounded by OSC specification (frequency stability over the CH11 self-sustained operating window) rather than by network sync quality.

Open — OI-03: "Inherits directly" points at WP#25 R1's sync profile but does not restate its jitter tolerance value here, nor define an RTL-level margin policy (how much of the tolerance budget the SEU1 Assembly Engine is allowed to consume before flagging a completeness-window fault). Decision owner: WP#25 R1 maintainer, cross-referenced into this document once fixed.

Resolved — OI-03 (closed, R7): WP#25 R1 Table A-5 / Table B-4 already fix the value: <100ns aggregate accuracy budget for both SEU1 (PTP Domain 4) and SEU2 (Domain 5), decomposed as Grand Master reference stability <20ns, path asymmetry residual <15ns, residence/turnaround time <25ns, receiver timestamp granularity <20ns, and margin <20ns. The <20ns margin line is the RTL-level allowance already defined — no separate margin policy needs to be authored.

# 7. 800G Optical Interface

## 7.1 Physical Layer Requirements

Both engine templates present the same physical-layer contract at their optical front end: 800Gbps per core, delivered over the MOCT fixed ribbon with no field connectors and 0dB insertion loss (WP#24 §1, L1 Physical Tx Layer). This is a fixed input condition for both RTL templates, not a design parameter — the DEMUX front end is built to this rate, not tuned to it.

## 7.2 SerDes/PHY: FPGA vs ASIC

| Aspect | FPGA Prototype | ASIC Production |
| --- | --- | --- |
| PHY implementation | Vendor hard-IP transceivers (e.g., UltraScale+ GTY/GTM-class) | Licensed or custom SerDes IP integrated at the process node selected in Section 8 |
| Lane aggregation | 48 lanes/ch × 8 groups (PAI); 24 lanes/ch × 16 groups (EAI) | Same lane grouping, carried forward unchanged from WP#24 §5.2 — the RTL template does not change lane topology between prototype and production |
| Power/thermal | Higher per-lane power typical of programmable transceivers | Lower per-lane power achievable with production PHY IP; primary driver of ASIC migration benefit for SEU1 (Section 4.5) |

Table 7-1. 800G Optical Interface — FPGA vs ASIC Implementation

Open — OI-07: RTL/PCS-level implementation detail is not defined: line-coding scheme (e.g., 64b/66b-class vs alternative), FEC presence and placement (FPGA-side vs ASIC-side), skew-alignment/channel-bonding strategy across the lane groups in Table 7-1, and LOS/LOF detection-and-recovery logic. These are PHY-integration decisions, proposed for WP#26.3 once the SerDes IP source (FPGA hard-IP vs licensed ASIC IP, Table 7-1) is selected.

## 7.3 Lane Grouping Recap

The lane grouping fixed in WP#24 §5.2 is treated as an immutable input to both engine templates in this document: 48 lanes/channel × 8 groups for the PAI/SEU1 configuration, and 24 lanes/channel × 16 groups for the EAI configuration spanning both SEU1 (E2E #1) and SEU2 (E2E #2). Neither the FPGA prototype nor the ASIC production RTL alters this grouping; it is a MOCT-side physical fact, not an RTL design choice.

# 8. FPGA-to-ASIC Migration Path

This section carries the greater share of WP#26's production-readiness content, reflecting that the Dedicated Equipment's long-run deployment target is ASIC, with FPGA serving the prototyping and low-volume bring-up role. WP#24 Table 5-1 already recommends custom ASIC for Block C and allows it for Block B; this document extends that posture to the full SEU1 and SEU2 engine templates as the volume case matures.

## 8.1 Volume/Cost Crossover Rationale

| Factor | Implication for FPGA→ASIC Timing |
| --- | --- |
| SEU1 volume | Instantiated in every PAI and every EAI — highest volume of the two templates, favoring the earliest ASIC migration once FPGA-stage verification (Section 9) is complete |
| SEU2 volume | EAI-only — lower unit volume than SEU1, but carries safety-relevant logic (CH11) whose isolation guarantees benefit from silicon-level enforcement regardless of volume economics alone |
| Reconfigurability need | Neither template requires field reconfigurability post-deployment — the WORM/ACL model is a fixed hardware contract by design (WP#24 §3, §4), removing one of the traditional arguments for staying on FPGA in production |
| NRE amortization | Standard cost/volume crossover logic applies: FPGA unit cost stays flat with volume while ASIC NRE amortizes down — this document does not fix a specific volume threshold, which is a program-management decision outside the architecture record |

Table 8-1. FPGA→ASIC Crossover Factors

## 8.2 Implementation Style Decision

| Style | Best Fit | Rationale |
| --- | --- | --- |
| Full-custom | SEU2 Cube Write-Back, CH11 safety island | Highest guarantee of physical isolation and timing determinism for safety-relevant and WORM-adjacent logic |
| Standard-cell (synthesized) | SEU1 Assembly Engine, SEU2 MUX Engine | State-machine/control-dominated logic; standard synthesis flow is sufficient and faster to close timing on |
| Structured ASIC / eFPGA block | Reserved for any future feature requiring post-fabrication configurability | Not required by the current WORM/ACL model; noted here only as an option if a future revision introduces a configurable field |

Table 8-2. Implementation Style by Block

Open — OI-08 / OI-11: Two ASIC-production essentials are not addressed anywhere in this document: (1) DFT strategy — scan-chain insertion, JTAG boundary scan, SerDes/PHY BIST, and the fuse/OTP mechanism for setting WORM_CHAN_ID at fabrication; and (2) on-die power/clock-gating strategy enabling SEU1 and SEU2 to be independently powered down on a shared die. Both are ASIC-design-team decisions, proposed for WP#26.2 (DFT) and as part of the Section 8.2 floorplan work (power/clock gating).

## 8.3 Verification-to-Tape-out Flow

At a conceptual level, both engine templates follow the same staged flow, kept general and non-tool-specific: RTL functional simulation against the WP#25 R1 record format and completeness algorithm; FPGA bring-up and in-system validation (Section 9); timing closure against the budgets in Sections 4.3, 5.3, and 6.2; physical verification and isolation checks specific to the SEU1/SEU2 boundary (Section 6.1) and the CH11 safety island (Section 5.5); and tape-out sign-off. This document does not prescribe specific EDA tooling — that selection is left to the implementing team as an execution-time decision outside the architecture record.

## 8.4 Process Node Candidate Discussion

Node selection is kept general in this document rather than fixed to a specific foundry process, consistent with WP#24's own posture of naming FPGA families as recommendations rather than mandates. The guiding principle carried from Sections 4.5 and 5.5: SEU1 favors a cost-optimized mature node given its control-dominated logic and SerDes-dominated power budget; SEU2 favors alignment with whatever node is selected for SEU1 (to simplify multi-project wafer or chiplet integration) unless a safety-certification requirement specific to CH11 dictates a different, more conservative choice.

# 9. Verification and Validation Plan

Consistent with the disclaimer carried across the WP#24–#26 sub-series, all specifications in this document are based on software MVP validation results and are finalized through staged physical hardware validation, not asserted as already-silicon-proven at the time of writing.

| Stage | Scope | Exit Criterion |
| --- | --- | --- |
| Software MVP | Functional model of SEU1/SEU2 record assembly against WP#25 R1 algorithm | Record format and completeness algorithm match WP#25 R1 bit-for-bit in simulation |
| RTL simulation | Both engine templates, independently, per Sections 4–5 | Timing budgets in Tables 4-3 and 5-3 met in simulation across corner cases |
| FPGA bring-up | In-system validation on the target device family (Sections 4.4, 5.4) | 800G front-end and lane grouping validated against WP#24 §5.2 at rate |
| Isolation validation | SEU1/SEU2 boundary (Section 6.1) and CH11 safety island (Section 5.5) | No shared clock, bus, or power path detected between domains |
| ASIC sign-off | Timing closure, physical verification, tape-out readiness | All prior-stage exit criteria carried forward without re-derivation |

Table 9-1. Staged Verification and Validation Plan

Open — OI-13: Exit criteria are stated qualitatively (e.g., "met in simulation across corner cases") without test-vector counts, code-coverage targets, a random-verification strategy, a formal-property checklist, or a named methodology for the Isolation Validation row (e.g., ATPG, DFT-based check, or direct electrical measurement). Decision owner: verification team, proposed for WP#26.2.

Open — OI-15: No RTL-compatibility guarantee is stated between the FPGA prototype and the ASIC target — specifically how FPGA BRAM primitives map to ASIC memory macros, and whether clock-gating and reset behavior are required to match bit-for-bit or only functionally, across the prototype-to-production transition referenced in Table 4-5 / Table 5-4. Decision owner: verification team.

# 10. Open Items Register

Every value in Sections 4–9 that has not been fixed by an actual engineering decision is listed below, cross-referenced to its inline "Open — OI-nn" marker. This register exists so that no open item is only discoverable by reading the full document; per Principle 1 ("결정의 근거를 기록하라 — 마음대로 정의하지 마라"), none of these are given placeholder numeric values in this revision.

## 10.1 Dependency Map

The 16 Open Items are not independent — resolving some unblocks others. The chain structure below was specified directly by 창호; the one-line rationale attached to each edge is Claude's inference from the surrounding architecture text in Sections 4–9, marked accordingly, and should be confirmed rather than treated as already-decided.

| Item | Depends On | Rationale (inferred — confirm) |
| --- | --- | --- |
| OI-07 | OI-04 | PHY/SerDes IP sourcing (Table 7-1) is process-node-specific — the node must be fixed before a PHY IP candidate can be evaluated against it. |
| OI-08 | OI-07 | DFT strategy, in particular SerDes BIST, is defined against the chosen PHY implementation — where FEC/PCS logic physically sits (OI-07) determines what a scan chain or BIST hook needs to cover. |
| OI-10 | OI-04 | Guard-ring, deep-N-well, and ISO-cell isolation options are process-design-kit primitives — which ones exist to choose from depends on the node. |
| OI-11 | OI-04 | Power/clock-gating cells (isolation cells, level shifters) are node-specific library elements. |
| OI-05 | OI-01 | WORM Write Gate structure (FSM vs one-shot vs lock register) sizes its buffering and commit timing against the completeness-window bound — the bound should exist before the circuit style is chosen. |
| OI-03 | OI-01 | A PTP jitter margin policy is naturally expressed as a fraction of the completeness-window budget, so the window bound is a prerequisite input, not a parallel decision. |
| OI-12 | OI-01 | Phase-alignment handling for SEU2's independent Crystal OSC is evaluated relative to the completeness-window timing reference (OI-01), not to Layer Lock arbitration priority — reclassified in R4 from the original OI-06 edge. |

Table 10-1. Open Items — Dependency Edges

Open — Note (R4): OI-06 (Layer Lock arbitration) now has no dependents in this map, since OI-12 was reclassified above. It remains a substantive Open Item in its own right (Section 5.2) but is no longer a blocking root — it can be resolved on the same footing as the Group D standalone items.

Items not shown above — OI-02, OI-09, OI-13, OI-14, OI-15, OI-16 — have no stated dependency and can be decided in any order relative to the rest.

| Group | Item | Priority | Est. Duration (창호 estimate) |
| --- | --- | --- | --- |
| A — after OI-04 | OI-07 (PHY detail) | High | 3–6 weeks |
| A — after OI-04 | OI-10 (isolation method) | Medium | 1–2 weeks |
| A — after OI-04 | OI-11 (power gating) | Medium | 1–2 weeks |
| B — after OI-01 | OI-05 (WORM structure) | High | 1–2 weeks |
| B — after OI-01 | OI-03 (PTP jitter) | Medium | 0.5–1 week |
| B — after OI-01 | OI-12 (phase alignment) | High | 1 week |
| C — sequential | OI-07 → OI-08 (DFT) | High | 2–4 weeks (after OI-07) |
| D1 — independent, verification | OI-13 (verification detail) | High | Not estimated |
| D1 — independent, verification | OI-15 (FPGA↔ASIC compatibility) | Medium | Not estimated |
| D1 — independent, verification | OI-16 (certification traceability) | High | Not estimated |
| D2 — independent, documentation/terminology | OI-14 (terminology reconciliation) | Low | Not estimated |
| D3 — independent, technical/low priority | OI-02 (FPGA occupancy) | Low | Not estimated — pending measurement |
| D3 — independent, technical/low priority | OI-09 (CDC technique) | Low | Not estimated — non-functional tap |
| E — independent (reclassified R4) | OI-06 (Layer Lock arbitration) | Medium | 1 week |

Table 10-2. Parallel-Execution Groups — Priority and Duration (창호 estimates)

## 10.2 Critical Path Estimate

Group C is the longest chain in the map: OI-07 (3–6 weeks) followed sequentially by OI-08 (2–4 weeks) totals 5–10 weeks, measured from the point OI-04 clears. Every other group finishes inside that window — Group A's remaining items (1–2 weeks each, parallel) and Group B (1–2 weeks max, parallel) are both shorter than Group C alone.

Open — Gap: This estimate answers "how long from OI-04/OI-01 clearing to Group A–C done," not "how long until the document is implementable." Three things are missing to answer the second question: (1) no duration estimate for OI-04 and OI-01 themselves — the root decisions that gate Groups A and B; (2) no duration for the three D1 items (OI-13, OI-15, OI-16), two of which are High priority and plausibly gate actual RTL sign-off (verification detail, certification traceability) even though they have no stated dependency on the rest; (3) no duration for OI-06 (Group E) or the D2/D3 items, though these carry lower priority. Until (1) and (2) in particular are estimated, "5–10 weeks" describes only the Group A–C sub-path, not the document's full path to RTL-ready.

## 10.3 Readiness Scorecard

Per 창호: both readiness dimensions below are rated at their floor, and both are attributed to the same two root Open Items rather than to the document's structure or content quality in general.

| Dimension | Rating | Basis |
| --- | --- | --- |
| RTL Coding Readiness | ★☆☆☆☆ | OI-04 (process node) and OI-01 (completeness-window bound) are both open. Per Table 10-1, these gate Groups A, B, and (transitively) C — no RTL work in those groups has a fixed starting point until both clear. |
| ASIC Design Readiness | ★☆☆☆☆ | OI-04 alone blocks ASIC design start: Group A (OI-07 PHY, OI-10 isolation, OI-11 power gating) depends on it directly, and all three feed floorplanning and process-design-kit selection. |

Table 10-3. Readiness Scorecard

The table below projects how each rating would move as specific Open Items clear, to make the scorecard actionable rather than static. These are projections, not commitments — they follow directly from the dependency structure in Sections 10.1–10.2, not from a new estimate.

| Milestone | Projected Rating | Reasoning |
| --- | --- | --- |
| OI-04 and OI-01 both resolved | ★★☆☆☆ → ★★★☆☆ | Unblocks Groups A, B, C to start; still bounded by Group C's 5–10 week duration (Section 10.2) before those groups finish |
| Groups A–C complete | ★★★★☆ | RTL Coding Readiness reaches this level; ASIC Design Readiness also rises but remains capped by OI-13/OI-16 (D1, High priority, no duration estimate — Section 10.2 Gap note) |
| OI-13 and OI-16 resolved with duration known | ★★★★★ (RTL) — ceiling not yet knowable for ASIC | Verification detail and certification traceability are the remaining High-priority gates on RTL sign-off; ASIC readiness additionally depends on OI-08's DFT scope (Group C) and has no further projection until those durations exist |

Table 10-4. Readiness Projection by Milestone

## 10.4 Register

| ID | Section | Open Item | Decision Owner | Status (R8) |
| --- | --- | --- | --- | --- |
| OI-01 | 4.3 | SEU1 completeness-window latency — exact min/max clock-cycle bound | 창호 + hardware validation results | Narrowed — window value fixed (WP#25 R1 §A.7.3); cycle count awaits clock frequency. Root item — outside Claude's ability to close further. |
| OI-02 | 4.4 | Precise FPGA resource occupancy (LUT/BRAM/DSP/SerDes %) on a named device (e.g., VU9P/VU13P) | FPGA bring-up team (Section 9) | Open — requires real synthesis tooling Claude does not have |
| OI-03 | 6.3 | WP#25 R1 PTP jitter tolerance value and RTL-level margin policy | WP#25 R1 maintainer | Closed — WP#25 R1 Table A-5/B-4 |
| OI-04 | 4.5 / 8.4 | ASIC process node candidate — not yet selected for either engine | 창호 / foundry partner selection | Open — root item, requires 창호's negotiation, outside Claude's ability to close |
| OI-05 | 4.1 | WORM Write Gate RTL structure — FSM vs one-shot pulse vs lock register | RTL architecture — WP#26.1 | Drafted — worm_write_gate.sv, compiled + simulated (Section 10.5), pending 창호 review |
| OI-06a | 5.2 | Layer Lock arbitration priority and deadlock avoidance (split from OI-06, R8) | RTL architecture — WP#26.1 | Drafted — m_axis_layer_lock_arbiter.sv, compiled + simulated (Section 10.5), pending 창호 review |
| OI-06b | 5.2 | HBF ACL bit-field evaluation timing (split from OI-06, R8) | RTL architecture — WP#26.1 | Open — not addressed by the arbiter module |
| OI-07 | 7.2 | 800G PHY/PCS detail — line coding, FEC placement, skew alignment/channel bonding, LOS/LOF logic | PHY integration — WP#26.3 | Open |
| OI-08 | 8.2 | DFT strategy — scan chain, JTAG boundary scan, SerDes BIST, WORM_CHAN_ID fuse/OTP method | ASIC design team — WP#26.2 | Open |
| OI-09 | 6.1 | CDC synchronization technique for any SEU1/SEU2 monitoring tap | RTL architecture (low priority — tap is non-functional) | Closed — Claude proposal, pending 창호 confirmation |
| OI-10 | 5.5 | Physical isolation implementation method — guard ring / deep N-well / ISO cell | ASIC physical design team | Open |
| OI-11 | 8.2 | On-die power/clock-gating strategy for independent SEU1/SEU2 shutdown | ASIC design team | Open |
| OI-12 | 5.3 | Phase-alignment handling between SEU2 Crystal OSC and SEU1 PTP domain | RTL architecture | Open |
| OI-13 | 9 | Verification specifics — test-vector counts, coverage targets, random/formal verification plan, isolation-validation methodology | Verification team — WP#26.2 | Open |
| OI-14 | 5.1 | Cross-document terminology tension: WP#24 core ranges ("#1–24") vs WP#25 R1-3 singular framing ("1 core = 1 instance") | WP#24 + WP#25 R1 record owners | Closed — WP#25 R1 §B.9 |
| OI-15 | 9 | FPGA↔ASIC RTL compatibility guarantee — BRAM-to-macro mapping, clock-gating/reset behavior match | Verification team | Open |
| OI-16 | 5.5 | EN 50205 safety-certification RTL traceability methodology | Safety-certification lead — WP#26.2 | Open |

Table 10-5. Open Items Register — 17 items (OI-06 split), R8

Items marked "WP#26.1/.2/.3" above are grouped only by natural fit; none of those addenda have scoped content yet (Table 2-1, Table 11-1 list them as contingent, not proposed-with-content).

R7 closed three items by locating answers already present in WP#25 R1 (OI-03, OI-14) and narrowed one (OI-01) to a pure calculation once a clock frequency is chosen. R8 went further: two items (OI-05, OI-06a) now have actual compiled and simulated RTL, not just a chosen approach in prose. OI-01 and OI-04 are explicitly marked as outside Claude's ability to close by further document work — they require 창호's hardware validation and foundry negotiation respectively, and no amount of additional revision closes that gap.

## 10.5 Companion RTL Artifacts (R8)

Two SystemVerilog modules and two directed testbenches accompany this revision as external files, not reproduced inline. Both modules were compiled with a real Verilog compiler (Icarus Verilog 12.0) and both testbenches were simulated to completion — this is the first content in this document series that is executable rather than descriptive.

| File | Resolves | Verification Result |
| --- | --- | --- |
| worm_write_gate.sv | OI-05 — 4-state FSM (IDLE/LATCH/COMMIT/LOCKED) | Compiles clean (iverilog -g2012). Simulated via tb_worm_write_gate.sv: 2 back-to-back records, audit-pointer incremented correctly, hbm_wr_en pulsed exactly once per record — all checks PASS. |
| m_axis_layer_lock_arbiter.sv | OI-06a — 2-requester alternate-on-contention arbiter | Compiles clean. Simulated via tb_m_axis_layer_lock_arbiter.sv: single-requester grant verified, 4 rounds of simultaneous contention resolved with zero starvation and zero dual-grant violations — all checks PASS. |

Table 10-6. Companion RTL Files — Compiled and Simulated

Open — Scope limit: "Simulated and passing" means the RTL behaves as designed against the checks Claude wrote — it is not synthesis-clean, not timing-closed, not reviewed by 창호, and not validated against real hardware. worm_write_gate.sv also carries one flagged design assumption (WORM_SECTOR_LOCK as a per-slot lock word) that needs 창호's confirmation before this can be treated as more than a draft. This closes the gap between "described" and "coded and simulated," not the gap between "coded and simulated" and "production-ready."

# 11. Conclusion and Future Document Proposals

WP#26 completes the RTL-to-silicon translation of the Dedicated Equipment structure fixed in WP#24 and the SEU1/SEU2 record structure fixed in WP#25 R1. Its central contribution is the explicit two-engine template split — SEU1 as the common, higher-volume, PTP-synchronized engine; SEU2 as the EAI-only, independently clocked, safety-relevant engine — carried consistently from RTL module decomposition through FPGA prototyping to ASIC production guidance, with ASIC treated as the primary production target for both templates given the fixed (non-reconfigurable) nature of the WORM/ACL hardware contract.

This R2 revision does not add new architectural claims. It makes the document's 16 undecided values explicit (Section 10) rather than leaving them as qualitative language that could be mistaken for a settled decision. WP#26 R2 is therefore complete as a structural/RTL-macro specification and explicit about not yet being a coding-ready implementation guide — that gap is the Open Items Register itself, not a hidden one.

Next in sequence per WP#24 §11.1 / WP#25 R1 Table C-2: WP#27 (PAI AI DC Deployment Guide), covering multi-instance parallel operation of the SEU1 engine template defined here, rack wiring, and AI Data Center power specification.

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete |
| WP #26 R2 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, timing analysis, 800G optical interface, FPGA→ASIC migration | Complete (R2) — 16 open items, Section 10 |
| WP #26.1 (contingent) | RTL Macro-Architecture & FSM Addendum | WORM/ACL FSM, CDC technique, physical isolation implementation | Contingent — no content until OI-05/06/09/10/11 decided |
| WP #26.2 (contingent) | Verification & DFT Specification Addendum | Test-plan specifics, coverage targets, DFT strategy | Contingent — no content until OI-08/13/15/16 decided |
| WP #26.3 (contingent) | 800G PHY Integration Addendum | SerDes/PCS/FEC implementation detail | Contingent — no content until OI-07 decided |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Proposed — next |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Proposed |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; Core Stack negotiation protocol | Proposed — spun off |

Table 11-1. Future Document Proposals — Updated at WP#26 R2

This document is an internal technical design document of the Velsanet Architecture Group. All design specifications are based on software MVP validation results and will be finalized through physical hardware validation.

Velsanet Architecture Group  —  Internal Technical Document

VELSANET ARCHITECTURE GROUP | TECHNICAL DESIGN DOCUMENT

# WP#27 — PAI AI Data Center Deployment Guide

Multi-Instance Parallel Operation · Rack Wiring · DC Power Specification

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Public — Open Specification | v1.1 (R2) │ 2026 | PAI AI Data Center Deployment — Multi-Instance Operation, Rack Wiring, Power Spec | Extends Cognition Cube HW Design v1.1 §1.1, §7; WP#24 §11.1; WP#25 R1 Table B-5; WP#26 §4 |

Republic of Korea | Velsanet Architecture Group

Governance note: This specification is published in full under the Velsanet open-specification policy. Certification against the deployment parameters in this document — not confidentiality — is the mechanism by which conformance and interoperability are controlled.

# 1. Purpose and Scope

WP#27 is the deployment-facing counterpart to WP#26 for the PAI (Personal AI) side of the Velsanet Cognition Cube architecture. Where WP#26 fixed the SEU1 RTL engine template shared by every PAI and every EAI, this document fixes how that engine is deployed at scale in an AI Data Center: multiple concurrent PAI instances sharing a Node_8 hub, the physical rack wiring that connects them, and the power specification that differs fundamentally from EAI’s field deployment (WP#28).

Scope includes: the architectural basis for multi-instance PAI operation (Section 4); rack-level physical wiring using the MOCT fixed-ribbon convention (Section 5); AI Data Center power specification (Section 6); and the environmental/physical contrasts with EAI that justify a separate deployment guide (Section 3).

Out of scope: the SEU1 RTL engine itself (WP#26 Section 4, unchanged here — this document is deployment, not RTL); EAI/E2E Center deployment (WP#28); O8/D12/I20 geographic placement (deferred per WP#25 R1-4 to a separate forthcoming document).

# 2. Positioning in the Document Sequence

WP#27 was proposed at WP#24 §11.1 and restated at WP#25 R1 Table C-2 and WP#26 Table 2-1/11-1. This document completes it.

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete |
| WP #26 R8 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, timing analysis, 800G optical interface, FPGA→ASIC migration | Complete (R8) — 12 open items remain |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete (this document) |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete (companion document) |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; Core Stack negotiation protocol | Proposed — spun off |

Table 2-1. Document Sequence — Updated at WP#27

# 3. Deployment Context — Why PAI Needs a Separate Deployment Guide

Cognition Cube HW Design v1.1 §1.1 fixes the core premise: EAI is deployed at the E2E Center (city/regional scale); PAI is deployed in the AI Data Center. Both use Velsanet MOCT fixed ribbon cable, but location, processing objective, and channel configuration differ fundamentally — summarized in Table 3-1, restated from that document’s Table 7-1.

| Design Item | PAI (AI Data Center) | EAI (E2E Center, for contrast — see WP#28) |
| --- | --- | --- |
| Channel count | 8-channel single E2E (SEU1 only) | 16-channel Dual E2E (SEU1 + SEU2) |
| Safety Override | Not applied | CH11 independent power + relay required |
| Environmental rating | Data center standard (20–25°C) | IP54+, −10°C to +60°C |
| Ribbon cable length | 1–3 m (server rack wiring) | 5 m+ (outdoor pole installation) |
| Parallel instances | PAI-1, PAI-2, …, PAI-N concurrent | Robot/drone unit, 1:1 |
| Primary M-Axis use | Interpretation tags, policy updates | Mission experience, autonomy growth |
| Power supply | Data center PDU standard power | PoE++ or independent DC + UPS |

Table 3-1. PAI vs EAI Design Differences (restated from Cognition Cube HW Design v1.1 Table 7-1)

Per WP#25 R1 Table B-5 / §B.13.1: a PAI instance operates at Core Stack Layer 0 only (SEU1, 1 fixed core) — Layer 1 (SEU2-A) is what distinguishes EAI mode from PAI mode by definition, and a PAI implementation exposes no SEU2 register surface or record path at all. This document therefore concerns SEU1-engine deployment exclusively; nothing here applies to SEU2, which does not exist in a PAI instance.

# 4. Multi-Instance Parallel Operation

Cognition Cube HW Design v1.1 §7.2 fixes the architectural basis for running many PAI instances concurrently against a shared Node_8 (O8) hub.

| Element | Specification |
| --- | --- |
| Lane source | Node_8 Face B/C: 192 optical cores × 2 faces = 384 Cube lanes |
| Instance allocation | Each lane maps to one independent PAI instance |
| Non-collision guarantee | Core(ri) ∩ Core(rj) = ∅ — physical guarantee, not a software policy |
| Instance independence | PAI-1, PAI-2, …, PAI-N, each with its own 8-channel SEU1 engine, independent T-Axis and M-Axis |
| Uplink | Each PAI instance → its own 24-core Bundle → D12 AAI node |

Table 4-1. Parallel Multi-Instance PAI Architecture (restated from Cognition Cube HW Design v1.1 §7.2)

Because the non-collision guarantee (Core(ri) ∩ Core(rj) = ∅) is physical rather than logical, instance isolation at the AI Data Center follows the same structural principle WP#26 established for SEU1/SEU2 isolation at the EAI level (WP#26 §6.1): no shared clock domain, no shared register bus, and — per Section 5 below — no shared physical ribbon segment between instances.

## 4.1 Instance Addressing

Each PAI instance’s SEU1 engine carries its own WORM_CHAN_ID (WP#24 §1, Table 1-1, offset 0x14, fixed at fabrication) and its own NODE_ID (WP#25 R1 Table A-1), both fixed per physical optical core. Instance identity at the rack level is therefore a direct physical consequence of which core a PAI instance is wired to — there is no separate software-assigned instance ID layer to keep synchronized with the physical wiring.

Open — OI-27-01: Maximum practical N (number of concurrent PAI instances per Node_8) is bounded above by 384 lanes (Table 4-1) but no realistic operating N is fixed in this document — that depends on per-instance compute load at the AI Data Center, which is outside Velsanet*’*s architecture scope and belongs to deployment capacity planning. Decision owner: 창호 / data center operations planning.

# 5. Rack Wiring Specification

Rack wiring follows the same MOCT fixed-ribbon convention used throughout the architecture (WP#24 §1.4; WP#03 MOCT White Paper): factory-fused, no field connectors, no branching, 0dB insertion loss. The AI Data Center context changes only the physical scale and routing pattern, not the connector philosophy.

| Parameter | Specification |
| --- | --- |
| Ribbon length | 1–3 m (server rack wiring), per Table 3-1 — short-run, intra-rack or adjacent-rack |
| Connector policy | None — factory-fused MOCT ribbon throughout, consistent with WP#24 §1.4 and WP#03 |
| Per-instance lane count | 8 channels × lane grouping per WP#26 Table 4-2 (48 lanes/ch × 8 groups, PAI configuration) |
| Rack-to-Node_8 routing | Each rack’s PAI instances trace back to a Node_8 Face B or C lane (Table 4-1) — routing is a physical fan-out from the 384-lane hub, not a switched fabric |
| Uplink wiring | 24-core Bundle per instance toward D12 AAI node (Table 4-1), physically distinct from the 8-channel inbound wiring |

Table 5-1. Rack Wiring Specification

## 5.1 Rack Density — Reference Calculation

Reference — EIA-310-D basis: Standard 19-inch rack (EIA-310-D): 1 rack unit (1U) = 1.75 in (44.45 mm) vertical height; usable rack width = 19 in (482.6 mm) front panel, 18.312 in (465.1 mm) mounting-rail spacing. A standard full-height rack is 42U (≈1866 mm usable height).

Applying this standard to the per-instance wiring in Table 5-1 gives a reference density calculation — illustrative, not a fixed spec, since it does not yet account for actual MOCT ribbon bend-radius or connector-free routing clearance (OI-27-02 below).

| Step | Reference Calculation |
| --- | --- |
| Rack height available | 42U × 1.75in = 73.5in (1866mm), per EIA-310-D |
| Illustrative per-instance slot | If each PAI instance’s compute + Cognition Cube interface occupies 1U (illustrative — actual PAI compute hardware height is outside this document’s scope) |
| Reference instance density | Up to 42 PAI instances per full-height rack, before accounting for power distribution units, cable management, and airflow gaps that a real rack layout requires |

Table 5-2. Illustrative Rack Density Reference (EIA-310-D basis; example only)

Open — OI-27-02: The 42-instance figure above is a reference ceiling from rack geometry alone — it does not account for real MOCT ribbon minimum bend radius, connector-free routing clearance, PDU/cable-management rack space, or actual PAI compute hardware height (which may exceed 1U). Real achievable density is lower and is a physical/mechanical design question for actual rack hardware selection. Decision owner: data center physical infrastructure team.

# 6. DC Power Specification

Per Table 3-1, PAI draws data center PDU standard power — a deliberate contrast to EAI’s PoE++/independent-DC-plus-UPS requirement (WP#28 §7). This reflects that PAI carries no CH11-class safety obligation (Table 3-1, “Safety Override: Not applied”) and no field-power-loss scenario to defend against; AI Data Center power infrastructure already provides the redundancy a PAI instance needs.

| Parameter | Specification |
| --- | --- |
| Primary power | Data center PDU standard power (Table 3-1) |
| Backup/UPS | Data center facility-level UPS — no per-instance independent power path required, unlike EAI’s CH11 (WP#28 §6) |
| Environmental rating | Data center standard, 20–25°C (Table 3-1) — no IP-rated enclosure required |
| Power domain isolation | Not required between PAI instances at the power layer — inter-instance isolation is enforced optically (Section 4, Core(ri) ∩ Core(rj) = ∅), not electrically |

Table 6-1. DC Power Specification

Reference — PoE++ basis, for contrast (IEEE 802.3bt): WP#28 specifies EAI may use PoE++ (IEEE 802.3bt): Type 3 delivers up to 60W at the PSE (51W guaranteed at the PD), Type 4 up to 100W at the PSE (71.3W–100W at the PD depending on class), both at 50–57V DC. This is cited here only for contrast — PAI does NOT use PoE++ (Table 3-1: *“**Data center PDU standard power**”); a PAI instance‘**s power draw is a data-center-facilities question (rack PDU capacity planning), not a Velsanet-specified figure, unlike EAI**’*s field power path which the architecture does specify (WP#28 §7).

Open — OI-27-03: Per-instance or per-rack power budget (watts) is not specified — this depends on the compute hardware hosting each PAI instance*’*s downstream processing, which is outside the Cognition Cube hardware boundary (WP#24 §1) and is a data-center facilities question, not a Velsanet architecture question.

# 7. Conclusion and Open Items

WP#27 completes the PAI-side deployment guide proposed at WP#24 §11.1: multi-instance parallel operation is fixed at the architecture level (Section 4, restating Cognition Cube HW Design v1.1 §7.2), rack wiring follows the established MOCT connector-free convention at data-center scale (Section 5), and DC power specification is fixed as data-center-standard with no EAI-class safety power path (Section 6).

R2 adds external, industry-standard reference values (EIA-310-D rack geometry, IEEE 802.3bt PoE++ figures for contrast) — clearly distinguished from Velsanet-specific decisions still pending. No Velsanet-specific number was invented in this revision: actual per-instance compute footprint and power draw remain open and are owned by 창호 / data center operations.

| ID | Open Item |
| --- | --- |
| OI-27-01 | Maximum practical concurrent-instance count N per Node_8 — bounded above by 384 lanes, not fixed by workload |
| OI-27-02 | Real achievable rack density — EIA-310-D reference ceiling now given (Section 5.1, up to 42/rack by geometry alone); real MOCT bend-radius and compute-hardware height still unspecified |
| OI-27-03 | Per-instance/per-rack power budget in watts — depends on downstream compute hardware, outside Cognition Cube boundary |

Table 7-1. WP#27 Open Items (R2)

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete |
| WP #26 R8 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, timing analysis, 800G optical interface, FPGA→ASIC migration | Complete (R8) — 12 open items remain |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete (this document) — 3 open items |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete (companion document) |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; Core Stack negotiation protocol | Proposed — spun off |

Table 7-2. Future Document Proposals — Updated at WP#27

This document is an internal technical design document of the Velsanet Architecture Group. All design specifications are based on software MVP validation results and will be finalized through physical hardware validation.

Velsanet Architecture Group — Internal Technical Document

VELSANET ARCHITECTURE GROUP | TECHNICAL DESIGN DOCUMENT

# WP#28 — EAI E2E Center Deployment Guide

Outdoor Installation · CH11 Independent Power · Dual E2E Field Validation

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Public — Open Specification | v1.1 (R2) │ 2026 | EAI E2E Center Deployment — Outdoor Install, CH11 Power, Dual E2E Field Validation | Extends Cognition Cube HW Design v1.1 §1.1, §5.3, §7.1; Structure Diagram §5; WP#24 §11.1; WP#25 R1; WP#26 §5, §6 |

Republic of Korea | Velsanet Architecture Group

Governance note: This specification is published in full under the Velsanet open-specification policy. Certification against the deployment and field-validation parameters in this document — not confidentiality — is the mechanism by which conformance and interoperability are controlled.

# 1. Purpose and Scope

WP#28 is the deployment-facing counterpart to WP#26 for the EAI (Embodied AI) side of the Velsanet Cognition Cube architecture — the companion document to WP#27 (PAI AI Data Center Deployment Guide). Where WP#26 fixed the SEU1 and SEU2 RTL engine templates and their physical isolation at the register-transfer level, this document fixes how an EAI instance is actually installed and validated in the field: outdoor environmental requirements, CH11 independent-power field implementation, and the acceptance test procedure for Dual E2E physical isolation.

Scope includes: outdoor installation requirements (Section 4); CH11 independent-power field implementation (Section 5); Dual E2E field validation procedure (Section 6); and EAI field power specification (Section 7).

Out of scope: the SEU1/SEU2 RTL engines themselves (WP#26, unchanged here); PAI/AI Data Center deployment (WP#27); O8/D12/I20 geographic placement (deferred per WP#25 R1-4 to a separate forthcoming document).

# 2. Positioning in the Document Sequence

WP#28 was proposed at WP#24 §11.1 and restated at WP#25 R1 Table C-2 and WP#26 Table 2-1/11-1. This document, together with WP#27, completes that proposal.

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete |
| WP #26 R8 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, timing analysis, 800G optical interface, FPGA→ASIC migration | Complete (R8) — 12 open items remain |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete (companion document) |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete (this document) |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; Core Stack negotiation protocol | Proposed — spun off |

Table 2-1. Document Sequence — Updated at WP#28

# 3. Deployment Context — EAI at the E2E Center

Cognition Cube HW Design v1.1 §1.1: EAI is deployed at the E2E Center (city/regional scale, O8/D12 nodes), running a 16-channel Dual E2E Cognition Cube (perception + control) — structurally distinct from PAI’s 8-channel single-E2E deployment at the AI Data Center (WP#27). Per WP#25 R1 §B.13.1, an EAI instance is exactly the instance for which Core Stack Layer 1 (SEU2-A) is active; this is the control-activation criterion that makes it EAI rather than PAI, and it is why this document’s field requirements — outdoor rating, CH11, Dual E2E isolation — have no PAI counterpart.

| Design Item | EAI (E2E Center) | PAI (AI Data Center, for contrast — see WP#27) |
| --- | --- | --- |
| Channel count | 16-channel Dual E2E (SEU1 + SEU2) | 8-channel single E2E (SEU1 only) |
| Safety Override | CH11 independent power + relay required | Not applied |
| Environmental rating | IP54+, −10°C to +60°C | Data center standard (20–25°C) |
| Ribbon cable length | 5 m+ (outdoor pole installation) | 1–3 m (server rack wiring) |
| Deployment pattern | Robot/drone unit, 1:1 | PAI-1, …, PAI-N concurrent |
| Primary M-Axis use | Mission experience, autonomy growth | Interpretation tags, policy updates |
| Power supply | PoE++ or independent DC + UPS | Data center PDU standard power |

Table 3-1. EAI vs PAI Design Differences (restated from Cognition Cube HW Design v1.1 Table 7-1)

# 4. Outdoor Installation Requirements

| Parameter | Specification |
| --- | --- |
| Environmental rating | IP54+ (dust/splash protection), per Table 3-1 |
| Operating temperature | −10°C to +60°C, per Table 3-1 |
| Ribbon cable length | 5 m+ — sized for outdoor pole installation, per Table 3-1 |
| Connector policy | None — factory-fused MOCT ribbon throughout, unchanged from the indoor/DC case (WP#24 §1.4, WP#27 §5); outdoor length does not introduce field connectors |
| Mounting pattern | Robot/drone unit, 1:1 — one EAI instance per physical platform, per Table 3-1, distinct from PAI’s shared-hub multi-instance pattern (WP#27 §4) |

Table 4-1. Outdoor Installation Requirements

Reference — IP54 basis (IEC 60529 / EN 60529): IP54 = first digit 5 (dust-protected: no harmful dust deposit, not fully dust-tight) + second digit 4 (protected against water splashing from any direction). Per IEC 60529 and general industry practice, IP54 covers indoor and sheltered outdoor use; it is not rated for direct water jets, pressure washing, or sustained heavy rain. Fully exposed outdoor pole installations (no canopy/shelter) more commonly specify IP65 (dust-tight + low-pressure water jets) or IP66 (dust-tight + powerful water jets) in comparable telecom/industrial outdoor equipment.

Open — OI-28-01: The *“**+**” in IP54+ is confirmed by the source spec (Cognition Cube HW Design v1.1 Table 7-1) as a floor, not a ceiling — but given the IEC 60529 distinction above, a fully exposed pole-mounted EAI unit (Table 4-1, no canopy) is a IP65/IP66 use case by common industry practice, not a IP54 one. This is flagged as a technical caution, not a change to the spec: if the deployment is truly fully exposed (not sheltered), IP54 alone is likely under-specified. Final rating remains 창호**’*s site-specific decision.

Open — OI-28-02: Pole/mounting mechanical hardware (bracket load rating, vibration tolerance for a MOCT ribbon under mounting stress) is not specified — this is mechanical engineering scope, not Cognition Cube architecture scope, and is out of scope for this document as an architecture record.

# 5. CH11 Independent-Power Field Implementation

CH11 (Safety Override) is structurally isolated from the rest of the EAI’s channels: it shares no register, bus, or logic path with SEU synchronization of any kind (WP#24 §5; WP#25 R1 Table B-3, “Excluded” classification). This section fixes the field-hardware implementation of that isolation.

| Component | Specification |
| --- | --- |
| Relay type | Force-guided relay, EN 50205 Type A/B — ≥1 NO (load path) + ≥1 NC (monitor path), mechanically linked so both cannot close under a single fault (Cognition Cube Structure Diagram §5) |
| Independent power source | Supercapacitor bank, sized as C ≥ (I×t)/ΔV against CH11 controller + relay-hold current (Cognition Cube Structure Diagram §5) |
| Self-sustain duration | ≥30 s independent of main power, per WP#24 Table 5-2 and Structure Diagram §5 |
| Power domain isolation | Backup UPS + supercapacitor parallel — physically separate from E2E #1’s Main PSU (WP#24 Table 5-2) |
| Fault propagation guarantee | E2E #2 (control plane, carrying CH11) failure is structurally independent of E2E #1 (perception plane) failure and vice versa (WP#24 Table 5-2; WP#26 §6.1) |

Table 5-1. CH11 Field Power Implementation

This field requirement is the physical-layer counterpart to WP#26’s OI-06a resolution (m_axis_layer_lock_arbiter.sv): that module arbitrates AAI/AsAI access to M-Axis at the RTL level, but explicitly does not touch CH11, which — per WP#24 §5 — shares no register, bus, or logic path with anything else in the system, by design, at both the RTL layer (WP#26) and the field-hardware layer (this section).

## 5.1 Reference Relay Options — EN 50205

The specification requires a force-guided relay certified to EN 50205 Type A or Type B; it does not mandate a vendor. The table below lists commercially available reference options meeting that certification, to make the requirement concrete without narrowing it to a single sole-sourced part.

| Reference Part | EN 50205 Type | Contact Rating | Notes |
| --- | --- | --- | --- |
| Finder 50 Series | Type B | Up to 8A, 250VAC | PCB relay, 2 CO contacts, compact form factor |
| Schrack/TE SR6 Series | Type A | 8A, 250VAC (3NO+3NC / 4NO+2NC / 5NO+1NC) | Reinforced inter-contact insulation; industrial safety-relay family |
| Weidmuller RIDERSERIES FG | Type B | 6A, 2 CO | Miniature industrial relay for signal monitoring in safety circuits |

Table 5-2. EN 50205 Reference Relay Options (commercially available, non-exclusive)

Open — OI-28-05: Final relay selection (specific part number, contact rating margin against actual CH11 controller/relay-hold current, and Type A vs Type B choice) is not made here — the three options above are presented as a shortlist meeting the certification requirement, not as the selected part. Decision owner: 창호 / hardware procurement.

## 5.2 Supercapacitor Sizing — Worked Example

Cognition Cube Structure Diagram §5 fixes the sizing formula (C ≥ (I×t)/ΔV) but not the input values, which depend on the actual CH11 controller and relay-coil current draw — not yet fixed in any Velsanet document. The worked example below applies the formula using illustrative industry-typical values, to make the calculation concrete and reusable once real current figures are available; the resulting capacitance is explicitly not a spec value.

| Step | Illustrative Calculation |
| --- | --- |
| Assumed load | P ≈ 20W (illustrative — CH11 controller + relay coil hold current at typical 24V industrial control levels; NOT a Velsanet-confirmed figure) |
| Target hold-up time | t = 30s (fixed — WP#24 Table 5-2, Structure Diagram §5) |
| Voltage window | 24V nominal, 20% allowable droop → V_start = 24V, V_end = 19.2V (standard industrial-backup design practice) |
| Energy required | E = P × t = 20W × 30s = 600J |
| Capacitance | C = 2E / (V_start² − V_end²) = 1200 / (576 − 368.64) = 1200 / 207.36 ≈ 5.8F minimum, before tolerance margin |
| With margin | Applying a typical ±20% cell-tolerance margin (standard supercapacitor datasheet practice): C ≈ 7F class bank |

Table 5-3. Illustrative Supercapacitor Sizing Calculation (formula per Structure Diagram §5; example values only)

Reference — Reference power-management ICs: For implementation reference only: Analog Devices LTC4041 (supercapacitor backup power management, step-down charging + step-up backup boost, up to 2.5A) and LTC3226 (2-cell supercapacitor charger with backup, auto cell balancing) are commercially available parts designed specifically for this class of supercapacitor-backup circuit, cited here as a starting point for the CH11 power path design, not as a selected component.

Open — OI-28-03: The actual CH11 controller + relay-hold current draw — the real input to the formula above — is not yet measured or specified in any Velsanet document. Table 5-3*’*s ≈7F figure is illustrative only and will change once that current is known. Field-replaceable-unit policy (replacement interval, end-of-life detection, field test procedure to verify ≥30s self-sustain without full discharge) is also not specified. Decision owner: 창호 / hardware validation.

# 6. Dual E2E Field Validation

This section defines the field acceptance test for the physical isolation WP#24 Table 5-2 requires and WP#26 §6.1 assumes at the RTL level. Passing this procedure is what confirms, for a specific installed unit, that the architecture’s isolation guarantee actually holds in that unit’s hardware — not just in the specification.

| Validation Item | Procedure | Pass Criterion |
| --- | --- | --- |
| Optical core allocation | Verify E2E #1 traffic confined to Cores #1–24 (ribbon upper group) and E2E #2 to Cores #25–48 (lower group), per WP#24 Table 5-2 | Zero cross-talk between groups, measured at the DEMUX Engine output (WP#26 §7) |
| Power domain isolation | Disconnect Main PSU; verify E2E #2 (Backup UPS + supercapacitor) remains powered | E2E #2 operation continues uninterrupted; CH11 self-sustain ≥30s confirmed (Section 5) |
| Clock source isolation | Inject fault on PTP master clock (E2E #1 source); verify E2E #2’s independent Crystal OSC is unaffected | E2E #2 timing reference unaffected, consistent with WP#25 R1 Table B-4 (“independent oscillator… itself part of the fault-isolation guarantee”) |
| Fault propagation | Force an E2E #1 (perception plane) fault; verify no E2E #2 (control plane) fault is observed, and vice versa | SEU1 ABORTED has no effect on SEU2 assembly state and vice versa, per WP#25 R1 §B.11.2 / §B.8 |
| CH11 override path | Trigger CH11 Safety Override with E2E #2 main path simulated-faulted | Force-guided relay opens load path via independent power alone, per Section 5 Table 5-1 |

Table 6-1. Dual E2E Field Validation Procedure

This procedure is the field-hardware analog of WP#26 Table C-1 (Cross-Reference to Hardware Validation Strategy, WP#25 R1), which specifies the same independence guarantees at the RTL/simulation level. Table 6-1 above is what confirms those simulated guarantees against a physically installed unit, not a substitute for the RTL-level validation WP#26 already requires.

Open — OI-28-04: Test equipment and exact fault-injection method for each row of Table 6-1 (e.g., how a PTP master fault is actually injected in the field vs. in simulation) are not specified — this is field-test-procedure detail, a step below architecture-record scope, and is proposed as a candidate for a future field validation runbook rather than resolved here.

# 7. EAI Field Power Specification

Per Table 3-1, EAI draws PoE++ or independent DC + UPS — a deliberate contrast to PAI’s data-center-PDU-only requirement (WP#27 §6), reflecting that a field-deployed EAI instance has no facility-level power redundancy to rely on and must carry its own.

| Parameter | Specification |
| --- | --- |
| Primary power (E2E #1) | Main PSU (WP#24 Table 5-2) — PoE++ or independent DC, per Table 3-1 |
| Backup/independent power (E2E #2) | Backup UPS + supercapacitor parallel (WP#24 Table 5-2; Section 5 above) |
| Environmental rating | IP54+, −10°C to +60°C (Section 4) — power components share the same outdoor rating as the rest of the unit |
| Power domain isolation | Required between E2E #1 and E2E #2, unlike PAI where no such isolation is needed between instances (WP#27 §6) — here the isolation is between planes within one instance, not between instances |

Table 7-1. EAI Field Power Specification

# 8. Conclusion and Open Items

WP#28 completes the EAI-side deployment guide proposed at WP#24 §11.1, alongside its companion WP#27. Outdoor installation requirements are fixed at the environmental and physical level (Section 4), CH11 independent power is specified down to relay type and supercapacitor sizing basis (Section 5), a Dual E2E field acceptance test procedure is defined against the isolation guarantees WP#24/#25/#26 already require at the specification and RTL levels (Section 6), and EAI field power is fixed as carrying its own redundancy rather than relying on facility power (Section 7).

R2 adds external, industry-standard reference values and reference products (IEC 60529 IP-rating basis, EN 50205-certified relay shortlist, a worked supercapacitor sizing example) — all clearly distinguished from Velsanet-specific decisions still pending. No Velsanet-specific number was invented in this revision: real current draw, final relay selection, and final IP rating remain open and are owned by 창호.

| ID | Open Item |
| --- | --- |
| OI-28-01 | Exact IP rating beyond the IP54+ floor — IEC 60529 basis now cited (Section 4); IP65/66 flagged as the likely fit for fully exposed installs; final rating still site-specific |
| OI-28-02 | Pole/mounting mechanical hardware specification — mechanical engineering scope, not Cognition Cube architecture scope |
| OI-28-03 | Actual CH11 controller + relay-hold current draw — worked sizing methodology now given (Section 5.2); real input current still unmeasured |
| OI-28-04 | Exact test equipment and fault-injection method for each Dual E2E field validation item — candidate for a future field validation runbook |
| OI-28-05 | Final EN 50205 relay part selection — shortlist of 3 reference options now given (Section 5.1); final part/rating margin still pending |

Table 8-1. WP#28 Open Items (R2)

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete |
| WP #26 R8 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, timing analysis, 800G optical interface, FPGA→ASIC migration | Complete (R8) — 12 open items remain |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete (companion document) — 3 open items |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete (this document) — 4 open items |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; Core Stack negotiation protocol | Proposed — spun off |

Table 8-2. Future Document Proposals — Updated at WP#28

This document is an internal technical design document of the Velsanet Architecture Group. All design specifications are based on software MVP validation results and will be finalized through physical hardware validation.

Velsanet Architecture Group — Internal Technical Document