VELSANET ARCHITECTURE GROUP | TECHNICAL DESIGN DOCUMENT

# WP#28 — EAI E2E Center Deployment Guide

Outdoor Installation · CH11 Independent Power · Dual E2E Field Validation

| Classification | Version | Scope | Basis |
| --- | --- | --- | --- |
| Public — Open Specification | v1.1 (R2) │ 2026 | EAI E2E Center Deployment — Outdoor Install, CH11 Power, Dual E2E Field Validation | Extends Cognition Cube HW Design v1.1 §1.1, §5.3, §7.1; Structure Diagram §5; WP#24 §11.1; WP#25 R1; WP#26 §5, §6 |

Republic of Korea | Velsanet Architecture Group

Governance note: This specification is published in full under the Velsanet open-specification policy. Certification against the deployment and field-validation parameters in this document — not confidentiality — is the mechanism by which conformance and interoperability are controlled.

# 1. Purpose and Scope

WP#28 is the deployment-facing counterpart to WP#26 for the EAI (Embodied AI) side of the Velsanet Cognition Cube architecture — the companion document to WP#27 (PAI AI Data Center Deployment Guide). Where WP#26 fixed the SEU1 and SEU2 RTL engine templates and their physical isolation at the register-transfer level, this document fixes how an EAI instance is actually installed and validated in the field: outdoor environmental requirements, CH11 independent-power field implementation, and the acceptance test procedure for Dual E2E physical isolation.

Scope includes: outdoor installation requirements (Section 4); CH11 independent-power field implementation (Section 5); Dual E2E field validation procedure (Section 6); and EAI field power specification (Section 7).

Out of scope: the SEU1/SEU2 RTL engines themselves (WP#26, unchanged here); PAI/AI Data Center deployment (WP#27); O8/D12/I20 geographic placement (deferred per WP#25 R1-4 to a separate forthcoming document).

# 2. Positioning in the Document Sequence

WP#28 was proposed at WP#24 §11.1 and restated at WP#25 R1 Table C-2 and WP#26 Table 2-1/11-1. This document, together with WP#27, completes that proposal.

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete |
| WP #26 R8 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, timing analysis, 800G optical interface, FPGA→ASIC migration | Complete (R8) — 12 open items remain |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete (companion document) |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete (this document) |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; Core Stack negotiation protocol | Proposed — spun off |

Table 2-1. Document Sequence — Updated at WP#28

# 3. Deployment Context — EAI at the E2E Center

Cognition Cube HW Design v1.1 §1.1: EAI is deployed at the E2E Center (city/regional scale, O8/D12 nodes), running a 16-channel Dual E2E Cognition Cube (perception + control) — structurally distinct from PAI’s 8-channel single-E2E deployment at the AI Data Center (WP#27). Per WP#25 R1 §B.13.1, an EAI instance is exactly the instance for which Core Stack Layer 1 (SEU2-A) is active; this is the control-activation criterion that makes it EAI rather than PAI, and it is why this document’s field requirements — outdoor rating, CH11, Dual E2E isolation — have no PAI counterpart.

| Design Item | EAI (E2E Center) | PAI (AI Data Center, for contrast — see WP#27) |
| --- | --- | --- |
| Channel count | 16-channel Dual E2E (SEU1 + SEU2) | 8-channel single E2E (SEU1 only) |
| Safety Override | CH11 independent power + relay required | Not applied |
| Environmental rating | IP54+, −10°C to +60°C | Data center standard (20–25°C) |
| Ribbon cable length | 5 m+ (outdoor pole installation) | 1–3 m (server rack wiring) |
| Deployment pattern | Robot/drone unit, 1:1 | PAI-1, …, PAI-N concurrent |
| Primary M-Axis use | Mission experience, autonomy growth | Interpretation tags, policy updates |
| Power supply | PoE++ or independent DC + UPS | Data center PDU standard power |

Table 3-1. EAI vs PAI Design Differences (restated from Cognition Cube HW Design v1.1 Table 7-1)

# 4. Outdoor Installation Requirements

| Parameter | Specification |
| --- | --- |
| Environmental rating | IP54+ (dust/splash protection), per Table 3-1 |
| Operating temperature | −10°C to +60°C, per Table 3-1 |
| Ribbon cable length | 5 m+ — sized for outdoor pole installation, per Table 3-1 |
| Connector policy | None — factory-fused MOCT ribbon throughout, unchanged from the indoor/DC case (WP#24 §1.4, WP#27 §5); outdoor length does not introduce field connectors |
| Mounting pattern | Robot/drone unit, 1:1 — one EAI instance per physical platform, per Table 3-1, distinct from PAI’s shared-hub multi-instance pattern (WP#27 §4) |

Table 4-1. Outdoor Installation Requirements

Reference — IP54 basis (IEC 60529 / EN 60529): IP54 = first digit 5 (dust-protected: no harmful dust deposit, not fully dust-tight) + second digit 4 (protected against water splashing from any direction). Per IEC 60529 and general industry practice, IP54 covers indoor and sheltered outdoor use; it is not rated for direct water jets, pressure washing, or sustained heavy rain. Fully exposed outdoor pole installations (no canopy/shelter) more commonly specify IP65 (dust-tight + low-pressure water jets) or IP66 (dust-tight + powerful water jets) in comparable telecom/industrial outdoor equipment.

Open — OI-28-01: The *“**+**” in IP54+ is confirmed by the source spec (Cognition Cube HW Design v1.1 Table 7-1) as a floor, not a ceiling — but given the IEC 60529 distinction above, a fully exposed pole-mounted EAI unit (Table 4-1, no canopy) is a IP65/IP66 use case by common industry practice, not a IP54 one. This is flagged as a technical caution, not a change to the spec: if the deployment is truly fully exposed (not sheltered), IP54 alone is likely under-specified. Final rating remains 창호**’*s site-specific decision.

Open — OI-28-02: Pole/mounting mechanical hardware (bracket load rating, vibration tolerance for a MOCT ribbon under mounting stress) is not specified — this is mechanical engineering scope, not Cognition Cube architecture scope, and is out of scope for this document as an architecture record.

# 5. CH11 Independent-Power Field Implementation

CH11 (Safety Override) is structurally isolated from the rest of the EAI’s channels: it shares no register, bus, or logic path with SEU synchronization of any kind (WP#24 §5; WP#25 R1 Table B-3, “Excluded” classification). This section fixes the field-hardware implementation of that isolation.

| Component | Specification |
| --- | --- |
| Relay type | Force-guided relay, EN 50205 Type A/B — ≥1 NO (load path) + ≥1 NC (monitor path), mechanically linked so both cannot close under a single fault (Cognition Cube Structure Diagram §5) |
| Independent power source | Supercapacitor bank, sized as C ≥ (I×t)/ΔV against CH11 controller + relay-hold current (Cognition Cube Structure Diagram §5) |
| Self-sustain duration | ≥30 s independent of main power, per WP#24 Table 5-2 and Structure Diagram §5 |
| Power domain isolation | Backup UPS + supercapacitor parallel — physically separate from E2E #1’s Main PSU (WP#24 Table 5-2) |
| Fault propagation guarantee | E2E #2 (control plane, carrying CH11) failure is structurally independent of E2E #1 (perception plane) failure and vice versa (WP#24 Table 5-2; WP#26 §6.1) |

Table 5-1. CH11 Field Power Implementation

This field requirement is the physical-layer counterpart to WP#26’s OI-06a resolution (m_axis_layer_lock_arbiter.sv): that module arbitrates AAI/AsAI access to M-Axis at the RTL level, but explicitly does not touch CH11, which — per WP#24 §5 — shares no register, bus, or logic path with anything else in the system, by design, at both the RTL layer (WP#26) and the field-hardware layer (this section).

## 5.1 Reference Relay Options — EN 50205

The specification requires a force-guided relay certified to EN 50205 Type A or Type B; it does not mandate a vendor. The table below lists commercially available reference options meeting that certification, to make the requirement concrete without narrowing it to a single sole-sourced part.

| Reference Part | EN 50205 Type | Contact Rating | Notes |
| --- | --- | --- | --- |
| Finder 50 Series | Type B | Up to 8A, 250VAC | PCB relay, 2 CO contacts, compact form factor |
| Schrack/TE SR6 Series | Type A | 8A, 250VAC (3NO+3NC / 4NO+2NC / 5NO+1NC) | Reinforced inter-contact insulation; industrial safety-relay family |
| Weidmuller RIDERSERIES FG | Type B | 6A, 2 CO | Miniature industrial relay for signal monitoring in safety circuits |

Table 5-2. EN 50205 Reference Relay Options (commercially available, non-exclusive)

Open — OI-28-05: Final relay selection (specific part number, contact rating margin against actual CH11 controller/relay-hold current, and Type A vs Type B choice) is not made here — the three options above are presented as a shortlist meeting the certification requirement, not as the selected part. Decision owner: 창호 / hardware procurement.

## 5.2 Supercapacitor Sizing — Worked Example

Cognition Cube Structure Diagram §5 fixes the sizing formula (C ≥ (I×t)/ΔV) but not the input values, which depend on the actual CH11 controller and relay-coil current draw — not yet fixed in any Velsanet document. The worked example below applies the formula using illustrative industry-typical values, to make the calculation concrete and reusable once real current figures are available; the resulting capacitance is explicitly not a spec value.

| Step | Illustrative Calculation |
| --- | --- |
| Assumed load | P ≈ 20W (illustrative — CH11 controller + relay coil hold current at typical 24V industrial control levels; NOT a Velsanet-confirmed figure) |
| Target hold-up time | t = 30s (fixed — WP#24 Table 5-2, Structure Diagram §5) |
| Voltage window | 24V nominal, 20% allowable droop → V_start = 24V, V_end = 19.2V (standard industrial-backup design practice) |
| Energy required | E = P × t = 20W × 30s = 600J |
| Capacitance | C = 2E / (V_start² − V_end²) = 1200 / (576 − 368.64) = 1200 / 207.36 ≈ 5.8F minimum, before tolerance margin |
| With margin | Applying a typical ±20% cell-tolerance margin (standard supercapacitor datasheet practice): C ≈ 7F class bank |

Table 5-3. Illustrative Supercapacitor Sizing Calculation (formula per Structure Diagram §5; example values only)

Reference — Reference power-management ICs: For implementation reference only: Analog Devices LTC4041 (supercapacitor backup power management, step-down charging + step-up backup boost, up to 2.5A) and LTC3226 (2-cell supercapacitor charger with backup, auto cell balancing) are commercially available parts designed specifically for this class of supercapacitor-backup circuit, cited here as a starting point for the CH11 power path design, not as a selected component.

Open — OI-28-03: The actual CH11 controller + relay-hold current draw — the real input to the formula above — is not yet measured or specified in any Velsanet document. Table 5-3*’*s ≈7F figure is illustrative only and will change once that current is known. Field-replaceable-unit policy (replacement interval, end-of-life detection, field test procedure to verify ≥30s self-sustain without full discharge) is also not specified. Decision owner: 창호 / hardware validation.

# 6. Dual E2E Field Validation

This section defines the field acceptance test for the physical isolation WP#24 Table 5-2 requires and WP#26 §6.1 assumes at the RTL level. Passing this procedure is what confirms, for a specific installed unit, that the architecture’s isolation guarantee actually holds in that unit’s hardware — not just in the specification.

| Validation Item | Procedure | Pass Criterion |
| --- | --- | --- |
| Optical core allocation | Verify E2E #1 traffic confined to Cores #1–24 (ribbon upper group) and E2E #2 to Cores #25–48 (lower group), per WP#24 Table 5-2 | Zero cross-talk between groups, measured at the DEMUX Engine output (WP#26 §7) |
| Power domain isolation | Disconnect Main PSU; verify E2E #2 (Backup UPS + supercapacitor) remains powered | E2E #2 operation continues uninterrupted; CH11 self-sustain ≥30s confirmed (Section 5) |
| Clock source isolation | Inject fault on PTP master clock (E2E #1 source); verify E2E #2’s independent Crystal OSC is unaffected | E2E #2 timing reference unaffected, consistent with WP#25 R1 Table B-4 (“independent oscillator… itself part of the fault-isolation guarantee”) |
| Fault propagation | Force an E2E #1 (perception plane) fault; verify no E2E #2 (control plane) fault is observed, and vice versa | SEU1 ABORTED has no effect on SEU2 assembly state and vice versa, per WP#25 R1 §B.11.2 / §B.8 |
| CH11 override path | Trigger CH11 Safety Override with E2E #2 main path simulated-faulted | Force-guided relay opens load path via independent power alone, per Section 5 Table 5-1 |

Table 6-1. Dual E2E Field Validation Procedure

This procedure is the field-hardware analog of WP#26 Table C-1 (Cross-Reference to Hardware Validation Strategy, WP#25 R1), which specifies the same independence guarantees at the RTL/simulation level. Table 6-1 above is what confirms those simulated guarantees against a physically installed unit, not a substitute for the RTL-level validation WP#26 already requires.

Open — OI-28-04: Test equipment and exact fault-injection method for each row of Table 6-1 (e.g., how a PTP master fault is actually injected in the field vs. in simulation) are not specified — this is field-test-procedure detail, a step below architecture-record scope, and is proposed as a candidate for a future field validation runbook rather than resolved here.

# 7. EAI Field Power Specification

Per Table 3-1, EAI draws PoE++ or independent DC + UPS — a deliberate contrast to PAI’s data-center-PDU-only requirement (WP#27 §6), reflecting that a field-deployed EAI instance has no facility-level power redundancy to rely on and must carry its own.

| Parameter | Specification |
| --- | --- |
| Primary power (E2E #1) | Main PSU (WP#24 Table 5-2) — PoE++ or independent DC, per Table 3-1 |
| Backup/independent power (E2E #2) | Backup UPS + supercapacitor parallel (WP#24 Table 5-2; Section 5 above) |
| Environmental rating | IP54+, −10°C to +60°C (Section 4) — power components share the same outdoor rating as the rest of the unit |
| Power domain isolation | Required between E2E #1 and E2E #2, unlike PAI where no such isolation is needed between instances (WP#27 §6) — here the isolation is between planes within one instance, not between instances |

Table 7-1. EAI Field Power Specification

# 8. Conclusion and Open Items

WP#28 completes the EAI-side deployment guide proposed at WP#24 §11.1, alongside its companion WP#27. Outdoor installation requirements are fixed at the environmental and physical level (Section 4), CH11 independent power is specified down to relay type and supercapacitor sizing basis (Section 5), a Dual E2E field acceptance test procedure is defined against the isolation guarantees WP#24/#25/#26 already require at the specification and RTL levels (Section 6), and EAI field power is fixed as carrying its own redundancy rather than relying on facility power (Section 7).

R2 adds external, industry-standard reference values and reference products (IEC 60529 IP-rating basis, EN 50205-certified relay shortlist, a worked supercapacitor sizing example) — all clearly distinguished from Velsanet-specific decisions still pending. No Velsanet-specific number was invented in this revision: real current draw, final relay selection, and final IP rating remain open and are owned by 창호.

| ID | Open Item |
| --- | --- |
| OI-28-01 | Exact IP rating beyond the IP54+ floor — IEC 60529 basis now cited (Section 4); IP65/66 flagged as the likely fit for fully exposed installs; final rating still site-specific |
| OI-28-02 | Pole/mounting mechanical hardware specification — mechanical engineering scope, not Cognition Cube architecture scope |
| OI-28-03 | Actual CH11 controller + relay-hold current draw — worked sizing methodology now given (Section 5.2); real input current still unmeasured |
| OI-28-04 | Exact test equipment and fault-injection method for each Dual E2E field validation item — candidate for a future field validation runbook |
| OI-28-05 | Final EN 50205 relay part selection — shortlist of 3 reference options now given (Section 5.1); final part/rating margin still pending |

Table 8-1. WP#28 Open Items (R2)

| Doc No. | Title | Core Content | Status |
| --- | --- | --- | --- |
| WP #24 | Cognition Cube Physical Spec (HBM/HBF) | WORM register map, HBF ACL bit fields, electrical interfaces | Complete |
| WP #25 R1 | SEU1 / SEU2 Synchronization Specification | SEU1 (common) / SEU2 (EAI-only) record format, PTP profiles, completeness algorithms | Complete |
| WP #26 R8 | Dedicated Equipment FPGA/ASIC Design Guide | SEU1/SEU2 RTL engine templates, timing analysis, 800G optical interface, FPGA→ASIC migration | Complete (R8) — 12 open items remain |
| WP #27 | PAI AI DC Deployment Guide | Multi-instance parallel operation, rack wiring, DC power spec | Complete (companion document) — 3 open items |
| WP #28 | EAI E2E Center Deployment Guide | Outdoor install, CH11 independent power, Dual E2E field validation | Complete (this document) — 4 open items |
| (new, unnumbered) | O8 Connectivity & Placement Spec | D12 boundary as a function of O8-cluster density; Core Stack negotiation protocol | Proposed — spun off |

Table 8-2. Future Document Proposals — Updated at WP#28

This document is an internal technical design document of the Velsanet Architecture Group. All design specifications are based on software MVP validation results and will be finalized through physical hardware validation.

Velsanet Architecture Group — Internal Technical Document